FMUSER FUQ-4459R DVB-T Modulator 8 Tuner Inputs 4 RF Outputs DVB-C-T-S-S2-S2X-ISDB-T-ATSC IP UDP-RTP CAM BISS Descrambling Headend IPTV Cable TV Its production is stopped temporarily, and the date for resuming production is not fixed yet.

Product overview

The FMUSER FUQ-4459R is a high-performance DVB-T modulator with 8 tuner inputs and one RF output port, delivering 4 non-adjacent carriers. It processes signals by modulating them to DVB-T standards and supports IP input/output via UDP/RTP protocols. Features include CAM/CI descrambling and web-based NMS for remote management, ideal for broadcast transmission, IPTV headends, and cable TV systems.FU-4459T can be upgraded and controlled through network system, which allows it widely used in digital broadcasting network and provide good signals for scientific laboratory and DVB-T STB.

Key feature

Supports 8 tuner inputs with optional DVB-C/T/S/S2/S2X/ISDBT/ATSC standards and CAM slots.

Provides up to 128 IP inputs via UDP/RTP protocols (Version 1 only).

Features BISS descrambling and CAM/CI support.

Includes web-based NMS for remote control and upgrades.

Principle Chart

Specifications

| Input | 4 Tuner (DVB-C/T/(T)/S/S2/S2X/ISDBT optional) + 4 Tuner with CAM (DVB-C/T/(T)/S/S2/S2X/ATSC/ISDBT optional); 128 IP (SPTS/MPTS) input over UDP and RTP, RJ45, 1000M/100M, Unicast/Multicast (IP input is for Version 1 only) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New) | DVB-C | Input Frequency |  | 60 ~890 MHz |  |
|  |  |  | Standard |  | J.83A, J.83C | J.83B |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM | 64/256 QAM |
|  |  | DVB-T/(T) | Input Frequency |  | 60 ~890 MHz |  |
|  |  |  | Bandwidth |  | 6/7/8 M |  |
|  |  | ISDB-T | Input Frequency |  | 60 ~890 MHz |  |
|  | DVB-S/S2 Tuner | DVB-S | Input Frequency |  | 950~150MHz |  |
|  |  |  | Symbol rate |  | 0.5~45Msps |  |
|  |  |  | Signal Strength |  | -65~-25dBm |  |
|  |  |  | FEC Demodulation |  | 1/2, 2/3, 3/4, 5/6, 7/8 QPSK |  |
|  |  | DVB-S2 | Input Frequency |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | QPSK: 1~45 Msps; 8PSK: 0.5~45Msps |  |
|  |  |  | Code rate |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  | QPSK, 8PSK |  |
|  | DVB-S/S2/S2X Tuner (New) | DVB-S | Input Frequency |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | 0.5~45Msps |  |
|  |  |  | Signal Strength |  | - 65- -25dBm |  |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  |  | Constellation |  | QPSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  |  | DVB-S2 | Input Frequency |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  |  | DVB-S2X | Input Frequency |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  | ATSC-T |  | Input Frequency |  | 54~858MHz |  |
|  |  |  | Bandwidth |  | 6M |  |
| Descrambling | CAM/CI |  | 4 sets |  |  |  |
| BISS Descrambling | Mode 1, Mode E (descramble individual program) |  |  |  |  |  |
| Multiplexing | Maximum PID Remapping |  | 256 per input channel |  |  |  |
|  | Function |  | A/V PID filtering |  |  |  |
|  |  |  | PID Remapping |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |
| Modulation | DVB-T |  | Standard | EN300744 |  |  |
|  |  |  | FFT mode | 2K |  |  |
|  |  |  | Bandwidth | 6M, 7M, 8M |  |  |
|  |  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  |  | Guard Interval | 1/4, 1/8, 1/16, 1/32 |  |  |
|  |  |  | Code Rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  | MER | ≥42 dB |  |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  |  | RF out | 4*DVB-T non-adjacent carriers combined output, 40M total bandwidth limitation |  |  |
|  |  |  | RF output level | -28~ -3 dBm (79~104 dbµV), 0.1db step |  |  |
| Stream out | UDP and RTP protocol (RJ45, 1000M/100M Unicast/Multicast) 4*MPTS out --- Version 1 ; 32*SPTS out ---Version 2 |  |  |  |  |  |
| System | Web-based NMS support |  |  |  |  |  |
|  | Ethernet software upgrading |  |  |  |  |  |
| General | Dimension |  | 482mm×300mm×44.5mm (W*D*H) |  |  |  |
|  | Temperature |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |  |
|  | Power |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |  |

Order Guide

|  | Version 1 | Version 2 |
| --- | --- | --- |
| IP input | 128 SPTS/MPTS input | N/A |
| IP output | 4 MPTS output | 32 SPTS output |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227