FMUSER FUQ-4475 IP QAM Modulator 512 IP Inputs 64 QAM Dual RF Outputs GE Interface DVB-C ITU-T J.83 A-B-C Multiplexing Scrambling Web-Based Management for Broadcast Headends Product Overview

The FMUSER FUQ-4475 modulator converts IP signals to RF outputs, supporting up to 512 IP inputs via 6 GE ports and 64 QAM carriers through dual RF interfaces. It processes streams with multiplexing, scrambling, and modulation to DVB-C standards. Featuring web-based management, this device is ideal for broadcast headends and cable television distribution systems.

Key feature

Enables web-based control for network configuration.

Inner Principle Chart

Carrier Setting Illustration

Specifications

| Input | Input |  | Max 512 IP inputs from 6*100/1000M Ethernet Port (4*RJ45, 2*SFP) |  |
| --- | --- | --- | --- | --- |
|  | Transport Protocol |  | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate |  | Max 840Mbps for each GE input |  |
| Mux | Input Channel |  | 512 |  |
|  | Output Channel |  | 64 |  |
|  | Max PIDs |  | 256 per channel |  |
|  | Functions |  | PID remapping(auto/manually optional) |  |
|  |  |  | PCR accurate adjusting |  |
|  |  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulscrypt CA |  | 6 |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection |  | Local/remote connection |  |
| Modulation Parameters | Modulation Standard |  | EN300 429/ITU-T J.83A/B/C |  |
|  | Constellation |  | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  |  | Bandwidth :8M |
|  |  |  | J.83B/C | Constellation :64/256QAM |
|  |  |  |  | Bandwidth :6M |
|  | QAM Channel |  | 64 non-adjacent carriers 384Mbps bandwidth for each RF interface |  |
|  | Symbol Rate |  | 5.0~7.0Msps, 1ksps stepping 5057Ksps (J.83B, 64QAM) ; 5361Ksps (J.83B, 256QAM) |  |
|  | Constellation |  | 16, 32, 64 , 128, 256QAM |  |
|  | FEC |  | RS (204, 188) |  |
| RF Output | Interface |  | 2 F type output ports for 64 carriers, 75Ω |  |
|  | RF Range |  | 50~960MHz, 1kHz stepping |  |
|  | Output Level |  | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER |  | ≥ 40dB |  |
| TS output | 64 IP output over UDP/RTP/RTSP, unicast/multicast, 4*100/1000M Ethernet Ports (each port for 16 IP outputs) |  |  |  |
| System | Web-based Network management |  |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227