FMUSER FUQ-4512E Modulator 1 ASI Input to RF Output DVB-S QPSK RS Coding Convolution Interleaving Web-Based NMS for Broadcasting and Satellite Applications

Product overview

The FMUSER FUQ-4512E modulator performs QPSK modulation with 1 ASI input and RF output. It employs energy diffusion, RS coding, convolution interleaving, and pre-modulated baseband shaping for high transmission performance. Controlled via web-based NMS for local and remote operation, it is suitable for broadcasting, interactive services, and broadband satellite applications.

Key feature

Complies with DVB-S (EN300421) standard,

Supports 1 ASI input,

Features QPSK constellations,

Provides RF output range: 950-2150MHz with 10kHz stepping,

Offers web-based NMS for control

Principle Chart

Specifications

| ASI Input | 1×ASI input, BNC interface |  |
| --- | --- | --- |
| RF Output | RF Range | 950 ~ 2150 MHz, 10KHz stepping |
|  | Output Level | -10.0 dBm~-41.5 dBm,0.5dBm Stepping |
|  | MER | ≥26dB |
|  | Connector | N type, Impedance 50Ω |
|  | Outer coding | RS Coding |
|  | Inner coding | Convolution |
|  | Constellation | QPSK |
|  | FEC/Convolution Rate | 1/2,2/3, 3/4, 5/6, 7/8 |
|  | Roll-off Factor | 0.35 |
|  | Symbol Rate | 0.05~45Msps |
| System | Web-based NMS |  |
|  | Language: English |  |
|  | Ethernet software upgrade |  |
| Miscellaneous | Dimension | 482mm×245mm×44mm (W*D*H) |
|  | Temperature | 0~45℃(operation), -20~80℃ (storage) |
|  | Power | AC 100-240V, 50Hz-60Hz 2A |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227