FMUSER FUQ-4513A DVB-S-S2 Modulator 1-4 ASI Ports IP Input to DVB-S-S2 RF Output BISS Scrambling EN302307 Standard for Secure Satellite Broadcasting

Product overview

The FMUSER FBE4513E is a DVB-S2 modulator that converts ASI and IP inputs to DVB-S/S2 RF output, featuring BISS scrambling for secure distribution. It processes signals by modulating them according to EN302307 standards. Controlled via Web-server NMS and front-panel LCD, it is ideal for broadcasting and interactive services.

Key feature

Complies with DVB-S2 (EN302307) and DVB-S (EN300421) standards.

Supports ASI and IP inputs (1000/100M) for flexible integration.

Features BISS scrambling for secure program distribution.

Provides RF output at 950-2150MHz with 10KHz stepping and MER≥40dB.

Enables control via Web-server NMS and front-panel LCD.

Principle Chart

Specifications

| IP Input | 1*IP Input (RJ45, 1000/100M TS Over UDP) |  |  |  |
| --- | --- | --- | --- | --- |
| ASI Input | Supporting both188/204 Byte Packet TS Input |  |  |  |
|  | 1-4 ASI Inputs (Connector: BNC, Impedance 75Ω) |  |  |  |
| Tuner Input (0-4 inputs optional) | DVB-S; Input Frequency; 950~2150MHz. Symbol rate; 0.5~45Msps. Signal Strength; - 65- -25dBm. FEC; 1/2, 2/3, 3/4, 5/6, 7/8. Constellation; QPSK. Max input bitrate; ≤160 Mbps. DVB-S2; Input Frequency; 950~2150MHz. Symbol rate; QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps. FEC; QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10. Constellation; QPSK, 8PSK, 16APSK, 32APSK. Max input bitrate; ≤160 Mbps. DVB-S2X; Input Frequency; 950~2150MHz. Symbol rate; QPSK/8PSK /16APSK :0.5~45 Msps 8APSK/32APSK: 0.5~40Msps. FEC; QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9. Constellation; QPSK/8PSK/8APSK/16APSK/32APSK. Max input bitrate; ≤160 Mbps. |
| Multiplexing | Maximum PID Remapping |  | 128 input per channel |  |
|  | Function |  | PID remapping (automatically or manually) |  |
|  |  |  | Accurate PCR adjusting |  |
|  |  |  | Edit PSI/SI tables (PAT/PMT/SDT/CAT/NIT) |  |
| 10MHz Reference Clock | 1*External 10MHz Input (BNC Interface); 1*Inner 10MHz Reference clock |  |  |  |
| RF Output | RF Range: 950 ~ 2150 MHz, 10KHz stepping |  |  |  |
|  | Output Level Attenuation：-26 ~ 0 dBm，0.5dBm Stepping |  |  |  |
|  | MER≥40dB |  |  |  |
|  | Connector: N type, Impedance 50Ω; N to F adapter |  |  |  |
| Channel Coding and Modulation | Standard | DVB-S |  | DVB-S2 |
|  | Outer coding | RS Coding |  | BCH Coding |
|  | Inner coding | Convolution |  | LDPC Coding |
|  | Constellation | QPSK |  | QPSK,8PSK, 16APSK,32APSK |
|  | FEC/ Convolution Rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  | Roll-off Factor | 0.2, 0.25, 0.35 |  | 0.2, 0.25, 0.35 |
|  | Symbol Rate | 0.05~45Msps |  | 0.05~40Msps (32APSK); 0.05~45 Msps (16APSK/8PSK/QPSK) |
| BISS Scramble | Mode 0, mode 1, mode E |  |  |  |
| System | Web-server NMS |  |  |  |
|  | Language: English |  |  |  |
|  | Ethernet software upgrade |  |  |  |
|  | 24V power output through RF output port |  |  |  |
| Miscellaneous | Dimension | 482mm×410mm×44mm |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃ (storage) |  |  |
|  | Power | 100-240VAC±10%, 50Hz-60Hz |  |  |

Order Guide

|  | FU-4513E (New version) | FU-4513E (Old version) | FU-4513F | FU-4514 | FU-4514F |
| --- | --- | --- | --- | --- | --- |
| Input | 1 IP (100/1000M) input, ASI and/or DVB-S2 tuner inputs for re-mux or pass through | 1 IP (100M) or 1 ASI pass through alternative |  |  |  |
| DVB-S/S2 out | ● | ● | ● | ● | ● |
| DVB- S2X out |  |  |  | ● | ● |
| QPSK, 8PSK | ● | ● | ● | ● | ● |
| 16APSK, 32APSK | ● | ● |  | ● | ● |
| 8APSK-L,16APSK-L,32APSK-L |  |  |  | ● | ● |
| RF output (950-2150MHz) | ● | ● |  | ● |  |
| IF output ( 50-960MHz) |  |  | ● |  | ● |
| Mux | ● |  |  |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227