FMUSER FUQ-4514 DVB-S-S2-S2X Modulator 4 ASI + 1 IP Input to RF Output DVB-S-S2-S2X Standards BISS Scrambling Satellite Broadcasting Product overview

The FMUSER FUQ-4514 is a DVB-S/S2/S2X modulator with 4 ASI inputs and 1 IP input, outputting RF signals. It converts ASI and IP inputs to digital RF output, featuring BISS scrambling for secure distribution. Controlled via NMS software and LCD, it supports broadcasting and satellite applications.

Key feature

Supports full compliance with DVB-S, DVB-S2, and DVB-S2X standards (EN300 421, EN302 307, EN302 307-2).

Features 4 ASI inputs with backup and 1 IP input (1000/100M) for flexible re-muxing.

Includes BISS scrambling (Mode 0/1/E) for secure content distribution.

Offers Web-server NMS for local/remote control and SFN TS transmission support.

Equipped with constant temperature crystal oscillator (0.1ppm stability) and 24V power output via RF port.

Principle Chart

Specifications

| ASI Input | Supporting both188/204 Byte Packet TS Input |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 ASI Inputs, Supporting Backup |  |  |  |  |
|  | Connector: BNC, Impedance 75Ω |  |  |  |  |
| IP Input | 1*IP Input (RJ45, 1000/100M TS Over UDP) |  |  |  |  |
| 10MHz Input | 1*10MHz Input (BNC Interface) |  |  |  |  |
| Multiplexing | Maximum PID Remapping |  |  | 128 input per channel |  |
|  | Function |  |  | PID remapping (automatically or manually) |  |
|  |  |  |  | Accurate PCR adjusting |  |
|  |  |  |  | Edit PSI/SI tables (PAT/PMT/SDT/CAT/NIT) |  |
| RF Output | RF Range: 950 ~ 2150 MHz, 10KHz step |  |  |  |  |
|  | Output Level Attenuation：-27.5 dBm~-1.5 dBm，0.5dB Step |  |  |  |  |
|  | MER≥40dB |  |  |  |  |
|  | Connector: N type, impedance 50Ω |  |  |  |  |
| Channel Coding and Modulation | Standard | DVB-S |  | DVB-S2 | DVB-S2X |
|  | Outer coding | RS Coding |  | BCH Coding | BCH Coding |
|  | Inner coding | Convolution |  | LDPC Coding | LDPC Coding |
|  | Constellation | QPSK |  | QPSK,8PSK, 16APSK,32APSK | QPSK,8PSK, 8APSK, 16APSK, 32APSK |
|  | FEC/ Convolution Rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 , 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 , 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  | Roll-off Factor | 0.2,0.25,0.35 |  | 0.2, 0.25, 0.35 | 0.05, 0.10, 0.15, 0.2, 0.25, 0.35 |
|  | Symbol Rate | 0.05~45Msps |  | 0.05~40Msps (32APSK); 0.05~45Msps (QPSK/8PSK/16APSK) | 0.05~40Msps (8APSK /32APSK) 0.05~45 Msps (8PSK/QPSK/16APSK) |
| BISS Scramble | Mode 1, Mode E, Mode 0 |  |  |  |  |
| System | Web-server NMS |  |  |  |  |
|  | Language: English |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
|  | 24V power output through RF output port |  |  |  |  |
| Miscellaneous | Dimension |  | 482mm×410mm×44mm |  |  |
|  | Temperature |  | 0~45℃(operation), -20~80℃ (storage) |  |  |
|  | Power |  | 100-240VAC±10%,50Hz-60Hz |  |  |

Order Guide

|  | FU-4513E | FU-4513F | FU-4514 | FU-4514F |
| --- | --- | --- | --- | --- |
| DVB-S/S2 | ● | ● | ● | ● |
| DVB- S2X |  |  | ● | ● |
| RF output (950-2150MHz) | ● |  | ● |  |
| IF output ( 50- 960MHz) |  | ● |  | ● |

CID TEST SAMPLE ILLUSTRATION

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227