FMUSER FUQ-4514G DVB-S2X IF Modulator 4 ASI + 1 IP Input 1 IF Output ASI-IP to DVB-S-S2-S2X BISS Scrambling Web-server Control for Broadcasting and Satellite Communications

Product overview

FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.

Key feature

Complies with DVB-S, DVB-S2, and DVB-S2X standards.

Supports 4 ASI inputs with hot backup and IP input.

Features BISS scrambling for secure content protection.

Enables control via Web-server NMS and front panel LCD.

Output frequency: 50-960MHz with 1KHz stepping.

Principle Chart

FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.

FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.Specifications

| FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.ASI Input | Supporting both188 Byte Packet TS Input |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 ASI Inputs ( 3 ASI for backup) |  |  |  |  |
|  | Connector: BNC, Impedance 75Ω |  |  |  |  |
| IP Input | 1*IP Input (RJ45, 100M TS Over UDP) |  |  |  |  |
| 10MHz Input | 1*10MHz Input (BNC Interface) |  |  |  |  |
| IF Output | Range:50 ~ 960 MHz, 1KHz step |  |  |  |  |
|  | Output Level Attenuation: -28.5dBm~+3 dBm, 0.5dB Step |  |  |  |  |
|  | MER≥40dB |  |  |  |  |
|  | Connector: N type, impedance 50Ω |  |  |  |  |
| Channel Coding and Modulation | Standard | DVB-S |  | DVB-S2 | DVB-S2X |
|  | Outer coding | RS Coding |  | BCH Coding | BCH Coding |
|  | Inner coding | Convolution |  | LDPC Coding | LDPC Coding |
|  | Constellation | QPSK |  | QPSK,8PSK, 16APSK,32APSK | QPSK,8PSK, 8APSK, 16APSK, 32APSK |
|  | FEC/ Convolution Rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 , 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 , 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  | Roll-off Factor | 0.2,0.25,0.35 |  | 0.2, 0.25, 0.35 | 0.05, 0.10, 0.15, 0.2, 0.25, 0.35 |
|  | Symbol Rate | 0.05~45Msps |  | 0.05~40Msps (32APSK); 0.05~45Msps (16APSK/8PSK/QPSK) | 0.05~40Msps(32APSK,32APSK-L); 0.05~45 Mbps (16APSK/8PSK/QPSK/16APSK-L/8APSK-L) |
|  | BISS Scramble | Mode 1, Mode E |  |  |  |
| System | Web-server NMS |  |  |  |  |
|  | Language: English |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| Miscellaneous | Dimension |  | 482mm×410mm×44mm |  |  |
|  | Temperature |  | 0~45℃(operation), -20~80℃ (storage) |  |  |
|  | Power |  | 100-240VAC±10%,50Hz-60Hz |  |  |

FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.Order Guide

| FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | FU-4513E | FU-4513F | FU-4514 | FU-4514F |
| --- | --- | --- | --- | --- |
| DVB-S/S2 | ● | ● | ● | ● |
| DVB- S2X | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | ● | ● |
| RF output (950-2150MHz) | ● | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | ● | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. |
| IF output ( 50- 960MHz) | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | ● | FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications. | ● |

FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.FMUSER FUQ-4514G is a DVB-S2X compliant IF modulator that converts ASI and IP inputs to digital DVB-S/S2/S2X IF output. It modulates signals and applies BISS scrambling for secure distribution. Control is enabled via Web-server NMS and front panel LCD. Ideal for broadcasting, interactive services, and satellite applications.

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227