FMUSER FUQ-4515 Satellite Modem 1x Ethernet Port IP Input-Output DVB-S2-DVB-S TCP Acceleration High-Speed Two-Way Transmission for Satellite Networks Product Overview

The FMUSER FUQ-4515 is a two-way DVB-S2 satellite modem for efficient IP data transmission over satellite. It modulates and demodulates IP signals using TCP acceleration to optimize performance. Featuring web-based NMS with LCD and keyboard support, it simplifies management. Ideal for satellite networks in broadcast, enterprise, and remote data services.

Key feature

Supports DVB-S2 and legacy DVB-S standards for efficient satellite communication.

Embedded TCP acceleration optimizes high-speed links and heavy traffic.

High throughput up to 100 Mbps for upstream and downstream operations.

Web-based NMS operation with LCD and keyboard support for easy management.

Principle Chart

Specifications

| Input/Output | IP input/output over IPv4 (TCP/UDP/ICMP…) protocol 1 tuner in and 1 RF out |  |  |
| --- | --- | --- | --- |
| Tuner Section | DVB-S2 | Input Frequency | 950-2150MHz |
|  |  | Symbol rate | QPSK 1~45Mbauds; 8PSK 2~30Mbauds |
|  |  | Code rate | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation | QPSK, 8PSK |
| RF output | RF Range | 950 ~ 2150 MHz, 10KHz step |  |
|  | Output Level Attenuation | 0 dBm~-31.5 dBm, 0.5dB Step |  |
|  | MER | ≥40dB |  |
|  | Connector | N type, impedance 50Ω |  |
| Modulation | Standard | DVB-S2 |  |
|  | Outer coding | BCH Coding |  |
|  | Inner coding | LDPC Coding |  |
|  | Constellation | QPSK,8PSK,16APSK,32APSK |  |
|  | FEC/ Convolution Rate | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  | Roll-off Factor | 0.2, 0.25, 0.35 |  |
|  | Symbol Rate | 0.05~40Msps (32APSK); 0.05~45Msps (QPSK/8PSK/16APSK) |  |
| System | LCD/keyboard operating, web-based NMS manage |  |  |
|  | Language: English/Chinese |  |  |
|  | Ethernet software & hardware upgrade |  |  |
| Miscellaneous | Dimension | 482mm×328mm×44mm (W× L× H) |  |
|  | Temperature | 0~45℃ (work), -20~80℃（Storage） |  |
|  | Power | AC 100V-220V±10%, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227