FMUSER FUQ-4619C Tuner to IP Gateway 16 Tuner Inputs 2 ASI Inputs DVB-S-S2-S2X 4 IP Outputs UDP-RTP-RTSP BISS Descrambling Multiplexing Broadcast Headends Product overview

The FMUSER FUQ-4619C V2 is a tuner to IP gateway with 16 tuner inputs and 2 ASI inputs for IP MPTS output. It multiplexes signals and applies BISS descrambling with PSI/SI management. Controlled via web interface, it suits digital TV head-end systems for broadcasting.

Key feature

Supports up to 16 tuner inputs with DVB-S/S2/S2X standards and 2 ASI inputs.

Features embedded BISS descrambling with bit rate up to 80Mbps per channel.

Provides 4 IP outputs over UDP/RTP/RTSP and 2 GE mirrored outputs.

Enables PID filtering, re-mapping, and PSI/SI editing.

Working Principle

Specifications

| Input | 16 tuners input, (DVB-C, DVB-T/(T), ISDBT, S/S2/S2X, ATSC optional), F type 2 ASI input, BNC interface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tuner Section |  | DVB-C | Input Frequency |  |  | 30 ~1000 MHz |
|  |  |  | Standard |  |  | J.83A(DVB-C), J.83B, J.83C |
|  |  |  | Constellation |  |  | 16/32/64/128/256 QAM |
|  |  | DVB-T/(T) | Input Frequency |  |  | 30 ~1000 MHz |
|  |  |  | Bandwidth |  |  | 6/7/8 M |
|  | Version 1 | DVB-S | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 1~45 Msps |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  |  | QPSK |
|  |  |  | Signal Strength |  |  | - 65~ -25dBm |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 1~45 Msps |
|  |  |  | FEC |  |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  |  | QPSK, 8PSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  | Version 2 (New) | DVB-S | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 0.5~45Msps |
|  |  |  | Signal Strength |  |  | - 65- -25dBm |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  |  | QPSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2X | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | ISDB-T | Input Frequency |  |  | 30~1000MHz |
|  |  | ATSC | Input Frequency |  |  | 50 MHz~960MHz |
|  |  |  | Bandwidth |  |  | 6M |
|  |  |  | Constellation |  |  | 8VSB |
| Re-multiplex |  | PID re-mapping |  |  |  |  |
|  |  | PCR accurate adjusting |  |  |  |  |
|  |  | Automatic generating PSI/SI table |  |  |  |  |
| PID |  | Output range |  |  | 0x0000—0x1FFF |  |
|  |  | PID transparent |  |  | Any PID transparent and mapping achievable |  |
|  |  | Amount of output PID per input |  |  | 128 |  |
| BISS descrambling |  | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |  |
| Output |  | 4 IP (MPTS) output over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |  |
| System |  | Remote management |  | Web-server Management |  |  |
|  |  | Language |  | English |  |  |
|  |  | Upgrade |  | USB, web management |  |  |
| Miscellaneous |  | Dimension |  | 482mm×410mm×44mm (W×L×H) |  |  |
|  |  | Approx weight |  | 3.5kg |  |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |  |
|  |  | Power consumption |  | 20W |  |  |

Tuner Input Options:

- 8 DVB-C Tuners + 8 ATSC Tuners

- 8 DVB-S2/S2X Tuners + 8 DVB-(T) Tuners

- 8 DVB- S2/S2X Tuners + 8 ISDB-T Tuners

- 16 DVB-S/S2/S2X/DVB-C/(T) /ISDB-T/ATSC Tuners

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227