FUQ-4619C Tuner to IP Gateway 24 Tuners 2 ASI 1 USB Inputs DVB-S-S2-S2X DVB-C-T-T2 ISDB-T 1024 SPTS 27 MPTS IP Streaming BISS Descrambling Web GUI Broadcast Headend Product overview

The FUQ-4619C Tuner to IP Gateway converts signals from 24 FTA tuners, 2 DVB-ASI inputs, and 1 USB input into IP outputs. It processes signals with switchable MPTS (bypass) or SPTS modes and embedded BISS descrambling. Managed via Web GUI, it delivers up to 1024 SPTS streams over UDP/RTP/RTSP. This 1RU device is ideal for broadcast headends and IPTV distribution requiring flexible signal conversion.

Key feature

Supports 24 multi-standard tuners (switchable DVB-S/S2/S2X, DVB-C/T/T2, ISDB-T) plus 2 ASI inputs and 1 USB input.

Integrates BISS descrambling for Mode 1 and Mode E on tuner inputs.

Provides switchable MPTS/SPTS outputs (27 MPTS with Null PKT Filter or 1024 SPTS over dual DATA ports via UDP/RTP/RTSP).

Enables Web-based management for configuration, PID filtering, remapping, and updates.

Includes DiSEqC support for satellite signal control.

Specifications

| Input |  | 24 tuners inputs +2 ASI inputs +1 USB inputs |  |  |
| --- | --- | --- | --- | --- |
| Satellite | DVB-S | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | 0.5~45Msps |
|  |  | Signal Strength |  | - 65- -25dBm |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | Constellation |  | QPSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
| Multi-Mode Switchable Tuner Inputs | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  | Frequency In |  | 50~960MHz |
|  |  | Constellation |  | 16/32/64/128/256 QAM |
|  | DVB-T/T2 | Frequency In |  | 50~960MHz |
|  |  | Bandwidth |  | 6/7/8 M bandwidth |
|  | ISDB-T | Frequency In |  | 50~960MHz |
| BISS Descrambling |  | Mode 1, Mode E |  |  |
| Output |  | 1024 SPTS IP output over UDP and RTP/RTSP protocol through DATA1 and DATA2 port (IP address and port number of DATA1 and DATA2 are different) , Unicast and Multicast |  |  |
|  |  | 27 MPTS IP output (for Tuner/ASI/USB bypass) over UDP and RTP/RTSP protocol through DATA1 and DATA2 port, Unicast and Multicast |  |  |
|  |  | 2 ASI outputs(MPTS out mode only, mirror ports to output the same one TS) |  |  |
| System |  | Web based management |  |  |
|  |  | Ethernet software upgrade |  |  |
| Miscellaneous |  | Dimension | 482mm×270mm×44mm (W×L×H) |  |
|  |  | Environment | 0~45℃(work); -20~80℃（Storage） |  |
|  |  | Power requirements | 100~240VAC, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227