FMUSER FUQ-4619C Tuner-to-IP Gateway 16-24 DVB-S-S2-S2X Inputs 1 ASI 1 USB Output UDP-RTP-RTSP IP Streaming BISS Descrambling Broadcast IPTV HeadendProduct overview

The FUQ-4619C Tuner-to-IP Gateway converts 16-24 DVB-S/S2/S2X tuner inputs, 1 ASI, and 1 USB signals into IP streams via UDP/RTP/RTSP. It processes signals with BISS descrambling, DiSEqC control, and PID filtering, delivering up to 26 MPTS or 1024 SPTS streams through dual Gigabit Ethernet ports. This 1RU rack-mountable device features web-based management, ideal for broadcast headends and IPTV multiscreen distribution.

Key feature

Web-based configuration and Ethernet firmware upgrades for remote management

Working Principle

Specifications

| Input | 16 or 24 tuners input +1 ASI input +1 USB input---SPTS/MPTS output |  |  |  |
| --- | --- | --- | --- | --- |
| Satellite | DVB-S | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | 0.5~45Msps |
|  |  | Signal Strength |  | - 65- -25dBm |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | Constellation |  | QPSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
| BISS Descrambling | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |
| Output | 1024 SPTS IP output over UDP and RTP/RTSP protocol through GE1 and GE2 port (IP address and port number of GE1 and GE2 are different) , Unicast and Multicast |  |  |  |
|  | 26 MPTS IP output (for Tuner/ASI/USB pass-through) over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |
|  | 2 ASI output |  |  |  |
| System | Web based management |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension |  | 482mm×230mm×44mm (W×L×H) |  |
|  | Approx weight |  | 2.5 kg |  |
|  | Environment |  | 0~45℃(work); -20~80℃（Storage） |  |
|  | Power requirements |  | 100~240VAC, 50/60Hz |  |
|  | Power consumption |  | 20W |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227