FUQ-4619C Professional 16-Tuner IP Gateway 16 DVB-S-S2-S2X DVB-C-T-T2 ISDB-T ATSC Inputs 512 SPTS Output UDP-RTP-RTSP Protocol BISS Descrambling Broadcast Headend IPTV Distribution

Product overview

The FUQ-4619C is a high-density IP gateway converting 16 DVB-S/S2/S2X/DVB-C/T/T2/ISDB-T/ATSC tuner inputs and 2 ASI ports to IP streams. It processes signals with MPTS/SPTS conversion (16 MPTS or 512 SPTS) via UDP/RTP/RTSP, featuring BISS descrambling and DiSEqC control. The 1RU rack-mount unit includes dual Gigabit Ethernet ports and web-based management. Ideal for broadcast headend IP migration and multiscreen IPTV services.

Key feature

Web-based management for remote monitoring and firmware upgrades

Working Principle

Specifications

| Input |  | Optional 1:16 tuners input +2 ASI input---SPTS output Optional 2:14 tuners input +2 ASI input --- MPTS output Optional 3:16 tuners input --- MPTS output |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New Type) | DVB-C | Frequency In |  | 60~890MHz |
|  |  |  | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |
|  |  | DVB-T/(T) | Frequency In |  | 60~890MHz |
|  |  |  | Bandwidth |  | 6/7/8 M bandwidth |
|  |  | ISDB-T | Frequency In |  | 60~890MHz |
|  | Version 1 | DVB-S | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | 1~45 Msps |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  | DVB-S2 | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | 1~45 Msps |
|  |  |  | FEC |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK |
|  | Version 2 (New Type) | DVB-S | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | 0.5~45Msps |
|  |  |  | Signal Strength |  | - 65- -25dBm |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | ATSC | Frequency In |  | 54~858MHz |
|  |  |  | Bandwidth |  | 6M bandwidth |
|  |  |  | Constellation |  | 8VSB |
| BISS Descrambling |  | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |
| Output |  | 512 SPTS IP mirrored output over UDP and RTP/RTSP protocol through GE1 and GE2 port (IP address and port number of GE1 and GE2 are different) , Unicast and Multicast |  |  |  |
|  |  | 16 MPTS IP output (for Tuner/ASI passthrough) over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |
| System |  | Web based management |  |  |  |
|  |  | Ethernet software upgrade |  |  |  |
| Miscellaneous |  | Dimension |  | 482mm×410mm×44mm (W×L×H) |  |
|  |  | Approx weight |  | 3.6kg |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |
|  |  | Power consumption |  | 20W |  |

Tuner Input Options:

- 8 DVB-C Tuners + 8 ATSC Tuners

- 8 DVB-C Tuners + 8 DVB-T/(T) Tuners

- 8 DVB-S2/S2X Tuners + 8 DVB-T/(T) Tuners

- 8 DVB-S2/S2X Tuners + 8 ISDB-T Tuners

- 16 DVB-S/S2/S2X/DVB-C/T/(T) /ISDB-T/ATSC Tuners

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227