FMUSER FUQ-4619C Tuner to IP Gateway 24x Tuner 2x ASI 1x USB Input DVB-S-S2-S2X DVB-C-T-T2-ISDB-T Output 1024x SPTS 27x MPTS IP Streaming BISS Descrambling Broadcast IPTV Distribution Product overview

The FUQ-4619C Tuner to IP Gateway converts 24 FTA tuner inputs (supporting DVB-S/S2/S2X, DVB-C/T/T2, and ISDB-T), 2 ASI inputs, and 1 USB TS playback into IP streams. It processes signals through MPTS bypass or SPTS conversion with PID filtering/remapping, delivering outputs via UDP/RTP/RTSP protocols. This 1RU rack-mount device features web-based control for DiSEqC management and BISS Mode 1/E descrambling. Ideal for satellite-to-IP redistribution, multicast IPTV systems, and broadcast monitoring with 900Mbps throughput.

Key feature

Web-based management: Controls DiSEqC, firmware upgrades, and stream configurations via GUI

Working Principle

Specifications

| Input |  | 24 tuners inputs +2 ASI inputs +1 USB inputs |  |  |
| --- | --- | --- | --- | --- |
| Satellite | DVB-S | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | 0.5~45Msps |
|  |  | Signal Strength |  | - 65- -25dBm |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | Constellation |  | QPSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
| Multi-Mode Switchable Tuner Inputs | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  | Frequency In |  | 50~960MHz |
|  |  | Constellation |  | 16/32/64/128/256 QAM |
|  | DVB-T/T2 | Frequency In |  | 50~960MHz |
|  |  | Bandwidth |  | 6/7/8 M bandwidth |
|  | ISDB-T | Frequency In |  | 50~960MHz |
| BISS Descrambling |  | Mode 1, Mode E |  |  |
| Output |  | 1024 SPTS IP output over UDP and RTP/RTSP protocol through DATA1 and DATA2 port (IP address and port number of DATA1 and DATA2 are different) , Unicast and Multicast |  |  |
|  |  | 27 MPTS IP output (for Tuner/ASI/USB bypass) over UDP and RTP/RTSP protocol through DATA1 and DATA2 port, Unicast and Multicast |  |  |
|  |  | 2 ASI outputs(MPTS out mode only, mirror ports to output the same one TS) |  |  |
| System |  | Web based management |  |  |
|  |  | Ethernet software upgrade |  |  |
| Miscellaneous |  | Dimension | 482mm×270mm×44mm (W×L×H) |  |
|  |  | Environment | 0~45℃(work); -20~80℃（Storage） |  |
|  |  | Power requirements | 100~240VAC, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227