FUQ-4619C IP Gateway 24-Tuner DVB-S-DVB-S2-DVB-S2X Input, 1024 SPTS   26 MPTS IP Output  Dual GE Ports BISS Descrambling Broadcast Headend IPTV Delivery

Product overview

The FUQ-4619C IP gateway receives DVB-S/S2/S2X signals via 24 tuners or optional ASI/USB inputs. It converts satellite feeds into 26 MPTS or 1024 SPTS IP streams through dual GE ports, featuring BISS descrambling and PID filtering. The 1RU rack-mount unit supports web-based control with DiSEqC compatibility. Ideal for broadcast contribution links, IPTV headends, and multi-screen delivery systems requiring high-density satellite-to-IP conversion.

Key feature

Web Control: Browser-based management with DiSEqC 1.0/1.2 support for LNB configurationWorking Principle

Specifications

| Input | 16 or 24 tuners input +1 ASI input +1 USB input---SPTS/MPTS output |  |  |  |
| --- | --- | --- | --- | --- |
| Satellite | DVB-S | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | 0.5~45Msps |
|  |  | Signal Strength |  | - 65- -25dBm |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | Constellation |  | QPSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
|  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  | Max input bitrate |  | ≤125 Mbps |
| BISS Descrambling | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |
| Output | 1024 SPTS IP output over UDP and RTP/RTSP protocol through GE1 and GE2 port (IP address and port number of GE1 and GE2 are different) , Unicast and Multicast |  |  |  |
|  | 26 MPTS IP output (for Tuner/ASI/USB pass-through) over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |
|  | 2 ASI output |  |  |  |
| System | Web based management |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension |  | 482mm×230mm×44mm (W×L×H) |  |
|  | Approx weight |  | 2.5 kg |  |
|  | Environment |  | 0~45℃(work); -20~80℃（Storage） |  |
|  | Power requirements |  | 100~240VAC, 50/60Hz |  |
|  | Power consumption |  | 20W |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227