FMUSER FUQ-4633B HD Encoder Modulator HDMI Input to RF Output MPEG2 Encoding Multi-Standard DVB-C-T-ATSC-ISDB-T Modulation Web-NMS Control for TV Distribution and Surveillance

Product overview

The FMUSER FUQ-4633D is an HD Encoder Modulator that accepts HDMI input signals and outputs RF signals. It performs MPEG2 encoding and modulation to convert audio/video signals into broadcast-ready formats. Available in compact sizes (e.g., 20314452mm or 18011039.5mm), it supports management via Web or NMS. Ideal for TV distribution in home entertainment, surveillance, and commercial applications.

Key feature

Supports MPEG2 video encoding at 1.000-20.000 Mbps and audio in MPEG1 layer 2, AAC, AC3 formats.

Features multi-standard modulation including DVB-C, DVB-T, ATSC, ISDB-T with RF frequency 100-1000MHz and level -63 to -16dBm.

Offers flexible management via Web or NMS, with upgrade options.

Accepts HDMI input from sources like STBs, satellite receivers, CCTV cameras.

Compact design with dimensions 20314452mm or 18011039.5mm for space efficiency.

System Connection

Technical Specifications

| HDMI Encoding Section |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Video | Encoding |  |  | MPEG2 |  |
|  | Interface |  |  | HDMI*1 |  |
|  | Input |  |  | Output |  |
|  | 480@59.94/60i |  |  | 480@30p |  |
|  | 576@50i |  |  | 576@25p |  |
|  | 720@50/59.94/60p |  |  | 720@25/30p |  |
|  | 1080@50i |  |  | 1080@25p |  |
|  | 1080@59.94/60i |  |  | 1080@30p |  |
|  | 1080@50p |  |  | 1080@25p |  |
|  | 1080@59.94/60p |  |  | 1080@30p |  |
|  | Aspect Ratio |  |  | 16:9 |  |
|  | Bit rate |  |  | 1.000~20.000 Mbps |  |
| Audio | Encoding |  |  | MPEGI layer 2, AAC, AC3 |  |
|  | Sample rate 48KHz |  |  |  |  |
|  | Bit rate |  |  | MPEGI layer 2（192kbps） |  |
|  |  |  |  | AAC（VBR） |  |
|  |  |  |  | AC3（128kbps） |  |
| DVB-C Modulator Section |  |  |  |  |  |
| Standard |  |  | J.83A (DVB-C), J.83B |  |  |
| MER |  |  | ≥35dB |  |  |
| RF frequency |  |  | 100-1000MHz, 1KHz step |  |  |
| RF output level |  |  | -63~ -16dBm, 1dB step |  |  |
| Symbol rate |  |  | 3.000~7.000Msps adjustable |  |  |
|  |  | J.83A |  |  | J.83B |
| Constellation |  | 16/32/64/128/ 256 QAM |  |  | 64/ 256 QAM |
| Bandwidth |  | 8M |  |  | 6M |
| DVB-T Modulator Section |  |  |  |  |  |
| Standard |  | DVB-T COFDM |  |  |  |
| Bandwidth |  | 2M, 3M, 4M, 5M, 6M, 7M, 8M (Version—Ⅰ) 6M, 7M, 8M (Version—Ⅱ) |  |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |  |
| Transmission Mode: |  | 2K, 8K |  |  |  |
| MER |  | ≥35dB |  |  |  |
| RF frequency |  | 100-1000MHz, 1KHz step |  |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |  |
| ISDB Modulator Section |  |  |  |  |  |
| Standard |  | ARIB STD-B31 |  |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |  |
| Transmission Mode |  | 2K, 4K, 8K |  |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |
| RF frequency |  | 100~1000MHz, 1KHz step |  |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |  |
| ATSC Modulator Section |  |  |  |  |  |
| Standard |  | ATSC A/53 |  |  |  |
| Constellation |  | 8 VSB |  |  |  |
| RF output level |  | -63~-16dBm (44~91 dBµV), 1dB step |  |  |  |
| MER |  | ≥35dB |  |  |  |
| RF frequency |  | 100~1000MHz, 1KHz step |  |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |  |
| System |  |  |  |  |  |
| Management |  | Web (Version—Ⅰ) NMS (Version—Ⅱ) |  |  |  |
| Language |  | English |  |  |  |
| Upgrade |  | Web update (Version—Ⅰ) ST Link utility (Version—Ⅱ) |  |  |  |
| General |  |  |  |  |  |
| Power supply |  | DC 12V |  |  |  |
| Dimensions |  | 203*144*52mm (Version—Ⅰ) 180*110*39.5mm (Version—Ⅱ) |  |  |  |
| Weight |  | < 1kg |  |  |  |

Comparison

| Device | CC input | Management | Upgrade |
| --- | --- | --- | --- |
| FU-4633C (Version—Ⅰ) | √ | Web | Web |
| FU-4633C (Version—Ⅱ) | Optional | NMS | ST Link utility |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227