FMUSER FUQ-4633C HDMI-CVBS Input HD-SD Encoder Modulator ISDB-T-DVB-T Dual Standard H.264 Encoding RF Output for Surveillance Digital Signage

Product overview

The FMUSER FUQ-4633C is an HD/SD encoder modulator that accepts HDMI and CVBS inputs and outputs ISDB-T or DVB-T RF signals. It encodes video using H.264 MPEG-4 AVC and modulates for RF transmission. With local LCD control and USB upgrade, it supports easy configuration. Ideal for home entertainment, surveillance systems, and digital signage broadcasting.

Key feature

- Supports dual-standard modulation: ISDB-T (ARIB STD-B31) and DVB-T (COFDM) with 6M/7M/8M bandwidths.

- Features H.264 MPEG-4 AVC video encoding at 1.000-18.000 Mbps and MPEG-1 Layer 2 audio at 48KHz.

- Accepts HDMI and CVBS inputs, handling resolutions up to 1080p/i.

- Provides adjustable RF output with 150-950MHz frequency range and -47 to 0 dBm level.

- Includes local LCD management and USB upgrade for system maintenance.

System Connection

Technical Specifications

| HDMI Encoding Section |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video | Encoding |  | H.264 MP@L 3.0/3.1/4.0 |  |  |  |
|  | Interface |  | HDMI*1 |  |  |  |
|  |  |  | Input |  | Output |  |
|  | Resolution |  | 480@59.94/60p |  | 480@60p |  |
|  |  |  | 480@59.94/60i |  | 480@30p |  |
|  |  |  | 576@50i |  | 576@25p |  |
|  |  |  | 720@50/59.94/60p |  | 720@50/59.94/60p |  |
|  |  |  | 1080@50i |  | 1080@25p |  |
|  |  |  | 1080@59.94/60i |  | 1080@30p |  |
|  |  |  | 1080@59.94/60p |  | 1080@30p |  |
|  | Aspect Ratio |  | 16:9, 4:3 |  |  |  |
|  | Bit rate |  | 1.000~18.000 Mbps |  |  |  |
| Audio | Encoding |  | MPEGI layer 2 |  |  |  |
|  | Sample rate |  | 48KHz |  |  |  |
|  | Bit rate |  | 64, 96,128, 192, 256, 320kbps |  |  |  |
| CVBS Encoding Section |  |  |  |  |  |  |
| Video | Encoding |  |  | H.264 MP@L 3.0 |  |  |
|  | Interface |  |  | CVBS*1 (RCA) |  |  |
|  | Resolution |  |  | Input |  | Output |
|  |  |  |  | 480@60p |  | 480@60p |
|  |  |  |  | 480@60i |  | 480@30p |
|  |  |  |  | 576@50p |  | 576@50p |
|  |  |  |  | 576@50i |  | 576@25p |
|  | Aspect Ratio |  |  | 4:3 |  |  |
|  | Bit rate |  |  | 1.000~18.000 Mbps |  |  |
| Audio | Encoding |  |  | MPEGI layer 2 |  |  |
|  | Interface |  |  | Analog Stereo/Mono (Unbalanced RCA) |  |  |
|  | Sample rate |  |  | 48KHz |  |  |
|  | Bit rate |  |  | 64, 96,128, 192, 256, 320kbps |  |  |
| ISDB-T Modulator Section |  |  |  |  |  |  |
| Standard |  | ARIB STD-B31 |  |  |  |  |
| Bandwidth |  | 6M |  |  |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |  |  |
| Transmission Mode |  | 2K, 4K, 8K |  |  |  |  |
| MER |  | ≥30dB |  |  |  |  |
| RF frequency |  | 150~950MHz, 0.1MHz step |  |  |  |  |
| RF output level |  | -47~ 0 dBm (60~107 dbµV), 1db step |  |  |  |  |
| DVB-T Modulator Section |  |  |  |  |  |  |
| Standard |  | DVB-T COFDM |  |  |  |  |
| Bandwidth |  | 6M, 7M, 8M |  |  |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |  |  |
| Transmission Mode: |  | 2K, 8K |  |  |  |  |
| MER |  | ≥31dB |  |  |  |  |
| RF frequency |  | 150~950 MHz,0. 1KHz step |  |  |  |  |
| RF output level |  | -47~ 0 dBm (60~107 dbµV), 1db step |  |  |  |  |
| System |  |  |  |  |  |  |
| RF mix in ATT |  | 10 dB |  |  |  |  |
| Management |  | Local LCD + control buttons |  |  |  |  |
| Language |  | English |  |  |  |  |
| Upgrade |  | USB |  |  |  |  |
| General |  |  |  |  |  |  |
| Power supply |  | DC 12V |  |  |  |  |
| Dimensions |  | 183*110*50mm |  |  |  |  |
| Weight |  | < 1kg |  |  |  |  |

Typical Applications

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227