FMUSER FUQ-4635B Encoder Modulator 1-4-8 HDMI Inputs MPEG2 Encoding Multi-Standard RF DVB-C-DVB-T-ATSC-ISDB-T-DTMB Web NMS Control Broadcast Systems Cable TV Headend Product Overview

The FUQ-4635D Encoder Modulator integrates MPEG2 encoding and modulation, supporting scalable 1/4/8 HDMI inputs and multi-standard RF outputs including DVB-C, DVB-T, ATSC, ISDB-T, and DTMB. It converts audio/video signals into encoded and modulated RF formats. This device features a modular design and web-based NMS for control. Ideal for broadcast systems and cable TV headends.

Key feature

Supports scalable 1/4/8 HDMI inputs for flexible source integration.

Features MPEG2 video encoding with resolutions up to 1080p/i, 720p, 576i, and 480i.

Offers low latency operation from 50ms to 1000ms.

Provides multi-standard RF output including DVB-C, DVB-T, ATSC, ISDB-T, and DTMB with dynamic switching.

Includes web-based NMS for easy control and updates.

Inner Principle Chart

Specifications

| HDMI Encoding Section |  |  |  |
| --- | --- | --- | --- |
| Input |  | Interface | HDMI*1/4/8 |
| Video |  | Encoding | MPEG2 |
|  |  | Input | Output |
|  |  | 480@59.94/60i | 480@30p |
|  |  | 576@50i | 576@25p |
|  |  | 720@50/59.94/60p | 720@25/30p |
|  |  | 1080@50i | 1080@25p |
|  |  | 1080@59.94/60i | 1080@30p |
|  |  | 1080@50p | 1080@25p |
|  |  | 1080@59.94/60p | 1080@30p |
|  |  | Bit-rate | 2.000~20.000 Mbps |
|  |  | GOP | 1~60 |
|  |  | Rate Control | VBR |
|  |  | Aspect Ratio | 16 :9 |
| Audio |  | Encoding | MPEGI layer 2,MPEG-2 AAC, AC3 |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | MPEGI layer 2 (128kbps) |
|  |  |  | MPEG-2 AAC (VBR) |
|  |  |  | AC3 (128kbps) |
| DVB-C Modulator Section |  |  |  |
| Standard |  | J.83A (DVB-C), J.83B |  |
| MER |  | ≥35dB |  |
| RF frequency |  | 100-1000MHz, 1KHz step |  |
| RF output level |  | -48~ -10dBm, 1dB step |  |
| Symbol rate |  | 3.000~7.000Msps adjustable |  |
|  |  | J.83A | J.83B |
| Constellation |  | 16/32/64/128/256QAM | 64/ 256QAM |
| Bandwidth |  | 8M | 6M |
| DVB-T Modulator Section |  |  |  |
| Standard |  | DVB-T COFDM |  |
| Bandwidth |  | 2M, 3M, 4M, 5M, 6M, 7M, 8M |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |
| Transmission Mode |  | 2K, 8K |  |
| MER |  | ≥35dB |  |
| RF frequency |  | 100-1000MHz, 1KHz step |  |
| RF output level |  | -48~ -10dBm, 1dB step |  |
| ISDB Modulator Section |  |  |  |
| Standard |  | ARIB STD-B31 |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |
| Transmission Mode |  | 2K, 4K, 8K |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
| RF frequency |  | 100~1000MHz, 1KHz step |  |
| RF output level |  | -48~ -10dBm, 1dB step |  |
| ATSC Modulator Section |  |  |  |
| Standard |  | ATSC A/53 |  |
| Constellation |  | 8 VSB |  |
| RF output level |  | -48~ -10dBm, 1dB step |  |
| MER |  | ≥35dB |  |
| RF frequency |  | 100~1000MHz, 1KHz step |  |
| DTMB Modulator Section |  |  |  |
| RF frequency |  | 100-1000MHz, 1KHz step |  |
| RF output level |  | -48~ -10dBm, 1dB step |  |
| Transmission Mode |  | S-0.8-4QAM-595, S-0.8-16QAM-595, M-0.4-16QAM-945V, M-0.6-16QAM-945V, M-0.6-64QAM-420V, M-0.6-64QAM-945V, M-0.8-16QAM-420V, M-0.8-16QAM-945V, M-0.8-64QAM-420V, |  |
| System |  |  |  |
| Management | Web |  |  |
| Language | English |  |  |
| Upgrade | Web update |  |  |
| General |  |  |  |
| Power supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |  |
| Dimensions | 482*328*44.5mm |  |  |
| Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |

Technical Comparison

|  | FU-4653A | FU-4653I | FU-4635C |
| --- | --- | --- | --- |
| Chipset | Vixs | Hisilicon | VATek |
| Stream Input | 1*ASI input for re-mux | 1 DVB-C tuner | N/A |
| IP Input | N/A | 128*IP | N/A |
| Video Channel | 4* HDMI/SDI input optional | 4/8/12*HDMI input optional | 1/4/8*HDMI input optional |
| Video Encode | MPEG2 & MPEG4 AVC/H.264 | MPEG-4 AVC/H.264 | MPEG2 |
| Video Bit Rate | 0.8~19Mbps for H.264 encoding 1~19Mbps for MPEG-2 encoding | 1Mbps~13Mbps each channel (Bit rate is with a high fluctuation, suitable for cable system, not suitable for satellite uplink) | 2~20Mbps each channel |
| Resolution | Up to 1920*1080@50P/60P (MPEG4 AVC/H.264) Up to 1920*1080@50I/60I (MPEG2 HD) | Up to 1920*1080@50P/60P | Up to 1920*1080@50P/60P |
| Audio Encode | MPEG1 Layer 2, MPEG2-AAC, MPEG4-AAC, AC3 2.0 (Optional) | MPEG-1 Layer 2 | MPEGI layer 2, MPEG2-AAC, AC3 |
| Audio Gain | N/A | Yes | N/A |
| OSD Function | N/A | Yes | N/A |
| CC Input | CC (Closed Caption)for SDI interface (Optional), EIA608, EIA708 | N/A | CC (Closed Caption), EIA 608 |
| Low Latency | Yes | N/A | Yes |
| ASI output | Yes | N/A | N/A |
| IP output | 4 SPTS or 1 MPTS output | 4 MPTS IP output | N/A |
| Modulating | 4*DVB-C/2* DVB-T/4*ATSC-T /1/2*ISDB-T RF out | 4*DVB-C/DVB-T RF out | 4/8*DVB-C/T/ATSC/ISDB-T RF out in one device, Modulation dynamic switching |
| RF Frequency | 30~960MHz, 1KHz step | 50~960MHz, 1KHz step | 100-1000MHz, 1KHz step |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227