FMUSER FUQ-4635S HDMI Encoder Modulator 4 HDMI Inputs 2 RF DVB-C-T Outputs MPEG-4 AVC H.264 CATV Headend IPTV System

Product Overview

The FUQ-4635S/FUQ-4639S integrates MPEG-4 AVC/H.264 encoding and DVB-C/T modulation in a 1RU chassis, supporting 4/8 HDMI inputs, 64/128 IP streams, and 2/4 RF carriers. It processes signals with PID remapping and PSI/SI editing, delivering MPTS via Gigabit Ethernet. Controlled via web interface, this unit is designed for hotel TV systems, CATV headends, and multi-dwelling unit deployments.

Key feature

Provides web-based control for configuration and monitoring

Principle Chart

Technical Specification

| Input | 4/8 HDMI inputs; 64/128 IP inputs; 1 DVB-C/T/ISDB-Tb tuner input for re-mux (Optional for FU-4639S with DVB-C RF out version) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video | Encoding | MPEG-4 AVC/H.264 |  |  |  |  |
|  | Resolution | In-put |  | 1920×1080_60P, 1920×1080_60i, 1920×1080_50P, 1920×1080_50i, 1280×720_60P, 1280×720_50P, 720×576_50i,720×480_60i, |  |  |
|  |  | Out-put |  | 1920×1080_30P, 1920×1080_25P, 1280×720_30P, 1280×720_25P, 720×576_25P,720×480_30P, |  |  |
|  | Bit-rate | 1Mbps~13Mbps each channel |  |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |  |
| Audio | Encoding | MPEG-1 Layer 2, LC-AAC, HE-AAC and AC3 Pass Through |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Resolution | 24-bit |  |  |  |  |
|  | Audio Gain | 0-255 Adjustable |  |  |  |  |
|  | MPEG-1 Layer 2 bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | LC-AAC bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | HE-AAC bit-rate | 48/56/64/80/96/112/128 kbps |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 180 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Scrambling (optional for FU-4639S with DVB-C RF out version) | Maximum simulcrypt CA | 4 |  |  |  |  |
|  | Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |  |
|  | Connection | Local/remote connection |  |  |  |  |
| Modulation | DVB-C | RF out |  |  | 2/4*RF DVB-C out (2/4 carriers combined output), F type interface |  |
|  |  | Standard |  |  | EN300 429/ITU-T J.83A/B |  |
|  |  | MER |  |  | ≥40db |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  |  | -25~-1dBm (82~105 dbµV), 0.1dBm |  |
|  |  | Symbol Rate |  |  | 5.0Msps~7.0Msps, 1ksps stepping |  |
|  |  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  |  | 8M | 6M |
|  | DVB-T | RF out |  |  | 2/4*RF COFDM DVB-T out (2/4 carriers combined output) ,F type interface |  |
|  |  | Standard |  |  | EN300744 |  |
|  |  | FFT mode |  |  | 2K, |  |
|  |  | Bandwidth |  |  | 6M, 7M, 8M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥42 dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  |  | -28~ -3 dBm (77~97 dbµV), 0.1db step |  |
| Stream output | 2/4 IP MPTS output over UDP/RTP/RTSP, 1*1000M Base-T Ethernet interface |  |  |  |  |  |
| System function | Network management(WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×328mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ±10%,50/60Hz |  |  |  |

Order Guide

|  | FU-4635S | FU-4639S (DVB-C Out) | FU-4639S (DVB-T Out) |
| --- | --- | --- | --- |
| HDMI input | 4 | 8 | 8 |
| IP input | 64 | 128 | 128 |
| 1 DVB-C/T/ISDB-Tb tuner input | N/A | Option as order | N/A |
| Scrambling function | N/A | Option as order | N/A |
| IP output | 2 MPTS | 4 MPTS | 4 MPTS |
| DVB-C/T RF output channel | 2 | 4 | 4 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227