FMUSER FUQ-4647B-N DTV Processor 8-24 HDMI 1 ASI 1 USB 128 IP Inputs to 32 DVB-C 16 DVB-T-ATSC-ISDB-T Outputs HDMI-ASI-USB-IP Interfaces HEVC-H.265 AVC-H.264 Encoding Integrated DVB IPTV Streaming for Terrestrial Cable IPTV Headends

Product Overview

The FMUSER FUQ-4647B-P DTV Processor integrates DVB encoding and IPTV streaming in one chassis. It supports inputs including HDMI, ASI, USB, and IP, and outputs for RF and IP streams. With web-based control, it is ideal for broadcast applications in DVB and IPTV environments.

System Application

Key feature

Integrated DVB encoding and IPTV streaming in a single unit.

Supports up to 24 HDMI inputs, 1 ASI input, 1 USB player, and 128 IP inputs.

Delivers up to 32-channel RF output for DVB-C/T/ATSC/ISDB-T and IP streaming.

Features HEVC/H.265 and MPEG-4 AVC/H.264 encoding with web-based management.

Working Block

Technical Specifications

| Input | 8/16/24 HDMI inputs for option (or HDMI and different Tuner mix input, details pls check below order guide) 1 ASI in for re-mux 1 USB Player input for re-mux (“xxx.ts” video) 128 MPTS IP input over UDP and RTP, GE port, RJ45 SPTS input over UDP/RTP/RTSP/HLS/HTTP/SRT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video (HDMI Input) | Resolution | Input |  |  | 1920×1080_60P, 1920×1080_60i, 1920×1080_50P, 1920×1080_50i, 1280×720_60P, 1280×720_50P, 720×576_50i,720×480_60i, |  |
|  |  | Output |  |  | 1920×1080_30P, 1920×1080_25P, 1280×720_30P, 1280×720_25P, 720×576_25P,720×480_30P, |  |
|  | Encoding | HEVC/H.265 & MPEG-4 AVC/H.264 |  |  |  |  |
|  | Bit-rate | 1Mbps~13Mbps each channel |  |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |  |
|  | GOP Structure | IP…P (P Frame adjustment) |  |  |  |  |
| Audio (HDMI Input) | Encoding | MPEG-1 Layer 2, LC-AAC, HE-AAC and AC3 Pass through |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Resolution | 24-bit |  |  |  |  |
|  | Audio Gain | 0-255 Adjustable |  |  |  |  |
|  | MPEG-1 Layer 2 Bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | LC-AAC Bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | HE-AAC Bit-rate | 48/56/64/80/96/112/128 kbps |  |  |  |  |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |  |  |
|  |  | Symbol Rate |  | 0.5~45 Msps |  |  |
|  |  | Constellation |  | QPSK |  |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |  |  |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps 32APSK:0.5~40 Msps |  |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/32APSK |  |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9,9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |  |  |
|  |  | Max input bitrate |  | ≤170 Mbps per tuner |  |  |
|  | DVB-S2X | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |  |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/8APSK/32APSK |  |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |  |  |
|  | Signal Strength | -65 ~ -25dBm |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 255 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Modulation | DVB-C | Standard |  |  | EN300 429/ITU-T J.83A/B |  |
|  |  | RF output channel |  |  | 32 non-adjacent carriers output (maximum bandwidth 384MHz) |  |
|  |  | MER |  |  | ≥40db |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  |  | -20~+1dbm, 0.1db step |  |
|  |  | Symbol Rate |  |  | 5.0Msps~7.0Msps, 1ksps stepping |  |
|  |  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  |  | 8M | 6M |
|  | DVB-T | Standard |  |  | EN300744 |  |
|  |  | FFT mode |  |  | 2K,4K, 8K |  |
|  |  | Bandwidth |  |  | 6M, 7M, 8M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | Code Rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥42 dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output channel |  |  | 16 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~ +1dBm, 0.1db step |  |
|  | ATSC | Standard |  |  | ATSC A/53 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | 8VSB |  |
|  |  | FEC |  |  | RS(208 188)+Trellis |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | ACL |  |  | -55 dBc |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output channel |  |  | 16 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~+3dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | FFT Mode |  |  | 2K, 4K, 8K |  |
|  |  | Code rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output channel |  |  | 16 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20dBm~+1dBm, 0.1dB stepping |  |
| MPTS output | 1 ASI output as mirror of one of RF output carriers 32 MPTS output over UDP and RTP/RTSP as mirror of 32 DVB-C carriers(or 16 MPTS output over UDP and RTP/RTSP as mirror of 16 DVB-T/ATSC/ISDB-T carriers for option), 1*1000M Base-T Ethernet interface, GE port |  |  |  |  |  |
| SPTS output | SPTS over UDP/RTP/RTSP/RTMP/HLS/HTTP/SRT |  |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |  |
|  | English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×328mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |  |

Ordering Guide

Input Section:

|  | HDMI | DVB-S2/S2X | DVB-C/T/T2/ISDB-T |
| --- | --- | --- | --- |
| Option 1 | 8 | / | / |
| Option 2 | 16 | / | / |
| Option 3 | 24 | / | / |
| Option 4 | 8 | 8 | / |
| Option 5 | 8 | 16 | / |
| Option 6 | 16 | 8 | / |
| Option 7 | 8 | / | 8 |
| Option 8 | 8 | / | 16 |
| Option 9 | 16 | / | 8 |
| Option 10 | 8 | 8 | 8 |
| Option 11 | 16 | 4 | 4 |

Output Section:

| RF output standard | RF carrier channels |
| --- | --- |
| DVB-C out | 32 |
| DVB-T out | 16 |
| ATSC out | 16 |
| ISDB-T out | 16 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227