FMUSER FUQ-4647R HD Encoder Modulator 8-24 HDMI 1 ASI 1 USB 128 IP Input MPEG-4 AVC-H264 Encoding Multiplexing DVB-C-T-ATSC-ISDB-T Output CATV Headend Hotel TV SystemModulator

Product Overview

The FMUSER FUQ-4647B is an encoder modulator integrating encoding, multiplexing, and modulation in a 1RU chassis. It processes inputs including 8/16/24 HDMI, 1 ASI, 1 USB player, and 128 IP streams, encoding with MPEG-4 AVC/H.264 and outputting 12 DVB-C RF carriers plus ASI/IP mirrors. Featuring web-based control for remote management, it is ideal for compact CATV headends and hospitality TV systems.

Key feature

Supports multi-format inputs: 8/16/24 HDMI, 1 ASI, 1 USB player, and 128 IP streams via GE port.

Offers flexible modulation: Outputs 12 DVB-C RF carriers (or optional formats) with ≤192MHz bandwidth.

Provides advanced processing: MPEG-4 AVC/H.264 encoding, PID remapping, PCR adjustment, and PSI/SI editing.

Includes dual monitoring: ASI output mirror and MPTS IP streaming via UDP/RTP/RTSP.

Features web-based control for remote management and firmware updates.

Principle Chart

Technical Specifications

| Input | 8/16/24 HDMI inputs for option (or HDMI and different Tuner mix input, details pls check below order guide) 1 ASI in for re-mux 1 USB Player input for re-mux (“xxx.ts” video) 128 IP input over UDP and RTP, GE port, RJ45 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video (HDMI Input) | Resolution | Input |  |  | 1920×1080_60P, 1920×1080_60i, 1920×1080_50P, 1920×1080_50i, 1280×720_60P, 1280×720_50P, 720×576_50i,720×480_60i, |  |
|  |  | Output |  |  | 1920×1080_30P, 1920×1080_25P, 1280×720_30P, 1280×720_25P, 720×576_25P,720×480_30P, |  |
|  | Encoding | MPEG-4 AVC/H.264 |  |  |  |  |
|  | Bit-rate | 1Mbps~13Mbps each channel |  |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |  |
|  | GOP Structure | IP…P (P Frame adjustment, without B Frame ) |  |  |  |  |
| Audio (HDMI Input) | Encoding | MPEG-1 Layer 2, LC-AAC, HE-AAC and AC3 Pass through |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Resolution | 24-bit |  |  |  |  |
|  | Audio Gain | 0-255 Adjustable |  |  |  |  |
|  | MPEG-1 Layer 2 Bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | LC-AAC Bit-rate | 48/56/64/80/96/112/128/160/192/224/256/320/384 kbps |  |  |  |  |
|  | HE-AAC Bit-rate | 48/56/64/80/96/112/128 kbps |  |  |  |  |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |  |  |
|  |  | Symbol Rate |  | 0.5~45 Msps |  |  |
|  |  | Constellation |  | QPSK |  |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |  |  |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps 32APSK:0.5~40 Msps |  |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/32APSK |  |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9,9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |  |  |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |  |  |
|  | DVB-S2X | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |  |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/8APSK/32APSK |  |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |  |  |
|  | Signal Strength | -65 ~ -25dBm |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 255 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Modulation | DVB-C | QAM Channel |  |  | 12 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | Standard |  |  | EN300 429/ITU-T J.83A/B |  |
|  |  | MER |  |  | ≥40db |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  |  | -20~+3dbm, 0.1db step |  |
|  |  | Symbol Rate |  |  | 5.0Msps~7.0Msps, 1ksps stepping |  |
|  |  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  |  | 8M | 6M |
|  | DVB-T | Standard |  |  | EN300744 |  |
|  |  | FFT mode |  |  | 2K,4K, 8K |  |
|  |  | Bandwidth |  |  | 6M, 7M, 8M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥42 dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 (or 16) non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~ +3dBm, 0.1db step |  |
|  | ATSC | Standard |  |  | ATSC A/53 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | 8VSB |  |
|  |  | FEC |  |  | RS(208 188)+Trellis |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | ACL |  |  | -55 dBc |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~+3dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | Transmission Mode |  |  | 2K, 4K, 8K |  |
|  |  | Code rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 6 (or 16) non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20dBm~+3dBm, 0.1dB stepping |  |
| Stream output | 1 ASI output as mirror of one of RF output carriers (Optional) 12 MPTS output over UDP and RTP/RTSP as mirror of 12 DVB-C carriers, 8/16 MPTS output over UDP and RTP/RTSP as mirror of 8/16 DVB-T carriers(Optional) 8 MPTS output over UDP and RTP/RTSP as mirror of 8 ATSC carriers(Optional) 6/16 MPTS output over UDP and RTP/RTSP as mirror of 6/16 ISDB-T carriers(Optional) 1*1000M Base-T Ethernet interface, GE port |  |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×328mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work); -20~80℃ (Storage) |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |  |

Ordering Information

Input Section:

|  | HDMI | DVB-S2/S2X | DVB-C/T/T2/ISDB-T |
| --- | --- | --- | --- |
| Option 1 | 8 | / | / |
| Option 2 | 16 | / | / |
| Option 3 | 24 | / | / |
| Option 4 | 8 | 8 | / |
| Option 5 | 8 | 16 | / |
| Option 6 | 16 | 8 | / |
| Option 7 | 8 | / | 8 |
| Option 8 | 8 | / | 16 |
| Option 9 | 16 | / | 8 |
| Option 10 | 8 | 8 | 8 |
| Option 11 | 16 | 4 | 4 |

Output Section:

| RF output standard |  | IP output | RF carrier channels | Scrambling |
| --- | --- | --- | --- | --- |
| DVB-C out |  | 12 | 12 | N/A |
| DVB-T out | V1 | 8 | 8 | N/A |
|  | V2 | 16 | 16 | N/A |
| ATSC out |  | 8 | 8 | N/A |
| ISDB-T out | V1 | 6 | 6 | N/A |
|  | V2 | 16 | 16 | N/A |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227