FMUSER FUQ-4653 Encoder Modulator 1 HDMI-1 SDI Input Supporting DVB-C-T-ATSC-ISDB-T RF ASI IP Output MPEG2 MPEG4 AVC H.264 Encoding Multiplexing for Broadcast Hospitality

Product Overview

The FUQ-4653 series integrates encoding, multiplexing, and modulation, featuring modular input modules for HDMI and SDI and outputs including ASI and IP. It processes signals by encoding video in MPEG2 or MPEG4 AVC H.264 and audio in formats like MPEG4-AAC and Dolby Digital AC3. With a modular drawer design and web-based NMS for control, it supports MPTS/SPTS streaming for broadcast and public venue applications.

Key feature

Offers Low Delay Encoding: With multiple delay modes for SDI and HDMI inputs.

Technical Specifications

| HDMI Input | Video | Input | HDMI*1(the other is backup) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |  |  |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |  |  |
|  |  | Chroma | 4:2:0 |  |  |
|  |  | Aspect Ratio | 16:9,4:3 |  |  |
|  | Audio | Encoding | MPEG1 Layer II;MPEG 2-AAC; MPEG 4-AAC and Dolby Digital AC3 2.0 |  |  |
|  |  | Sample rate | 48KHz |  |  |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |  |  |
| HDMI/YPbPr/CVBS 3-in-1 Input (under developing) | Video (HDMI) | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  | Input | HDMI*1 |  |  |
|  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, -for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |  |  |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |  |  |
|  |  | Chroma | 4:2:0 |  |  |
|  |  | Aspect Ratio | 16:9,4:3 |  |  |
|  | Audio (HDMI) | Encoding | MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0 |  |  |
|  |  | Input | HDMI*1 |  |  |
|  |  | Sample rate | 48KHz |  |  |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |  |  |
|  | Video (YpbPr/ CVBS) | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  | Input | YpbPr*1 / CVBS *1 |  |  |
|  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  | Resolution | CVBS: 720x576_50i (PAL); 720x480_60i (NTSC) YpbPr: 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |  |  |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |  |  |
|  |  | Chroma | 4:2:0 |  |  |
|  |  | Aspect Ratio | 16:9, 4:3 |  |  |
|  | Audio (YpbPr/ CVBS) | Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC and Dolby Digital AC3 2.0 |  |  |
|  |  | Interface | 1*Stereo/2*mono |  |  |
|  |  | Sample rate | 48KHz |  |  |
|  |  | Bit rate | 64/96/128/ 192/256/320kbps |  |  |
| SDI Input; Video; Encoding; MPEG2; MPEG4 AVC/H.264. Input; SDI*1. Bitrate; 1-19.5Mbps. Resolution; 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i. Low Delay; Normal, Mode 1, Mode 2. Chroma; 4:2:0. Aspect Ratio; 16:9,4:3. Audio; Encoding; MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0. Sample rate; 48KHz. Bitrate; 64/96/128/ 192/256/320kbps. |
| Modulator Section | DVB-T (Optional) | Standard | DVB-T COFDM |  |  |
|  |  | Bandwidth | 6M, 7M, 8M |  |  |
|  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8. |  |  |
|  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  | MER | Appx 40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF Out | 4*DVB-T carriers combined output |  |  |
|  |  | RF output level | -20 ~ -3dbm, 0.1db step |  |  |
|  | DVB-C (Optional) | Standard | J.83A (DVB-C), J.83B, J.83C |  |  |
|  |  | MER | Appx 40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level | -20~ +3dbm, 0.1db step |  |  |
|  |  | Symbol rate | 3.000~9.000Msps adjustable |  |  |
|  |  | RF Out | 4*DVB-C carriers combined output |  |  |
|  |  |  | J.83A | J.83B | J.83C |
|  |  | Constellation | 16/32/64/128/256QAM | 64/ 256 QAM | 64/ 256 QAM |
|  |  | Bandwidth | 8M | 6M | 6M |
|  | ATSC (Optional) | Standard | ATSC A/53 |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step. |  |  |
|  |  | RF Out | 4*ATSC carriers combined output |  |  |
|  |  | RF output level | -20~ +3dbm, 0.1db step |  |  |
|  |  | Constellation | 8VSB |  |  |
|  | ISDB-T (Optional) (under developing) | Standard | ARIB STD-B31 |  |  |
|  |  | Bandwidth | 6M |  |  |
|  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  | MER | Appx 40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF Out | 1*ISDBT |  |  |
|  |  | RF output level | -20~ +3dbm, 0.1db step |  |  |
| General | System | Local interface | LCD + control buttons |  |  |
|  |  | Remote management | Web NMS |  |  |
|  |  | Stream Out | 2 ASI out (BNC type, same one TS, mirror as MPTS or SPTS) |  |  |
|  |  |  | DVB-C/-T/ATSC: IP (4 MPTS & 2 SPTS) out over UDP, RTP/RTSP (4 RF carriers out) ISDBT: IP (1 MPTS & 2 SPTS) out over UDP, RTP/RTSP (1 RF carrier out) |  |  |
|  |  | DATA Port | 1000M |  |  |
|  |  | NMS interface | RJ45, 100M |  |  |
|  |  | Language | English |  |  |
|  | Physical Specification | Power supply | 100~240VAC, 50/60Hz |  |  |
|  |  | Dimensions | 482*242*44mm |  |  |
|  |  | Operation temp | 0~45℃ |  |  |

Principle Chart

Typical Applications

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227