FMUSER FUQ-4653AB Multi-channel Encoder 8-24 CVBS Input to IP Output MPEG2-MPEG4 AVC-H264 Encoding Multiplexing Digital CATV Satellite Headend

Product Overview

The FUQ-4653AB multi-channel encoder performs MPEG-2 and MPEG4 AVC/H.264 encoding and multiplexing. It converts 8/12/16/24 CVBS inputs into IP output streams. This unit offers optional ASI output and is managed via web interface. It provides cost-effective solutions for digital cable TV headends and satellite broadcasting systems.

Key feature

Integrated MPEG-2 and MPEG4 AVC/H.264 video encoding. Supports 8/12/16/24 CVBS inputs with SPTS or MPTS output. Features MPEG1 Layer 2, LC-AAC, and HE-AAC audio encoding. Delivers IP output via UDP, RTP/RTSP + optional ASI copy. Provides management via web interface for control/updates.

Principle Chart

Technical specification

| Input | 8/12/16/24 CVBS inputs , RCA interface |  |  |  |
| --- | --- | --- | --- | --- |
| Video | Resolution | Input |  | Output |
|  |  | 720*576@50i |  | 720*576/352*288/320*240/320*180/ 176*144/160*120/160*90@50Hz |
|  |  | 720*480@60i |  | 720*480/352*288/320*240/320*180/ 176*144/160*120/160*90@60Hz |
|  | Encoding | MPEG-2, MPEG4 AVC/H.264 |  |  |
|  | Bit-rate | 0.5~5Mbps |  |  |
|  | Rate Control | CBR/VBR |  |  |
|  | GOP Structure | IPPP, IBPBP, IBBPB, IBBBP |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |
| Audio | Encoding | MPEG1 Layer 2, LC-AAC, HE-AAC |  |  |
|  | Sampling rate | 48KHz |  |  |
|  | Bits per sample | 32-bit |  |  |
|  | Bit-rate | 48Kbps~384Kbps (each channel) |  |  |
| Multiplexing | Function | PID remapping (automatically or manually) |  |  |
|  |  | Accurate PCR adjusting |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |
| Stream output | 1 ASI out as copy of MPTS IP output over UDP/RTP/RTSP through Data 1(1000M) or Data 2 (100M) : 8/12 CVBS inputs with 8/12 SPTS and 1 MPTS output 16/24 CVBS inputs with 16/24 SPTS or 1 MPTS output (alternative) |  |  |  |
| System function | Network management (WEB) |  |  |  |
|  | Chinese and English language |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension(W×L×H) |  | 482mm×410mm×44mm |  |
|  | Approx weight |  | 8kg |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |
|  | Power consumption |  | 20W |  |

Order Information

|  | FU-4653KB-8 | FU-4653KB-12 | FU-4653KB-16 | FU-4653KB-24 |
| --- | --- | --- | --- | --- |
| 8 CVBS Input | √ |  |  |  |
| 12 CVBS Input |  | √ |  |  |
| 16 CVBS Input |  |  | √ |  |
| 24CVBS Input |  |  |  | √ |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227