FMUSER FUQ-4653B Encoder Modulator 4x HDMI-SDI Inputs MPEG-2-4 AVC Encoding DVB-ATSC-ISDB-T Outputs IP-RF Streaming Broadcast Headend IPTV

Product overview

The FMUSER FUQ-4653B combines MPEG-2/MPEG-4 AVC encoding and DVB/ATSC/ISDB-T modulation in a 1RU chassis, accepting 4x HDMI/SDI and optional ASI inputs, while delivering IP/UDP/RTP streams or RF output (47-862MHz). It processes signals with <85ms latency in selectable modes and supports MPTS/SPTS IP multiplexing. Compatible with HMS/SMS management systems, this hot-swappable unit is ideal for digital terrestrial TV headends and multi-screen IPTV distribution.

Key feature

Modular design: Hot-swappable encoder/ASI boards with tool-free maintenance

Principle Chart

Specifications

| HDMI Encoder Board | Video | Input | HDMI*2/board |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |  |  |
|  |  | Support object resolution (for downscale conversion) | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |  |  |
|  |  | Low Delay | Normal, Mode 1, Mode 2, Manual |  |  |
|  |  | Chroma | 4:2:0 |  |  |
|  |  | Aspect Ratio | 16:9,4:3 |  |  |
|  | Audio | Encoding | MPEG1 Layer II;MPEG 2-AAC; MPEG 4-AAC and Dolby Digital AC3 2.0(optional) |  |  |
|  |  | Dialog Normalization Applicable for DD AC3 encoding only) | -31 ~ -1 dB |  |  |
|  |  | Sample rate | 48KHz |  |  |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |  |  |
| SDI Encoder Board | Video; Encoding; MPEG2; MPEG4 AVC/H.264. Input; SDI*2/board. Bitrate; 1-19.5Mbps. Resolution; 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i. Support object resolution (for downscale conversion); 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i. Low Delay; Normal, Mode 1, Mode 2, Manual. Chroma; 4:2:0. Aspect Ratio; 16:9,4:3. Audio; Encoding; MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0(optional). Dialog Normalization Applicable for DD AC3 encoding only); -31 ~ -1 dB. Sample rate; 48KHz. Bitrate; 64/96/128/ 192/256/320kbps. |
| Modulator Section | DVB-T (Optional) | Standard | DVB-T |  |  |
|  |  | Bandwidth | 6M, 7M, 8M |  |  |
|  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8. |  |  |
|  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  | MER | ≥40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF Out | 4*DVB-T carriers(40MHz bandwidth) |  |  |
|  |  | RF output level | -20~ -3dbm, 0.1db step |  |  |
|  | DVB-C (Optional) | Standard | J.83A (DVB-C), J.83B, J.83C |  |  |
|  |  | MER | ≥40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level | -20~ +3dbm, 0.1db step |  |  |
|  |  | Symbol rate | 3.000~9.000Msps adjustable |  |  |
|  |  | RF Out | 4*DVB-C carriers(40MHz bandwidth) |  |  |
|  |  |  | J.83A | J.83B | J.83C |
|  |  | Constellation | 16/32/64/128/256QAM | 64/ 256 QAM | 64/ 256 QAM |
|  |  | Bandwidth | 8M | 6M | 6M |
|  | ATSC (Optional) (under developing) | Standard | ATSC A/53 |  |  |
|  |  | MER | ≥40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step. |  |  |
|  |  | RF Out | 4*ATSC carriers |  |  |
|  |  | RF output level | -20~ +3dbm, 0.1db step |  |  |
|  |  | Constellation | 8VSB |  |  |
|  | ISDB-T (Optional) | Standard | ARIB STD-B31 |  |  |
|  |  | Bandwidth | 6M |  |  |
|  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  | MER | ≥40dB |  |  |
|  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  | RF Out | 2*ISDBT carriers(30MHz bandwidth) |  |  |
|  |  | RF output level | -20~ -3dbm, 0.1db step |  |  |
| General | System | Local interface | LCD + control buttons |  |  |
|  |  | Remote management | Web NMS |  |  |
|  |  | Stream Out | 2 ASI out (BNC type, same one mirror out ts as one of 4/2 MPTS outputs and 4 SPTS outputs) |  |  |
|  |  |  | DVB-C/DVB-T/ATSC: IP (4 MPTS & 4 SPTS) out over UDP, RTP/RTSP ISDBT: IP (2 MPTS & 4 SPTS) out over UDP, RTP/RTSP |  |  |
|  |  | DATA Port | 1000M |  |  |
|  |  | NMS interface | RJ45, 100M |  |  |
|  |  | Language | English |  |  |
|  | Physical Specification | Power supply | 100~240VAC, 50/60Hz |  |  |
|  |  | Dimensions | 482*400*44mm |  |  |
|  |  | Operation temperature | 0~45℃ |  |  |

Major Technical Comparison

|  | The Former Version | The Current Version（V2） |
| --- | --- | --- |
| Bit rate Mode | CBR/VBR option | CBR |
| Audio Group/Pair option-SDI | No | Yes |
| Low Delay | Normal/Mode 1/Mode 2 | Normal/Mode 1/Mode 2/Manual |
| Character Encoding option | No | Yes |
| DATA Port | 100M port | 1000M port |
| Frequency Range | 30-960MHz | 50-960MHz |
| Symbol Rate--DVB-C | 5-9Msps | 3-9Msps |
| ISDB-T carriers | 1 or 2 carriers for order option | 2 carriers |
| IP out--DVB-C | 1 MPTS and 4 SPTS | 4 MPTS and 4 SPTS |
| IP out--DVB-T | 4 SPTS | 4 MPTS and 4 SPTS |
| DVB-T carriers | 2 carriers | 4 carriers |

The other differences are existing Web GUI design and chassis design etc(NMS/DATA port position), no further description here.

Internal Test Report of Latency

The values cover the progress from Encoding → Decoding

| Decoding Terminal | Encoding Details |  |  |  |  | Average Latency (ms) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Resolution | Encoding Bit Rate | Low Latency Mode | Signal Source | Encoding Type |  |
| DVB-C HD STB | 1080i@50 | 14M | MODE 1 | HDMI | mpeg2 | 170 |
|  |  |  |  |  | H.264 | 347.5 |
|  |  |  |  | SDI | mpeg2 | 227.5 |
|  |  |  |  |  | H.264 | 367.5 |
|  |  |  | MODE 2 | HDMI | mpeg2 | 222.5 |
|  |  |  |  |  | H.264 | 395 |
|  |  |  |  | SDI | mpeg2 | 240 |
|  |  |  |  |  | H.264 | 397.5 |
| DVB-C HD STB | 720p@50 | 14M | MODE 1 | HDMI | mpeg2 | 85.75 |
|  |  |  |  |  | H.264 | 237.5 |
|  |  |  |  | SDI | mpeg2 | 127.5 |
|  |  |  |  |  | H.264 | 295 |
|  |  |  | MODE 2 | HDMI | mpeg2 | 182.5 |
|  |  |  |  |  | H.264 | 277.5 |
|  |  |  |  | SDI | mpeg2 | 167.5 |
|  |  |  |  |  | H.264 | 325 |
| DVB-C HD STB | 576i@50 | 14M | MODE 1 | HDMI | mpeg2 | 310 |
|  |  |  |  |  | H.264 | 600 |
|  |  |  |  | SDI | mpeg2 | 330 |
|  |  |  |  |  | H.264 | 620 |
|  |  |  | MODE 2 | HDMI | mpeg2 | 270 |
|  |  |  |  |  | H.264 | 610 |
|  |  |  |  | SDI | mpeg2 | 280 |
|  |  |  |  |  | H.264 | 620 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227