FMUSER FUQ-4653C Encoder Modulator 4x HDMI-YPbPr-CVBS Inputs Digital RF Output MPEG2-MPEG4 AVC H.264 Encoding DVB-C-DVB-T IP Output Hospitality Broadcasting Product Overview

The FMUSER FUQ-4653G is an all-in-one encoder modulator with 4x HDMI, YPbPr, and CVBS inputs, converting to digital RF output. It encodes video using MPEG2 and MPEG4 AVC H.264, and modulates signals for distribution. Features include an LCD display and web-based NMS for control. Ideal for public venues, supporting sources like satellite receivers.

Key feature

Integrated MPEG2/MPEG4 AVC H.264 encoding for HD/SD video.

Supports DD AC3, MPEG4-AAC, MPEG2-AAC, and MPEG1 Layer II audio formats.

Features 4x HDMI, YPbPr, and CVBS inputs.

Provides DVB-C, DVB-T, optional ISDB-T, ATSC, ASI, and IP outputs.

Includes web-based NMS for remote monitoring and updates.

Principle Chart

Specifications

| Encoder Section | HDMI | Video | Interface | HDMI*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  |  | Resolution | 1920*1080_60P, 1920*1080_50P (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P |  |  |
|  |  |  | Low Delay | Normal, Mode 1, Mode 2, Manual |  |  |
|  |  |  | Chroma | 4:2:0 |  |  |
|  |  |  | Aspect Ratio | 16:9,4:3 |  |  |
|  |  | Audio | Interface | HDMI*4 /SPDIF*4/Audio(L/R)*4 |  |  |
|  |  |  | Encoding | MPEG1 Layer II; MPEG 2-AAC; MPEG 4-AAC and Dolby Digital AC3 2.0 (optional) |  |  |
|  |  |  | Dialog Normalization | -31 ~ -1 dB (Applicable for DD AC3 only) |  |  |
|  |  |  | Sample rate | 48KHz |  |  |
|  |  |  | Bitrate | 64/96/128/ 192/256/320kbps |  |  |
|  | CVBS /YPbPr | Video | Encoding | MPEG2; MPEG4 AVC/H.264 |  |  |
|  |  |  | Interface | CVBS/YPbPr*4 (RCA) |  |  |
|  |  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  |  | Resolution | YPbPr: 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P CVBS: 720*480_60i, 720*576_50i |  |  |
|  |  |  | Low Delay | Normal, Mode 1, Mode 2, Manual |  |  |
|  |  |  | Chroma | 4:2:0 |  |  |
|  |  |  | Aspect Ratio | 16:9,4:3 |  |  |
|  |  | Audio | Encoding | MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0 (optional) |  |  |
|  |  |  | Interface | HDMI*4 /SPDIF*4/Audio(L/R)*4 |  |  |
|  |  |  | Dialog Normalization | -31 ~ -1 dB (Applicable for DD AC3 only) |  |  |
|  |  |  | Sample rate | 48KHz |  |  |
|  |  |  | Bitrate | 64/96/128/ 192/256/320kbps |  |  |
|  | Resolution Downscale |  | In Resolution | Available Out Resolution |  |  |
|  |  |  | 1920*1080_60P | 1920*1080_60P, 1920*1080_60i, 1440*1080_60i, 1280*720_60p, 720*480_60i |  |  |
|  |  |  | 1920*1080_50P | 1920*1080_50P, 1920*1080_50i, 1440*1080_50i, 1280*720_50P, 720*576_50i |  |  |
|  |  |  | 1920*1080_60i | 1920*1080_60i, 1440*1080_60i, 720*480_60i |  |  |
|  |  |  | 1920*1080_50i | 1920*1080_50i, 1440*1080_50i, 720*576_50i |  |  |
|  |  |  | 1280*720_60p | 1280*720_60p, 720*480_60i |  |  |
|  |  |  | 1280*720_50P | 1280*720_50P, 720*576_50i |  |  |
|  |  |  | 720*480_60i | 720*480_60i |  |  |
|  |  |  | 720*576_50i | 720*576_50i |  |  |
| Modulator Section | DVB-T (Optional) |  | Standard | DVB-T |  |  |
|  |  |  | Bandwidth | 6M, 7M, 8M |  |  |
|  |  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  |  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  |  | MER | About 40dB |  |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  |  | RF Out | 4*DVB-T carriers |  |  |
|  |  |  | RF output level | -20~ -9dbm, 0.1db step |  |  |
|  | DVB-C (Optional) |  | Standard | J.83A (DVB-C), J.83B, J.83C |  |  |
|  |  |  | MER | About 40dB |  |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  |  | RF output level | -20~ -3dbm, 0.1db step |  |  |
|  |  |  | Symbol rate | 3.000~9.000Msps adjustable |  |  |
|  |  |  | RF Out | 4*DVB-C carriers |  |  |
|  |  |  |  | J.83A | J.83B | J.83C |
|  |  |  | Constellation | 16/32/64/128/256QAM | 64/ 256 QAM | 64/ 256 QAM |
|  |  |  | Bandwidth | 8M | 6M | 6M |
|  | ATSC (Optional) under developing |  | Standard | ATSC A/53 |  |  |
|  |  |  | Constellation | 8 VSB |  |  |
|  |  |  | MER | About 40dB |  |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step. |  |  |
|  |  |  | RF Out | 4*ATSC carriers |  |  |
|  |  |  | RF output level | -20~ -9dbm, 0.1db step |  |  |
|  |  |  | Constellation | 8VSB |  |  |
|  | ISDB-T (Optional) under developing |  | Standard | ARIB STD-B31 |  |  |
|  |  |  | Bandwidth | 6M |  |  |
|  |  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  |  | Transmission Mode | 2K, 4K, 8K |  |  |
|  |  |  | MER | About 40dB |  |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |  |
|  |  |  | RF Out | 2*ISDBT carriers |  |  |
|  |  |  | RF output level | -20~ -9dbm, 0.1db step |  |  |
| General | System |  | Local interface | LCD + control buttons |  |  |
|  |  |  | Remote management | Web NMS |  |  |
|  |  |  | Stream Out | 2 ASI out (BNC type, the same one mirror ts out as one of 4/2 MPTS outputs and 4 SPTS outputs) |  |  |
|  |  |  |  | DVB-C/DVB-T/ATSC: IP (4 MPTS & 4 SPTS) out over UDP, RTP/RTSP ISDBT: IP (2 MPTS & 4 SPTS) out over UDP, RTP/RTSP |  |  |
|  |  |  | DATA Port | 1000M |  |  |
|  |  |  | NMS interface | RJ45, 100M |  |  |
|  |  |  | Language | English |  |  |
|  | Physical Specification |  | Power supply | 100~240VAC, 50/60Hz |  |  |
|  |  |  | Dimensions | 482*400*44mm |  |  |
|  |  |  | Operation temperature | 0~45℃ |  |  |

Major Technical Comparison

|  | The Former Version | The Current Version（V2） |
| --- | --- | --- |
| DD AC3 (2.0/5.1/7.1) passthrough | Yes | No |
| Bit rate Mode | CBR/VBR option | CBR |
| Low Delay | Normal/Mode 1/Mode 2 | Normal/Mode 1/Mode 2/Manual |
| Character Encoding option | No | Yes |
| DATA Port | 100M port | 1000M port |
| Frequency Range | 30-960MHz | 50-960MHz |
| Symbol Rate--DVB-C | 5-9Msps | 3-9Msps |
| IP out--DVB-C | 1 MPTS and 4 SPTS | 4 MPTS and 4 SPTS |
| IP out--DVB-T | 4 SPTS | 4 MPTS and 4 SPTS |
| DVB-T carriers | 2 carriers | 4 carriers |

The other differences are existing Web GUI design and chassis design etc(NMS/DATA port position), no further description here.

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227