FMUSER FUQ-4653D Encoder Modulator 2x HDMI YPbPr CVBS Input DVB-C ATSC DVB-T ISDB-T Output MPEG2 MPEG4 AVC H.264 Encoding Web NMS Remote Management for Hotel IPTV and Public Venues

Product Overview

The FMUSER FUQ-4653D is an encoder modulator with MPEG2 HD and MPEG4 HD encoding, featuring inputs: 2x HDMI, YPbPr, CVBS, and outputs: RF, 2x ASI, IP. It converts signals through encoding and modulation for digital broadcasting. Controlled via web-based NMS, it supports remote management and updates. Ideal for hotels, metros, and public venue applications.

Key feature

- Features integrated MPEG2 HD and MPEG4 HD encoding for high-definition processing.

- Supports modulation standards: DVB-C, ATSC, DVB-T, ISDBT with dynamic switching.

- Provides flexible I/O: 2x HDMI/YPbPr/CVBS inputs, 2x ASI outputs, IP streaming over UDP/RTP/RTSP.

- Includes web-based NMS for remote control and firmware updates.

Specifications

| Encoding Section |  |
| --- | --- |
| Video (HDMI) |  |
| Encoding | MPEG2; MPEG4 AVC/H.264 |
| Interface | HDMI*2 |
| Resolution | 1920*1080_60P, 1920*1080_50P (For MPEG 4 AVC/H.264 only), 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P |
| Low Delay | Normal, Mode 1, Mode 2, Manual |
| Audio (HDMI) |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC; DD AC3(2.0) optional; DD AC3 (2.0/5.1/7.1) passthrough |
| Interface | HDMI*2/SPDIF*2/2*Stereo/4*mono |
| Sample rate | 48KHz |
| Bit rate | 64/96/128/ 192/256/320kbps |
| Video (CVBS/YPbPr) |  |
| Encoding | MPEG2; MPEG4 AVC/H.264 |
| Interface | CVBS/YPbPr*2(RCA) |
| Resolution | CVBS: 720*576_50i, 720*480_60i YPbPr:1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |
| Audio (L/R) |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC; DD AC3(2.0) optional; |
| Interface | 2*Stereo/4*mono |
| Sample rate | 48KHz |
| Bit rate | 64/96/128/ 192/256/320kbps |

| DVB-C Modulator Section |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Standard |  |  | J.83A (DVB-C), J.83B, J.83C |  |  |
| MER |  |  | ≥40dB |  |  |
| RF frequency |  |  | 30~960MHz, 1KHz step |  |  |
| RF output level |  |  | -30~ -10dbm (77~97 dbµV), 0.1db step |  |  |
| Symbol rate |  |  | 5.000~9.000Msps adjustable |  |  |
| RF Out | 4*DVB-C adjacent carriers combined output |  |  |  |  |
|  |  | J.83A |  | J.83B | J.83C |
| Constellation |  | 16/32/64/128/ 256 QAM |  | 64/ 256 QAM | 64/ 256 QAM |
| Bandwidth |  | 8M |  | 6M | 6M |

| DVB-T Modulator Section |  |
| --- | --- |
| Standard | EN300744 |
| FFT mode | 2K, 8K |
| Bandwidth | 6M, 7M, 8M |
| Constellation | QPSK, 16QAM, 64QAM |
| Guard Interval | 1/4, 1/8, 1/16, 1/32 |
| FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
| MER | ≥40dB |
| RF frequency | 30~960MHz, 1KHz step |
| RF out | COFDM DVB-T out (2 carriers combined out) |
| RF output level | -30~ -10dbm (77~97 dbµV), 0.1db step |

| ATSC Modulator Section |  |  |  |  |
| --- | --- | --- | --- | --- |
| Standard |  | ATSC A/53 |  |  |
| Constellation |  | 8 VSB |  |  |
| RF output level |  | -30~ -10dbm (77~97 dbµV), 0.1db step |  |  |
| MER |  | ≥40dB |  |  |
| RF frequency |  | 36~960MHz, 1KHz step |  |  |
| RF Out |  | 4*ATSC adjacent carriers combined output |  |  |
| ISDBT Modulator Section |  |  |  |  |
| Standard |  | ARIB STD-B31 |  |  |
| Bandwidth |  | 6M |  |  |
| Constellation |  | DQPSK,QPSK, 16QAM, 64QAM |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |
| Transmission Mode |  | 2K, 4K, 8K |  |  |
| MER |  | ≥40dB |  |  |
| RF frequency |  | 30~960MHz, 1KHz step |  |  |
| RF output level |  | -30~ -10dbm (77~97 dbµV), 0.1db step |  |  |
| RF Out | ISDBT adjacent carriers combined output |  |  |  |
| System |  |  |  |  |
| Local interface |  |  | LCD + control buttons |  |
| Remote management |  |  | Web NMS |  |
| Stream Out |  |  | 2 separate ASI out (BNC type); IP (2* SPTS&1*MPTS) over UDP/RTP/RTSP out (RJ45, 100M) RTSP is only for DVB-C modulation |  |
| NMS interface |  |  | RJ45, 1000M |  |
| Language |  |  | English |  |
| General |  |  |  |  |
| Power supply |  |  |  | AC 100V~240V |
| Dimensions |  |  |  | 333*232*44mm |
| Weight |  |  |  | 2.5 kg |
| Operation temperature |  |  |  | 0~45℃ |

Principle Chart

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227