FMUSER FUQ-4653E Modulator 4 Tuner + 2 ASI Inputs DVB-T-T2-S-S2-S2X to DVB-C RF ASI UDP IP Outputs Integrated Demodulation Transmux Modulation for Broadcast HeadendsProduct overview

The FMUSER FUQ-4653E modulator integrates demodulation, trans-mux, and modulation in a 1-U case, supporting 4 tuner inputs for satellite/cable/terrestrial sources and 2 ASI inputs, with outputs including RF, ASI, and UDP IP ports. It processes signals by converting them for distribution, featuring an LCD display, keyboard, and Web NMS for control via web or USB. Ideal for broadcasting headends, cable TV systems, and IP-based content distribution networks.

Key feature

Delivers RF outputs (e.g., DVB-T or DVB-C), ASI outputs, and UDP IP outputs (MPTS or SPTS)

Working Principle

Specifications

| Input | 4 Tuner (DVB-T/(T)/S/S2/S2X optional) and 2 ASI input |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | DVB-T/(T) |  |  | Input Frequency | 30~1000MHz |  |  |
|  |  |  |  | Bandwidth | 6/7/8M |  |  |
|  | Version 1 | DVB-S |  | Input Frequency | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate | 1~45 Msps |  |  |
|  |  |  |  | Signal Strength | -65~-25dBm |  |  |
|  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  | Constellation | QPSK |  |  |
|  |  |  |  | Max input bitrate | ≤120 Mbps |  |  |
|  |  | DVB-S2 |  | Input Frequency | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate | QPSK 1~45 Msps 8PSK 2~30 Msps |  |  |
|  |  |  |  | FEC | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  | Constellation | QPSK, 8PSK |  |  |
|  |  |  |  | Max input bitrate | ≤120 Mbps |  |  |
|  | Version 2 (New) | DVB-S |  | Input Frequency | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate | 0.5~45Msps |  |  |
|  |  |  |  | Signal Strength | - 65- -25dBm |  |  |
|  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  | Constellation | QPSK |  |  |
|  |  |  |  | Max input bitrate | ≤160 Mbps |  |  |
|  |  | DVB-S2 |  | Input Frequency | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |  |  |
|  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |  |  |
|  |  |  |  | Max input bitrate | ≤160 Mbps |  |  |
|  |  | DVB-S2X |  | Input Frequency | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |  |  |
|  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |  |
|  |  |  |  | Max input bitrate | ≤160 Mbps |  |  |
| Multiplexing | Maximum PID Remapping |  |  | 128per input channel |  |  |  |
|  | Function |  |  | PID remapping (automatically or manually) |  |  |  |
|  |  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  |  | Generate PSI/SI table automatically |  |  |  |
| Modulation | DVB-C |  |  | Standard | J.83A(DVB-C), J.83B, J.83C |  |  |
|  |  |  |  | MER | ≥40dB |  |  |
|  |  |  |  | RF frequency | 30~999MHz, 1KHz step |  |  |
|  |  |  |  | RF output level | -20~0dbm(87~107 dbµV),0.1db step |  |  |
|  |  |  |  | Symbol rate | 5.0Msps-9.0Msps, 1ksps stepping |  |  |
|  |  |  |  | RF Out | 4*DVB-C carriers output |  |  |
|  |  |  |  |  | J.83A | J.83B | J.83C |
|  |  |  |  | Constellation | 16/32/64/128/256QAM | 64/256 QAM | 64/256 QAM |
|  |  |  |  | Bandwidth | 8M | 6M | 6M |
|  | DVB-T |  |  | Bandwidth | 6M, 7M, 8M |  |  |
|  |  |  |  | Constellation | QPSK, 16QAM, 64QAM |  |  |
|  |  |  |  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |  |  |
|  |  |  |  | Transmission Mode | 2K, 8K |  |  |
|  |  |  |  | MER | ≥40dB |  |  |
|  |  |  |  | RF frequency | 30~999MHz, 1KHz step |  |  |
|  |  |  |  | RF Out | 2*DVB-T carriers output |  |  |
|  |  |  |  | RF output level | -20~0dbm(87~107 dbµV),0.1db step |  |  |
| TS Output | 2 ASI out as mirror of 2 RF carriers (BNC type) 2 MPTS over UDP out (RJ45, 100/1000M self-adaption) (for DVB-T RF out) 4 MPTS over UDP out (RJ45, 100/1000M self-adaption) (for DVB-C RF out) ---versionⅠ 8 SPTS over UDP out (RJ45, 100/1000M self-adaption) (for DVB-C RF out) --- versionⅡ |  |  |  |  |  |  |
| System | Local interface |  | LCD + control buttons |  |  |  |  |
|  | Remote management |  | Web NMS |  |  |  |  |
|  | Language |  | English and Chinese |  |  |  |  |
|  | Upgrading |  | Web and USB |  |  |  |  |
| General | Dimension |  | 482mm×300mm×44.5mm(W*D*H) |  |  |  |  |
|  | Weight |  | 3.7kg |  |  |  |  |
|  | Temperature |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |  |  |
|  | Power |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |  |  |
|  | Consumption |  | 25W |  |  |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227