FMUSER FUQ-4653H-HA Encoder Modulator 8-24 CVBS Inputs 4 RF Outputs DVB-C-T-ISDBT MPEG-2 Scrambling Web Management for Hotel IPTV and CATV 8/12/24* CVBS/1 DVB-C/T/(T)/ISDBT Tuner ——> 4* DVB-C/T Digital RF

DVB-T RF out front Panel

Product Overview

The FUQ-4653H/HA combines MPEG-2 encoding, multiplexing, and DVB-C/T/ISDBT modulation in a 1RU chassis. It processes 8-24 CVBS inputs, 128 IP streams, and delivers 4 RF outputs with adjustable levels. This web-managed solution supports PSI/SI editing and ETSI-compliant scrambling. Ideal for hotel TV systems, hospital networks, and small-scale CATV operators.

Key feature

Broadcast Security: ETSI-compliant scrambling (H/HA models) and OSD/QR code insertion (HA model) for content control.

Principle Chart

DVB-C RF output

DVB-T RF output

Technical specification

| Input | 8/12/16/20/24 CVBS inputs , RCA interface , 1 DVB-C/T/(T)/ISDBT Tuner for remux, F type interface (only for DVB-C RF out) 128 IP input over UDP and RTP, DATA1 and DATA2,RJ45 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | DVB-C | Standard |  |  | J.83A(DVB-C), J.83B, J.83C |  |
|  |  | Frequency In |  |  | 60 MHz~890 MHz |  |
|  |  | Constellation |  |  | 16/32/64/128/256 QAM |  |
|  | DVB-T/(T) | Frequency In |  |  | 60 MHz~890 MHz |  |
|  |  | Bandwidth |  |  | 6/7/8 M bandwidth |  |
|  | ISDB-T | Frequency In |  |  | 60 MHz~890 MHz |  |
| Video | Resolution | 720×480_60i, 544×480_60i, 352×480_60i 352×240_60i,320×240_60i,176×240_60i, 176×120_60i |  |  |  |  |
|  |  | 720×576_50i,704×576_50i,640×576_50i, 352×288_50i 320×288_50i, 176×288_50i, 176×144_50i |  |  |  |  |
|  | Encoding | MPEG-2 |  |  |  |  |
|  | Bit-rate | 0.5Mbps~8Mbps each channel |  |  |  |  |
|  | Rate Control | CBR, VBR |  |  |  |  |
|  | GOP Structure | GOP_0_B, GOP_1_B, GOP_2_B, GOP_3_B |  |  |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |  |  |
| Audio | Encoding | MPEG-1 Layer 2, AC3 (2.0) |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Resolution | 24-bit |  |  |  |  |
|  | Bit-rate | 64Kbps,128Kbps,192kbps,256kbps,320kbps,384kbps each channel |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 180 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Scrambling (Only for DVB-C RF out) | Maximum simulcrypt CA | 4 |  |  |  |  |
|  | Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |  |
|  | Connection | Local/remote connection |  |  |  |  |
| Modulation | DVB-C | RF out |  | 4*RF DVB-C out (4 carriers combined output) |  |  |
|  |  | Standard |  | EN300 429/ITU-T J.83A/B |  |  |
|  |  | MER |  | ≥40db |  |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level |  | -25 ~ -1 dBm, 1dBm step |  |  |
|  |  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps stepping |  |  |
|  |  | Constellation |  | 16/32/64/128/256QAM |  |  |
|  |  | Constellation |  | J.83A |  | J.83B |
|  |  |  |  | 16/32/64/128/256QAM |  | 64/256 QAM |
|  |  | Bandwidth |  | 8M |  | 6M |
|  | DVB-T | RF out |  | 4*RF DVB-T out (4 carriers combined output) |  |  |
|  |  | Standard |  | EN300744 |  |  |
|  |  | FFT mode |  | 2K, |  |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  | MER |  | ≥42 dB |  |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level |  | -28~ -3 dBm, 1 dBm step |  |  |
| Stream output | RF output (F type interface) |  |  |  |  |  |
|  | 4 IP MPTS output over UDP/RTP, 1*1000M Base-T Ethernet interface |  |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |  |
|  | English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×410mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |  |

Order Information

|  | FU-4653H (DVB-C RF out) | FU-4653H (DVB-T RF out) | FU-4653HA (DVB-C RF out) | FU-4653HA (DVB-T RF out) |
| --- | --- | --- | --- | --- |
| Scrambling Function | √ |  | √ |  |
| DVB-C/T/(T)/ISDBT Tuner Input | √ |  | √ |  |
| OSD Insertion Function |  |  | √ | √ |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227