FMUSER FUQ-4653L-KA-8 8ch Encoder Multiplexing CVBS Input to IP-ASI Output MPEG-2 MPEG1 Layer II AC3 PAL-NTSC Digital Distribution Headend

Product Overview

The FUQ-4653B/FUQ-4653AB Encoder processes 8/12/24 CVBS inputs to deliver IP and ASI outputs with encoding and multiplexing. It converts signals by encoding video in MPEG-2 and audio in MPEG1 Layer II/AC3, supporting PAL/NTSC formats. Control is via web-based management for configuration and updates. Ideal for digital head-end systems in broadcasting and cable TV applications.

Key feature

For FUQ-4653AB: Adds QR Code, Caption, and Logo insertion capabilities

Principle Chart

Technical specification

| Input | 8/12/24CVBS inputs , RCA interface |  |  |
| --- | --- | --- | --- |
| Video | Resolution | 720×480_60i, 544×480_60i, 352×480_60i 352×240_60i, 320×240_60i,176×240_60i, 176×120_60i |  |
|  |  | 720×576_50i, 704×576_50i, 640×576_50i, 352×288_50i, 320×288_50i, 176×288_50i, 176×144_50i |  |
|  | Encoding | MPEG-2 |  |
|  | Bit-rate | 0.5Mbps~8Mbps each channel |  |
|  | Rate Control | CBR/VBR |  |
|  | GOP Structure | GOP_0_B, GOP_1_B, GOP_2_B, GOP_3_B |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |
| Audio | Encoding | MPEG-1 Layer 2, AC3(2.0) |  |
|  | Sampling rate | 48KHz |  |
|  | Resolution | 24-bit |  |
|  | Bit-rate | 64Kbps,128Kbps,192kbps,256kbps,320kbps,384kbps each channel |  |
| Multiplexing | Function | PID remapping (automatically or manually) |  |
|  |  | Accurate PCR adjusting |  |
|  |  | Generate PSI/ SI table automatically |  |
| Stream output | 1 ASI as copy of MPTS IP (MPTS/SPTS) output over UDP/RTP/RTSP 8/12 CVBS inputs with 8/12 SPTS and 1 MPTS output 24×CVBS inputs with 24 SPTS or 1 MPTS output |  |  |
| System function | Network management (WEB) |  |  |
|  | Chinese and English language |  |  |
|  | Ethernet software upgrade |  |  |
| Miscellaneous | Dimension(W×L×H) |  | 482mm×410mm×44mm |
|  | Approx weight |  | 8kg |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |
|  | Power consumption |  | 20W |

Order Information

|  | FU-4653K/KA-8 | FU-4653K/KA-12 | FU-4653K/KA-24 |
| --- | --- | --- | --- |
| 8CVBS Input | √ |  |  |
| 12CVBS Input |  | √ |  |
| 24CVBS Input |  |  | √ |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227