FMUSER FUQ-4653U 4K Encoder Modulator Dual HDMI 2.0 2xASI 32xIP Inputs DVB-C-T ASI-IP Outputs H.265 HEVC Encoding IPTV Broadcast Headend

Product Overview

The FMUSER FUQ-4653U is a professional 4K encoder modulator supporting dual HDMI 2.0, 2xASI, and 32xIP inputs with DVB-C/T RF, ASI, and IP outputs. It performs H.265/HEVC encoding and DVB modulation in a 1U rack-mountable chassis. The device features web-based NMS for remote control and supports PSI/SI table generation. Ideal for digital terrestrial television (DTT) headends and IPTV distribution systems requiring 4K UHD processing.

Key feature

Flexible Audio: MPEG-1 Layer 2 and LC-AAC audio encoding options

Specifications

| Input | 1×HDMI input (2.0) with 2 HDMI port, HDCP 2.2/1.4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2 ASI input, BNC interface |  |  |  |  |  |
|  | 32 IP input over UDP and RTP protocol |  |  |  |  |  |
| Video Input | Resolution |  |  |  | Chroma |  |
|  | 3840×2160_60P,3840×2160_59.94P, 3840×2160_50P,3840×2160_30P, 3840×2160_29.96P,3840×2160_25P, 3840×2160_24P,1920×1080_60P, 1920×1080_59.94P,1920×1080_50P, 1920×1080_30P ,1920×1080_25P 1920×1080_24P, |  |  |  | RGB, YCbCr 4:4:4, YCbCr 4:2:2, YCbCr 4:2:0 (2160P_50/60Hz only) |  |
| Video Encoding | Encoding |  | H.265/HEVC, H.264/AVC |  |  |  |
|  | Chroma |  | 4:2:2, 4:2:0 |  |  |  |
|  | Bit-rate |  | 1Mbps~23Mbps |  |  |  |
|  | Rate Control |  | CBR |  |  |  |
| Audio Encoding | Encoding |  | MPEG-1 Layer 2, LC-AAC |  |  |  |
|  | Sampling rate |  | 48KHz |  |  |  |
|  | Bit-rate |  | 64Kbps~256Kbps each channel |  |  |  |
| Multiplexing | 2 ASI input multiplexed with local 1 channel of TS |  |  |  |  |  |
|  | PID remapping (automatically or manually) |  |  |  |  |  |
|  | Accurate PCR adjusting |  |  |  |  |  |
|  | Generate PSI/SI table automatically |  |  |  |  |  |
| Modulator Section | DVB-C | Standard |  | J.83A, J.83B |  |  |
|  |  | MER |  | ≥43dB |  |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level |  | -25.0~ -1 dbm, 0.1db step |  |  |
|  |  | Symbol rate |  | 5.0 - 7.0 Msps |  |  |
|  |  |  |  | J.83A |  | J.83B |
|  |  | Constellation |  | 16/32/64/128/ 256 QAM |  | 64/ 256 QAM |
|  |  | Bandwidth |  | 8M |  | 6M |
|  |  | RF out |  | 1*RF DVB-C out |  |  |
|  | DVB-T | Standard |  | EN300744 |  |  |
|  |  | FFT mode |  | 2K/4K/8K |  |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |  |
|  |  | Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  | MER |  | ≥42dB |  |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  |  | RF output level |  | -28~ -3dbm, 0.1db step |  |  |
|  |  | RF out |  | 1*RF DVB-T out |  |  |
| Stream output | 1*ASI output through 4 BNC interfaces |  |  |  |  |  |
|  | 1 MPTS over UDP/RTP, 1000M/100M Base-T Ethernet interface (unicast/ multicast) |  |  |  |  |  |
| System function | 2 versions of front panel: Option1: with LCD and Keyboard; Option2 without LCD or Keyboard. |  |  |  |  |  |
|  | LCD/keyboard operating(optional), web NMS supporting |  |  |  |  |  |
|  | Ethernet software & hardware upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W× L× H) |  |  | 482mm×455mm×44mm |  |  |
|  | Approx weight |  |  | 4kg |  |  |
|  | Temperature |  |  | 0~45℃(work), -20~80℃（Storage） |  |  |
|  | Power |  |  | AC 100V-220V±10%, 50/60Hz |  |  |
|  | Consumption |  |  | 25W |  |  |

Principle Chart

Resolution Upscale

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227