FMUSER FUQ-4653XY Encoder Modulator 24 CVBS & 128 IP Inputs DVB-C-T SD MPEG-2 H.264 Integrated Multiplexing Scrambling for CATV IPTV

Product Overview

The FMUSER FUQ-4653LM integrates SD encoding, multiplexing, scrambling, and modulation, supporting up to 24 CVBS inputs, an optional DVB-C/T/ISDBT/S/S2 tuner, and 128 IP inputs via Data1 (GE) and Data2 (FE) ports. It processes signals through encoding and modulation, delivering 4 DVB-C/T RF outputs and 4 MPTS IP streams. Managed via a web interface, this device is ideal for CATV headends and hospitality TV systems.

Key feature

- Supports multi-format encoding with MPEG-2/H.264 video and MPEG1 Layer 2/LC-AAC/HE-AAC audio for up to 24 CVBS inputs.

- Integrates optional DVB-C/T/ISDBT/S/S2 tuner and 128 UDP/RTP IP inputs via dual Ethernet ports.

- Delivers 4 DVB-C/T RF outputs (50-960MHz) and 4 MPTS IP streams over UDP/RTP.

- Features advanced signal control including PID remapping, PCR adjustment, PSI/SI editing, and logo/QR code insertion.

- Enables web-managed operations for remote configuration, scrambling control (DVB-C only), and firmware updates.

Principle Chart

DVB-C RF out

DVB-T RF out

Technical specification

| Input | 8/12/16/24 CVBS inputs , RCA interface , 1 DVB-C/T/(T)/ISDBT/S/S2 (Optional) Tuner for re-mux, F type interface 128 IP input over UDP and RTP through DATA1 and DATA2, RJ45 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Video | Resolution | 720*576@50i |  | 720*576/352*288/320*240/320*180/ 176*144/160*120/160*90@50Hz |  |
|  |  | 720*480@60i |  | 720*480/352*288/320*240/320*180/ 176*144/160*120/160*90@60Hz |  |
|  | Encoding | MPEG-2, MPEG4 AVC/H.264 |  |  |  |
|  | Bit-rate | 0.5~5Mbps |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |
|  | GOP Structure | IPPP, IBPBP, IBBPB, IBBBP |  |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |  |
| Audio | Encoding | MPEG1 Layer 2, LC-AAC, HE-AAC |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |
|  | Bits per sample | 32-bit |  |  |  |
|  | Bit-rate | 48Kbps~384Kbps (each channel) |  |  |  |
| Multiplexing | Maximum PID Remapping | 128 PID input per channel |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |
| Scrambling (Only for DVB-C RF out) | Maximum simulcrypt CA | 4 |  |  |  |
|  | Standard | EN300 429/ITU-T J.83A/B |  |  |  |
|  | Connection | Local/remote connection |  |  |  |
| Modulation | DVB-C | RF out |  | 4*RF DVB-C out (4 carriers combined output) |  |
|  |  | Standard |  | EN300 429/ITU-T J.83A/B |  |
|  |  | MER |  | ≥40dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  | -25~-1dBm, 1dBm step |  |
|  |  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps step |  |
|  |  | Constellation |  | 16/32/64/128/256QAM |  |
|  |  | Constellation |  | J.83A | J.83B |
|  |  |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  | 8M | 6M |
|  | DVB-T | RF out |  | 4*RF DVB-T out (4 carriers combined output) |  |
|  |  | Standard |  | EN300744 |  |
|  |  | MER |  | ≥42 dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  | -28~ -3 dBm, 0.1dBm step |  |
|  |  | FFT mode |  | 2K, |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
| Stream output | RF output (F type interface) |  |  |  |  |
|  | 4 IP MPTS output over UDP/RTP through Data1 (1000M Base-T Ethernet interface) |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×410mm×44mm |  |  |
|  | Approx weight |  | 8kg |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |

Order Information

|  | FU-4653HB-8 (DVB-C RF out) | FU-4653B-8 (DVB-T RF out) | FU-4653HB-12 (DVB-C RF out) | FU-4653HB-12 (DVB-T RF out) | FU-4653HB-24 (DVB-C RF out) | FU-4653HB-24 (DVB-T RF out) |
| --- | --- | --- | --- | --- | --- | --- |
| 8 CVBS Input | √ | √ |  |  |  |  |
| 12 CVBS Input |  |  | √ | √ |  |  |
| 24 CVBS Input |  |  |  |  | √ | √ |
| Scrambling function | √ |  | √ |  | √ |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227