FMUSER FUQ-4654C Encoder Modulator 1 HDMI CVBS SDI YPbPr Input 1 ASI Input DVB-S-S2 RF Output Web-Based NMS Satellite Broadcasting Headend System

Product Overview

The FMUSER FUQ-4654C integrates encoding, multiplexing, and modulation to convert audio/video signals into DVB-S/S2 RF output. It features modular inputs for HDMI, CVBS, SDI, and YPbPr, with ASI input and outputs including ASI and IP. Controlled via web-based NMS, it is ideal for satellite broadcasting, headend systems, and flexible signal distribution.

Key feature

Web-based NMS management for remote control, firmware updates, VBR/CBR rate control, and low delay encoding

Principle Chart(Encoder Modulator)

Principle Chart(Modulator)

Technical Specification

| HDMI Encoding Input | Video | Input | HDMI*1(1 for backup) |
| --- | --- | --- | --- |
|  |  | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio | Encoding | MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0 ( Optional) |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |

| HDMI Encoding Input (OptionⅡ) | Video | Input | HDMI*1 (1 for backup) |
| --- | --- | --- | --- |
|  |  | Encoding | MPEG4 AVC/H.264 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Chroma | 4:2:0 |
|  | Audio | Encoding | MPEG1 Layer II |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |

| HDMI/YPbPr/CVBS 3-in-1 Encoding Input | Video (HDMI) | Encoding | MPEG2; MPEG4 AVC/H.264 |
| --- | --- | --- | --- |
|  |  | Input | HDMI*1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, -for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio (HDMI) | Encoding | MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0( Optional) |
|  |  | Input | HDMI*1 |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |
|  | Video (YpbPr/ CVBS) | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Input | YpbPr*1 / CVBS *1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | CVBS: 720x576_50i (PAL); 720x480_60i (NTSC) YpbPr: 1920*1080_60i, 1920*1080_50i; 1920*1080_60p, 1920*1080_50p; 1280*720_60p, 1280*720_50P |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio (YpbPr/ CVBS) | Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC and Dolby Digital AC3 2.0( Optional) |
|  |  | Interface | 1*Stereo/2*mono |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | 64/96/128/ 192/256/320kbps |

| 2*(S-Video/YPbPr/CVBS) 3-in-1 Encoding Input | Video | Encoding | MPEG-2 & MPEG-4 AVC/H.264 |
| --- | --- | --- | --- |
|  |  | Input | S-Video/YPbPr/CVBS*2 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2(For option 1) |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio | Encoding | MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0( Optional) |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |

| SDI Encoding Input | Video | Encoding | MPEG2; MPEG4 AVC/H.264 |
| --- | --- | --- | --- |
|  |  | Input | SDI*1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio | Encoding | MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0 ( Optional) |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |

| VGA/HDMI Encoding Input | Video (HDMI) | Encoding | MPEG2; MPEG4 AVC/H.264 |
| --- | --- | --- | --- |
|  |  | Input | HDMI*1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*576-50i, 720*480-60i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio (HDMI) | Encoding | MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0 ( Optional) |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |
|  | Video (VGA) | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Input | VGA(SVGA/XGA/UXGA/SXGA) |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1280*720_60p |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Rate Control | VBR/CBR |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio (VGA) | Encoding | MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0 ( Optional) |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | 64/96/128/ 192/256/320kbps |

| Modulator Section | DVB-S/S2 | Constellation | DVB-S: QPSK DVB-S2: QPSK, 8PSK |
| --- | --- | --- | --- |
|  |  | FEC | 1/2,2/3,3/4,5/6,7/8 (DVB-S QPSK) |
|  |  |  | 1/2,3/5,2/3,3/4,4/5,5/6,8/9,9/10 (DVB-S2 QPSK) |
|  |  |  | 3/5,2/3,3/4,5/6,8/9,9/10 (DVB-S2 8PSK) |
|  |  | RF output | 950.00-1050.00 MHz, 1Khz step |
|  |  | Symbol rate | 0.05-40.0Msps (DVB-S and DVB-S2 QPSK) |
|  |  |  | 0.05-26.0Msps(DVB-S2 8PSK) |
|  |  | Roll Off | 0.35(DVB-S) |
|  |  |  | 0.35, 0.25, 0.2(DVB-S2) |

| General | System | Local interface | LCD + control buttons |
| --- | --- | --- | --- |
|  |  | Remote management | Web NMS |
|  |  | Stream Out | 2 ASI out (BNC type); IP (MPTS/SPTS) out (100M) |
|  |  | NMS interface | RJ45, 100M |
|  |  | Language | English |
|  | Physical Specification | Power supply | AC 100V~240V |
|  |  | Dimensions | 482*300*44mm |
|  |  | Weight | 4.5 kg |
|  |  | Operation temperature | 0~45℃ |

Key Features

|  | FU-4654B | FU-4654C | FU-4513E |
| --- | --- | --- | --- |
| Encode In | 2 encoding slots with different encoding boards available | 1 encoding slot with different encoding boards available | N/A |
| ASI In | 1 ASI input for re-mux | 1 ASI input for pass-through | 4 ASI inputs with 3 for backup |
| IP In | N/A | N/A | 1 IP Input over UDP (100M) |
| RF Range | 950 – 1050 MHz | 950 -2150 MHz | 950 -2150 MHz |
| Symbol Rate | QPSK: 0.05-40.0Msps 8PSK: 0.05-26.0Msps | 0.05-20.0Msps | 32APSK: 0.5~40Msps 16APSK/8PSK/QPSK: 0.5~45 Msps |
| Constellation | QPSK, 8PSK | QPSK, 8PSK | QPSK, 8PSK, 16APSK, 32APSK |
| Output Level | -30 dBm ~-10.0 dBm | -10.0 dBm ~-41.5 dBm | -10.0 dBm~-41.5 dBm |
| BISS Scramble | N/A | Support | Support |
| IP output | 1 IP over UDP (100M) | N/A | N/A |
| 10MHz Ref Clock | N/A | Support | Support |
| 24V RF Out | N/A | Support | Support |
| RF In for Mix | Support | N/A | N/A |
| SFN TS Transmission | N/A | N/A | Support |

Application Case

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227