FMUSER FUQ-4654E Encoder Modulator Multi-input SDI-HDMI-YPbPr-CVBS-ASI DVB-S2X RF Output HEVC H.265 Encoding APSK Modulation Low-Latency for Professional Broadcast

Product Overview

The FUQ-4654E integrates DVB-S/S2/S2X encoding and modulation, featuring pluggable modules for SDI/HDMI/YPbPr/CVBS inputs and ASI input for re-muxing. It encodes SD/HD programs and modulates them to RF output with low-latency modes. Controlled via web-based NMS or front-panel, this encoder modulator suits TV stations and DSNG uplinks for efficient broadcast transmission.

Key feature

- Integrated pluggable modules for selecting HEVC/H.265, MPEG-4 AVC/H.264, or MPEG2 encoding.

- Supports multi-format I/O including SDI/HDMI/YPbPr/CVBS/ASI inputs and DVB-S/S2/S2X RF output (950–2150MHz).

- Features advanced modulation with configurable QPSK/8PSK/8APSK/16APSK/32APSK constellations and BISS encryption.

- Managed via Web NMS or front-panel with low-latency modes and 10MHz reference clock.

CID TEST SAMPLE ILLUSTRATION

Principle Chart

Technical Specifications

Single HDMI/SDI/YPbPr Input

*H.265/H.264 Video Encoding  *Up to 4 stereo audio inputs  *OSD Insertion

Video

| Encoding |  | HEVC/H.265; MPEG-4 AVC/H.264 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Input |  | HDMI/SDI/YPbPr |  |  |  |
| Resolution | Input |  |  | Output | Interface Applicable |
|  | 1920×1080_60P, 1920×1080_60i, 1920×1080_59.94P, 1920×1080_59.94i |  |  | . 1920×1080_60P | . HDMI SDI |
|  | 1920×1080_50P, 1920×1080_50i |  |  | 1920×1080_50P |  |
|  | 1280×720_60P, 1280×720_59.94P, |  |  | 1280×720_60P |  |
|  | 1280×720_50P |  |  | 1280×720_50P |  |
|  | 1920×1080_60i, 1920×1080_50i, 1280×720_60P, 1280×720_50P |  |  | 1920×1080_60i, 1920×1080_50i, 1280×720_60P, 1280×720_50P | YPbPr |
| Bit rate |  |  | 4Mbps~20Mbps (for MPEG-4 AVC/H.264) 0.5Mbps~20Mbps (HEVC/H.265) |  |  |
| Chroma |  |  | 4:2:0 |  |  |
| Rate Control |  |  | CBR/VBR |  |  |
| GOP Structure |  |  | IBBP, IPPP |  |  |
| Advanced Pretreatment |  |  | De-interlacing, Noise Reduction, Sharpening |  |  |

Audio

| encoding | MPEG-1 Layer 2, LC-AAC, HE-AAC, HE-AAC V2, AC3 Passthrough |
| --- | --- |
| Input | Digital: HDMI/SDI/AES; Analog: XLR |
| Channel | AES/SDI: Up to 4 stereos HDMI: 1 stereo XLR: Up to 2 stereos |
| Sample rate | 48KHz |
| Bit rate | 48Kbps~384Kbps (MPEG-1 Layer 2 & LC-AAC) 24 Kbps~128 Kbps (HE-AAC) 18 Kbps~56 Kbps (HE-AAC V2) |
| Audio Gain | 0-255 |

Single SDI Input

Video

| Encoding | MPEG2; MPEG4 AVC/H.264 |
| --- | --- |
| Input | SDI*1 |
| Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |

Audio

| encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC |
| --- | --- |
| Sample rate | 48KHz |
| Bit rate | 64kbps, 96kbps, 128kbps, 192kbps, 256kbps, 320kbps |

Single HDMI Input

Video

| Encoding | MPEG2; MPEG4 AVC/H.264 |
| --- | --- |
| Input | HDMI*1 |
| Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |

Audio

| encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC |
| --- | --- |
| Sample rate | 48KHz |
| Bit rate | 64kbps, 96kbps,128kbps, 192kbps, 256kbps, 320kbps |

HDMI/YPbPr/CVBS 3-in-1 Input

| Video (HDMI) |  |
| --- | --- |
| Encoding | MPEG2; MPEG4 AVC/H.264 |
| Interface | HDMI*1 |
| Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P |
| Audio (HDMI) |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC |
| Interface | HDMI*1 |
| Sample rate | 48KHz |
| Bit rate | 64/96/128/ 192/256/320kbps |
| Video (YPbPr/CVBS) |  |
| Encoding | MPEG2; MPEG4 AVC/H.264 |
| Interface | YPbPr*1/CVBS *1 |
| Resolution | CVBS: 720x576_50i (PAL); 720x480_60i (NTSC) YPbPr: 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |
| Audio (L/R) |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC |
| Interface | 1*Stereo/2*mono |
| Sample rate | 48KHz |
| Bit rate | 64/96/128/ 192/256/320kbps |

2*(S-Video + YPbPr + CVBS) Inputs

Video

| Encoding | Option 1: MPEG-2 MP@ML(4:2:0) Option 2: MPEG-2 & MPEG-4 AVC/H.264 (4:2:0) |
| --- | --- |
| Input | 2*S-Video/YPbPr/CVBS (RCA Type) |
| Resolution | 720*576 (PAL); 720*480 (NTSC) |
| Bit rate | Option 1: 1~19.50 Mbps; Option 2: 1~10.00 Mbps |

Audio

| encoding | Option 1: MPEG1 Layer II Option 2: MPEG1 Layer II; LC-AAC; HE-AAC and Dolby Digital AC3 2.0( Optional) |
| --- | --- |
| Input | 2 Stereo/Mono |
| Sample rate | 48KHz |
| Bit rate | 64kbps, 96kbps,128kbps, 192kbps, 256kbps, 320kbps |

Generals

| Input Interface | 2*CVBS; 1*SDI; 1*HDMI; 1*(HDMI/SDI/YPbPr 3-in-1); 1*(HDMI/YPbPr/CVBS 3-in-1) input optional ASI intput |  |  |  |
| --- | --- | --- | --- | --- |
| Output | DVB-S/S2/S2X RF output ; ASI output as mirror of RF |  |  |  |
| Modulation Section | Standard | DVB-S | DVB-S2 | DVB-S2X |
|  | Outer coding | RS Coding | BCH Coding | BCH Coding |
|  | Inner coding | Convolution | LDPC Coding | LDPC Coding |
|  | Constellation | QPSK | QPSK,8PSK, 16APSK,32APSK | QPSK,8PSK, 8APSK, 16APSK, 32APSK |
|  | FEC/ Convolution Rate | 1/2, 2/3, 3/4, 5/6, 7/8 | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 , 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 , 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  | Roll-off Factor | 0.2, 0.25, 0.35 | 0.2, 0.25, 0.35 | 0.05, 0.10, 0.15, 0.2, 0.25, 0.35 |
|  | Symbol Rate | 0.5~45Msps | 0.5~40Msps (32APSK); 0.5~45Msps (QPSK/8PSK/16APSK) | 0.5~40Msps(32APSK); 0.5~45 Msps (QPSK/8PSK /8APSK/16APSK) |
|  | RF Range | 950-2150MHz |  |  |
|  | Output level | +3dBm~-28.5dBm |  |  |
| BISS | Mode 1, Mode E |  |  |  |
| System | Local Control | LCD + control buttons |  |  |
|  | Remote Control | Web NMS |  |  |
|  | Low Latency Mode | Normal, Mode 1, Mode 2 |  |  |
|  | Language | English |  |  |
| Physical Specification | Demission | 410×460×44mm (W*L*H) |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power Supply | AC 100-240V 50/60Hz 2A |  |  |

Order Guide

|  | FU-4654A | FU-4654C | FU-4654D |
| --- | --- | --- | --- |
| DVB-S/S2 | ● | ● | ● |
| DVB-S2X |  |  | ● |
| RF Output(950-2150MHz) |  | ● | ● |
| IF Output(50-200 MHz) | ● |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227