FMUSER FUQ-4655AB Encoder Modulator 12 HD-SDI Inputs 16 DVB-C Outputs DVB-C DVB-T ATSC ISDB-T Standards HD-SDI IP-RF Interfaces Encoding Multiplexing Scrambling Broadcast Headend System

Product Overview

The FUQ-4655XY is a high-integration encoder modulator combining encoding, multiplexing, scrambling, and modulation. It features 12 HD-SDI inputs, a DVB-C tuner, and up to 512 IP inputs via Data1 port, with DVB-C RF output supporting 16 carriers. Optional outputs include DVB-T, ATSC, or ISDB-T RF with fewer carriers and ASI mirroring. This device is ideal for broadcast studios, cable headends, and professional video distribution systems.

Key feature

Includes optional ASI output and web-based management for remote control.

Principle Chart

DVB-C Principle Chart

DVB-T/ATSC Principle Chart

ISDB-T Principle Chart

Technical specification

| Input | 12 HD-SDI inputs with CC 1 DVB-C (ATSC optional)Tuner for re-mux, F type interface 512 IP input over UDP and RTP, DATA1, RJ45—DVB-C RF out 256 IP input over UDP and RTP, DATA1, RJ45—DVB-T/ATSC RF out 192 IP input over UDP and RTP, DATA1, RJ45—ISDB RF out |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video | Resolution | Input |  | 1920*1080_60p, 1920*1080_50p,1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P, 720*480_60i, 720*576_50i |  |  |
|  |  | Output |  | 1920*1080_60p, 1920*1080_50p,1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i, 1280*720_60p, 1280*720_50P, 720*576_50p, 720*576_50i, 720*576_30p, 720*576_25p, 720*480_60p, 720*480_60i,720*480_30p, 720*480_25p,320*240_60p,320*240_50p, 320*240_30p, 320*240_25p,320*180_60p, 320*180_50p, 320*180_30p, 320*180_25p, 960*540_50i, 704*576_50i,704*480_60i, 640*576_50i 640*480_60i, 544*576_50i,544*480_60i, |  |  |
|  | Encoding | MPEG2 & MPEG4 AVC/H.264 |  |  |  |  |
|  | Bit-rate | 0.8~19Mbps for H.264 encoding 1~19Mbps for MPEG-2 encoding |  |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |  |
|  | GOP Structure | GOP B Frame: 0-3, GOP P Frame: 0-6 |  |  |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |  |  |
| Audio | Encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC, Dolby Digital AC3 (2.0) encoding (Optional), AC3 (2.0/5.1) passthrough |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Bit-rate | 64Kbps-320kbps each channel |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 256 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Scrambling | Maximum simulcrypt CA | 4 |  |  |  |  |
|  | Standard | ETR289, ETSI 101197, ETSI 103197 |  |  |  |  |
|  | Connection | Local/remote connection |  |  |  |  |
| Modulation | DVB-C | QAM Channel: 16 non-adjacent carriers output (maximum bandwidth 192MHz) Standard: EN300 429/ITU-T J.83A/B MER: ≥40db RF frequency: 50~960MHz, 1KHz step RF output level: -20~+10dbm, 0.1db step Symbol Rate: 5.0Msps~7.0Msps, 1ksps stepping |  |  |  |  |
|  |  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  |  | 8M | 6M |
|  | DVB-T | Standard |  |  | EN300744 |  |
|  |  | FFT mode |  |  | 2K, 4K, 8K |  |
|  |  | Bandwidth |  |  | 6M, 7M, 8M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥42 dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~+10 dBm, 0.1db step |  |
|  | ATSC | Standard |  |  | ATSC A/53 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | 8VSB |  |
|  |  | FEC |  |  | RS(208 188)+Trellis |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | ACL |  |  | -55 dBc |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~+10dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | Transmission Mode |  |  | 2K, 4K, 8K |  |
|  |  | Code rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 6 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20dBm~+10dBm, 0.1dB stepping |  |
| Stream output | 1 ASI output as mirror of one of RF output carriers-Optional 16 MPTS output over UDP and RTP/RTSP as mirror of 16 DVB-C carriers, 8 MPTS output over UDP and RTP/RTSP as mirror of 8 DVB-T/ATSC carriers(Optional) 6 MPTS output over UDP and RTP/RTSP as mirror of 6 ISDB-T carriers(Optional) 1*1000M Base-T Ethernet interface |  |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×440mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |  |

Ordering Information

|  | IP input | IP output | RF carriers | Scrambling |
| --- | --- | --- | --- | --- |
| DVB-C | 512 | 16 | 16 | √ |
| DVB-T | 256 | 8 | 8 | × |
| ATSC | 256 | 8 | 8 | × |
| ISDB-T | 192 | 6 | 6 | × |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227