FMUSER FUQ-4655BC Encoder Modulator 24 CVBS & 512 IP Inputs 16 RF Outputs MPEG-2 Encoding DVB-C-DVB-T-ATSC-ISDB-T Multiplexing Scrambling CATV Headend Hotel TV System

Product Overview

The FMUSER FUQ-4655BC is a professional SD encoder modulator integrating encoding, multiplexing, scrambling, and modulation, with 24 CVBS inputs and IP streaming via the DATA1 port. It processes signals using MPEG-2 video encoding and MPEG-1 Layer 2 or DD AC3 audio encoding, outputting DVB-C, DVB-T/ATSC, or ISDB-T signals. Featuring web-based management for control and updates, it is ideal for small CATV headends and broadcast systems, enabling efficient signal distribution.

Key feature

Comprehensive multiplexing with PID remapping, PCR adjusting, and PSI/SI editing.

Principle Chart

DVB-C Principle Chart

DVB-T Principle Chart

ISDB-T Principle Chart

Technical specification

| Input | 24 CVBS inputs 1 DVB-C(ATSC optional) Tuner for re-mux, F type interface 512 IP input over UDP and RTP, DATA1, RJ45—DVB-C RF out 256 IP input over UDP and RTP, DATA1, RJ45—DVB-T/ATSC RF out 192 IP input over UDP and RTP, DATA1, RJ45—ISDB RF out |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Video | Resolution | 720×480_60i, 544×480_60i, 352×480_60i 352×240_60i,320×240_60i,176×240_60i, 176×120_60i |  |  |  |
|  |  | 720×576_50i,704×576_50i,640×576_50i, 352×288_50i 320×288_50i, 176×288_50i, 176×144_50i |  |  |  |
|  | Encoding | MPEG-2 |  |  |  |
|  | Bit-rate | 0.5Mbps~8Mbps per channel |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |
|  | GOP Structure | IP, IBP, IBBP, IBBBP |  |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |  |
| Audio | Encoding | MPEG-1 Layer 2, DD AC3 (2.0) |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |
|  | Resolution | 24-bit |  |  |  |
|  | Bit-rate | 64Kbps,128Kbps,192kbps,256kbps,320kbps,384kbps each channel |  |  |  |
| Multiplexing | Maximum PID Remapping | 180 input per channel |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |
| Scrambling | Maximum simulcrypt CA | 4 |  |  |  |
|  | Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |
|  | Connection | Local/remote connection |  |  |  |
| Modulation | DVB-C | QAM Channel |  | 16 non-adjacent carriers output (maximum bandwidth |  |
|  |  | Standard |  | EN300 429/ITU-T J.83A/B |  |
|  |  | MER |  | ≥40db |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  | -20~+10dbm (87~107 dbµV), 0.1db step |  |
|  |  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps stepping |  |
|  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  | 8M | 6M |
|  | DVB-T | Standard |  | EN300744 |  |
|  |  | FFT mode |  | 2K,4K, 8K |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  | ≥42 dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 8 non-adjacent carriers DVB-T out (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20~ +10 dBm (77~97 dbµV), 0.1db step |  |
|  | ATSC | Standard |  | ATSC A/53 |  |
|  |  | Bandwidth |  | 6M |  |
|  |  | Constellation |  | 8VSB |  |
|  |  | FEC |  | RS(208 188)+Trellis |  |
|  |  | MER |  | ≥40dB |  |
|  |  | ACL |  | -55 dBc |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20~+10dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  | 6M |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | Transmission Mode |  | 2K, 4K, 8K |  |
|  |  | Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  | ≥40dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 6 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20dBm~+10dBm, 0.1dB stepping |  |
| Stream output | 1 ASI output as mirror of one of RF output carriers-Optional 16 MPTS output over UDP and RTP/RTSP as mirror of 16 DVB-C carriers, 8 MPTS output over UDP and RTP/RTSP as mirror of 8 DVB-T/ATSC carriers(Optional) 6 MPTS output over UDP and RTP/RTSP as mirror of 6 ISDB-T carriers(Optional) 1*1000M Base-T Ethernet interface |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |
|  | Chinese and English language |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×440mm×44mm |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |

Ordering Information

|  | IP input | IP output | RF carriers | Scrambling |
| --- | --- | --- | --- | --- |
| DVB-C | 512 | 16 | 16 | √ |
| DVB-T | 256 | 8 | 8 | × |
| ATSC | 256 | 8 | 8 | × |
| ISDB-T | 192 | 6 | 6 | × |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227