FMUSER FUQ-4655K Encoder Modulator 12 HDMI Inputs 256 IP Inputs MPEG2 H.264 Encoding DVB-C-DVB-T-ATSC-ISDB-T Modulation Web Management CATV Headend Application Product Overview

The FMUSER FUQ-4655L Encoder Modulator integrates encoding, multiplexing, scrambling, and modulating functions. It processes signals from 12 HDMI inputs and IP inputs, encoding to MPEG2 or H.264, and outputs via RF for DVB-C, DVB-T/ATSC, or ISDB-T standards. Managed through web-based interface for control and updates. Ideal for small CATV headend systems, broadcasting, and IP streaming applications.

Key feature

Supports 12 HDMI inputs with MPEG2 & H.264 encoding, Features DVB-C, DVB-T/ATSC, and ISDB-T modulation for RF output, Integrated multiplexing and scrambling with PID remapping, Provides IP I/O over UDP/RTP/RTSP protocols, Web-based management for easy control

Principle Chart

DVB-C Principle Chart

DVB-T/ATSC Principle Chart

ISDB-T Principle Chart

Technical specification

| Input | 12 HDMI inputs 12 CC(closed caption) input, BNC interface 1 DVB-C(ATSC optional) Tuner for re-mux, F type interface 512 IP input over UDP and RTP, DATA1, RJ45—DVB-C RF out 256 IP input over UDP and RTP, DATA1, RJ45—DVB-T/ATSC RF out 192 IP input over UDP and RTP, DATA1, RJ45—ISDB RF out |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video | Resolution | Input |  | 1920*1080_60p, 1920*1080_50p,1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P, 720*480_60i, 720*576_50i |  |  |
|  |  | Output |  | 1920*1080_60p, 1920*1080_50p,1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i, 1280*720_60p, 1280*720_50P, 720*576_50p, 720*576_50i, 720*576_30p, 720*576_25p, 720*480_60p, 720*480_60i,720*480_30p, 720*480_25p,320*240_60p,320*240_50p, 320*240_30p, 320*240_25p,320*180_60p, 320*180_50p, 320*180_30p, 320*180_25p, 960*540_50i, 704*576_50i,704*480_60i, 640*576_50i 640*480_60i, 544*576_50i,544*480_60i, |  |  |
|  | Encoding | MPEG2 & MPEG4 AVC/H.264 |  |  |  |  |
|  | Bit-rate | 0.8~19Mbps for H.264 encoding 1~19Mbps for MPEG-2 encoding |  |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |  |
|  | GOP Structure | GOP B Frame: 0-3, GOP P Frame: 0-6 |  |  |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |  |  |
| Audio | Encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC, Dolby Digital AC3 (2.0) encoding (Optional), AC3 (2.0/5.1) passthrough |  |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |  |
|  | Bit-rate | 64Kbps-320kbps each channel |  |  |  |  |
| Multiplexing | Maximum PID Remapping | 180 input per channel |  |  |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |  |
| Scrambling | Maximum simulcrypt CA | 4 |  |  |  |  |
|  | Standard | ETR289, ETSI 101197, ETSI 103197 |  |  |  |  |
|  | Connection | Local/remote connection |  |  |  |  |
| Modulation | DVB-C | QAM Channel: 16 non-adjacent carriers output (maximum bandwidth 192MHz) Standard: EN300 429/ITU-T J.83A/B MER: ≥40db RF frequency: 50~960MHz, 1KHz step RF output level: -20~+10dbm, 0.1db step Symbol Rate: 5.0Msps~7.0Msps, 1ksps stepping |  |  |  |  |
|  |  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  |  | 8M | 6M |
|  | DVB-T | Standard |  |  | EN300744 |  |
|  |  | FFT mode |  |  | 2K,4K, 8K |  |
|  |  | Bandwidth |  |  | 6M, 7M, 8M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥42 dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~ +10 dBm, 0.1db step |  |
|  | ATSC | Standard |  |  | ATSC A/53 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | 8VSB |  |
|  |  | FEC |  |  | RS(208 188)+Trellis |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | ACL |  |  | -55 dBc |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20~+10dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  |  | 6M |  |
|  |  | Constellation |  |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | Transmission Mode |  |  | 2K, 4K, 8K |  |
|  |  | Code rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  |  | ≥40dB |  |
|  |  | RF frequency |  |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  |  | 6 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  |  | -20dBm~+10dBm, 0.1dB stepping |  |
| Stream output | 1 ASI output as mirror of one of RF output carriers-Optional 16 MPTS output over UDP and RTP/RTSP as mirror of 16 DVB-C carriers, 8 MPTS output over UDP and RTP/RTSP as mirror of 8 DVB-T/ATSC carriers(Optional) 6 MPTS output over UDP and RTP/RTSP as mirror of 6 ISDB-T carriers(Optional) 1*1000M Base-T Ethernet interface |  |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×440mm×44mm |  |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |  |

Ordering Information

|  | IP input | IP output | RF carriers | Scrambling |
| --- | --- | --- | --- | --- |
| DVB-C | 512 | 16 | 16 | √ |
| DVB-T | 256 | 8 | 8 | × |
| ATSC | 256 | 8 | 8 | × |
| ISDB-T | 192 | 6 | 6 | × |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227