FMUSER FUQ-4655L Multi-Channel Encoder 12 HDMI & 32 IP Inputs MPEG-2 MPEG-4 AVC-H.264 Encoding PID Remapping for CATV Satellite Terrestrial TV       Product Overview

The FMUSER FUQ-4655R is a multi-channel encoder with 12 HDMI inputs, 12 CC inputs, and 32 IP inputs via UDP/RTP. It encodes video using MPEG-2 and MPEG-4 AVC/H.264 standards, and supports audio formats including MPEG1 Layer II and Dolby Digital AC3. Ideal for CATV, satellite, and terrestrial TV broadcasting systems.

Key feature

Supports 12 HDMI inputs with MPEG-2 and MPEG-4 AVC/H.264 encoding for HD video up to 1080p-60.

Includes 12 dedicated CC input ports for closed captioning support.

Offers flexible audio encoding with MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC, and optional Dolby Digital AC3.

Enables IP streaming with 32 inputs/outputs via UDP/RTP, supporting 1 MPTS and 12 SPTS streams.

Principle Chart

Technical specification

| Input | 12 HDMI inputs 12 CC (closed caption) input, BNC interface 32 IP input over UDP and RTP through DATA 1 port |  |  |  |
| --- | --- | --- | --- | --- |
| Video | Resolution | Input |  | 1920*1080_60p, 1920*1080_50p,1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P, 720*480_60i, 720*576_50i |
|  |  | Output |  | 1920*1080_60p,1920*1080_50p,1920*1080_60i, 1920*1080_50i,1440*1080_60i, 1440*1080_50i, 1280*720_60p,1280*720_50P, 720*576_50p, 720*576_50i, 720*576_30p, 720*576_25p, 720*480_60p, 720*480_60i,720*480_30p, 720*480_25p,320*240_60p,320*240_50p, 320*240_30p, 320*240_25p,320*180_60p, 320*180_50p, 320*180_30p, 320*180_25p, 960*540_50i, 704*576_50i,704*480_60i, 640*576_50i 640*480_60i, 544*576_50i,544*480_60i, |
|  | Encoding | MPEG2 & MPEG4 AVC/H.264 |  |  |
|  | Bit-rate | 0.8~19Mbps for H.264 encoding 1~19Mbps for MPEG-2 encoding |  |  |
|  | Rate Control | CBR/VBR |  |  |
|  | GOP Structure | GOP B Frame: 0-3, GOP P Frame: 0-6 |  |  |
|  | Advanced Pretreatment | De-interlacing, noise reduction |  |  |
| Audio | Encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC, Dolby Digital AC3 (2.0) encoding (Optional), AC3 (2.0/5.1) passthrough |  |  |
|  | Sampling rate | 48KHz |  |  |
|  | Resolution | 24-bit |  |  |
|  | Bit-rate | 64Kbps-320kbps each channel |  |  |
| Multiplexing | Maximum PID Remapping | 180 input per channel |  |  |
|  | Function | PID remapping ( automatically or manually) |  |  |
|  |  | Accurate PCR adjusting |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |
| Stream output | 1 MPTS and 12 SPTS IP output over UDP and RTP/RTSP, 2*1000M Base-T Ethernet interface, Data 1 and Data 2 1 ASI output as mirror of MPTS-Optional |  |  |  |
| System function | Network management (WEB) |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×440mm×44mm |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |

Technical Comparison

|  | FU-4655K | FU-4655KF | FU-4653L |
| --- | --- | --- | --- |
| Chipset | Vixs | Fujitsu | Hisilicon |
| Video Channel | 2/4/6/8/10/12×HDMI optional | 4/8/12/16/20/24×HDMI optional | 4/8/12/16/20/24×HDMI optional |
| Video Encode | MPEG2 & MPEG4 AVC/H.264 | MPEG-4 AVC/H.264 | MPEG-4 AVC/H.264 |
| CC Input | 12 CC input ports | N/A | N/A |
| IP Input | 32 ×IP | 32× IP | N/A |
| Video Bit Rate | 0.8~19Mbps for H.264 encoding 1~19Mbps for MPEG-2 encoding | 0.8~19Mbps each channel | 1Mbps~13Mbps each channel (Bit rate is with a high fluctuation, suitable for cable system, not suitable for satellite uplink) |
| GOP | GOP B Frame: 0-3, GOP P Frame: 0-6 | IBBP | GOP Size=25 (without B Frame) |
| Resolution | Up to 1080p-60 (Resolution and frame rate downscale convertion) | Up to 1080p-60 (1080i/720p to 576p/480p downscale conversion) | Up to 1080P-30 (1080i/p-60/50 to 1080p-30/25 auto downscale convertion) |
| Audio Encode | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC, Dolby Digital AC3 (2.0) encoding (Optional), AC3 pass through | MPEG1 Layer II, (MPEG-2 AAC, MPEG-4 AAC Optional) AC3 pass through | MPEG-1 Layer II |
| Audio Gain | Yes | Yes | Yes |
| OSD Function | N/A | N/A | Yes |
| ASI output | Option as required | Option as required | Option as required |
| IP output | SPTS and 1MPTS | SPTS and 1MPTS | SPTS and 1MPTS(for 4/8/12 channel) SPTS or 1MPTS(for 16/20/24 channel) |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227