FMUSER FUQ-46610F IP Gateway 16 Tuner Inputs 2 ASI Inputs DVB-S-S2-S2X DVB-C-T ATSC ISDB-T to 512 SPTS 16 MPTS IP Outputs BISS Descrambling Web Management Broadcast Headend

Product overview

The FUQ-46610F is a professional IP gateway converting 16 tuner inputs (supporting DVB-S/S2/S2X, DVB-C/T, ATSC, or ISDB-T) or 2 ASI feeds into IP streams. It processes signals into 512 SPTS (PID-filtered) or 16 MPTS (Null PKT filtered) outputs via dual Gigabit Ethernet ports with BISS descrambling. The 1RU rack-mount unit features web-based control and DiSEqC switching. Ideal for broadcast headend digitization, IPTV distribution, and multi-standard signal aggregation in terrestrial/satellite systems.

Key feature

Web-based management for remote signal monitoring and control

Working Principle

Specifications

| Input |  | Optional 1:16 tuners input +2 ASI input---SPTS output Optional 2:14 tuners input +2 ASI input --- MPTS output Optional 3:16 tuners input --- MPTS output |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New Type) | DVB-C | Frequency In |  | 60~890MHz |
|  |  |  | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |
|  |  | DVB-T/(T) | Frequency In |  | 60~890MHz |
|  |  |  | Bandwidth |  | 6/7/8 M bandwidth |
|  |  | ISDB-T | Frequency In |  | 60~890MHz |
|  | Version 1 | DVB-S | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | 1~45 Msps |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  | DVB-S2 | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | 1~45 Msps |
|  |  |  | FEC |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK |
|  | Version 2 (New Type) | DVB-S | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | 0.5~45Msps |
|  |  |  | Signal Strength |  | - 65- -25dBm |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~34Msps; |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | DVB-S2X | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~34Msps |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤125 Mbps |
|  |  | ATSC | Frequency In |  | 54~858MHz |
|  |  |  | Bandwidth |  | 6M bandwidth |
|  |  |  | Constellation |  | 8VSB |
| BISS Descrambling |  | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |
| Output |  | 512 SPTS IP mirrored output over UDP and RTP/RTSP protocol through GE1 and GE2 port (IP address and port number of GE1 and GE2 are different) , Unicast and Multicast |  |  |  |
|  |  | 16 MPTS IP output (for Tuner/ASI passthrough) over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |
| System |  | Web based management |  |  |  |
|  |  | Ethernet software upgrade |  |  |  |
| Miscellaneous |  | Dimension |  | 482mm×410mm×44mm (W×L×H) |  |
|  |  | Approx weight |  | 3.6kg |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |
|  |  | Power consumption |  | 20W |  |

Tuner Input Options:

- 8 DVB-C Tuners + 8 ATSC Tuners

- 8 DVB-C Tuners + 8 DVB-T/(T) Tuners

- 8 DVB-S2/S2X Tuners + 8 DVB-T/(T) Tuners

- 8 DVB-S2/S2X Tuners + 8 ISDB-T Tuners

- 16 DVB-S/S2/S2X/DVB-C/T/(T) /ISDB-T/ATSC Tuners

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227