FMUSER FUQ-4665 HD-SD Encoder Modulator 4 Input Channels HDMI-YPbPr-CVBS to DVB-T-C-ATSC RF MPEG-2 MPEG-4 AVC H.264 Encoding USB Record Playback Broadcast Headend

Product Overview

The FUQ-4665 performs MPEG-2/MPEG-4 AVC encoding and DVB-T/C/ATSC modulation with HDMI/YPbPr/CVBS inputs. It processes HD/SD signals into digital RF output (30-960MHz) with USB record/playback capability. This 1RU rack-mount unit supports NMS control via Ethernet. Ideal for digital terrestrial broadcasting and multi-screen headend systems requiring low-latency conversion.

Key feature

Dolby Digital AC3 2.0 audio pass-through via HDMI input

Principle Chart

Technical Specifications

| Encoding Section |  |  |  |
| --- | --- | --- | --- |
| HDMI | Video | Encoding | MPEG-2, MPEG-4 AVC/H.264 |
|  |  | Interface | HDMI*1 |
|  |  | Resolution | 1920*1080_60P,1920*1080_50P à (for MPEG-4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |
|  |  | Bit rate | 0.500~19.500 Mbps |
|  | Audio | Encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC; DD AC3 2.0 (Optional) AC3 Passthrough |
|  |  | Interface | HDMI |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | 64, 96,128, 192, 256, 320, 384kbps |
| YPbPr/ CVBS | Video | Encoding | MPEG-2, MPEG-4 AVC/H.264 |
|  |  | Interface | CVBS *1, YPbPr*1 |
|  |  | Resolution | CVBS: 720x576_50i (PAL); 720x480_60i (NTSC) YPbPr: 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |
|  |  | Bit rate | 0.500~19.500 Mbps |
|  | Audio | Encoding | MPEG1 Layer II, MPEG2-AAC, MPEG4-AAC; DD AC3 2.0 (Optional) |
|  |  | Interface | 1 Stereo /2 mono |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | 64, 96,128, 192, 256, 320, 384kbps |

| DVB-T Modulator Section |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Standard |  |  | DVB-T COFDM |  |  |  |
| Bandwidth |  |  | 6M, 7M, 8M |  |  |  |
| Constellation |  |  | QPSK, 16QAM, 64QAM, |  |  |  |
| Code rate |  |  | 1/2, 2/3, 3/4, 5/6, 7/8. |  |  |  |
| Guard Interval |  |  | 1/32, 1/16, 1/8, 1/4 |  |  |  |
| Transmission Mode: |  |  | 2K, 8K |  |  |  |
| MER |  |  | ≥42dB |  |  |  |
| RF frequency |  |  | 30~960MHz, 1KHz step |  |  |  |
| RF output level |  |  | -16~ -36 dBm (71~91 dbµV), 0.1db step |  |  |  |
| DVB-C Modulator Section (Option) |  |  |  |  |  |  |
| Standard | J.83A (DVB-C) |  |  |  | J.83B | J.83C |
| Constellation | 16/32/64/128 /256 QAM |  |  |  | 64/ 256 QAM | 64/ 256 QAM |
| Bandwidth | 8M |  |  |  | 6M | 6M |
| MER | ≥43dB |  |  |  |  |  |
| RF frequency | 30~960MHz, 1KHz step |  |  |  |  |  |
| RF level | -16~ -36dbm(71~91 dbµV), 0.1db step |  |  |  |  |  |
| Symbol rate | 5000-9000 Ksps |  |  |  |  |  |
| ATSC Modulator Section (Option) |  |  |  |  |  |  |
| Standard |  | ATSC A/53 |  |  |  |  |
| MER |  | ≥42dB |  |  |  |  |
| RF frequency |  | 30~960MHz, 1KHz step |  |  |  |  |
| RF output level |  | -16~ -36 dBm (71~91 dbµV), 0.1db step |  |  |  |  |
| Constellation |  | 8VSB |  |  |  |  |
| System |  |  |  |  |  |  |
| Management |  |  |  | Local control: LCD + control buttons |  |  |
| Language |  |  |  | English |  |  |
| LCN Insertion |  |  |  | yes |  |  |
| General |  |  |  |  |  |  |
| Power supply |  |  |  | DC 12V |  |  |
| Dimensions |  |  |  | 144*238*52mm |  |  |
| Weight |  |  |  | Approx 1kg |  |  |
| Operation temperature |  |  |  | 0~45℃ |  |  |

Technical Compare

| Main Features | FMUSER FU-4665/FU-4667 | Competing Products |
| --- | --- | --- |
| Encode | l MPEG2 HD/SD; MPEG4 HD/SD | l MPEG2 SD |
| Record TS to USB memory | l Yes (FU-4665&FU-4667) | l No |
| *.ts files play-mode | l Both Multi/Single files loop l Support HD&SD video play | l Single file loop l Support only SD video play |
| TS Creator.exe | l HD video to HD video transform l Null Packet Filter, effect bitrate up to 90% l Support output video size, rate, and format control | l HD video to SD video downscale l No Null Packet Filter, effect bit rate only around 50% |
| Single video length | l 2-3 Hours HD/SD | l 40 Minutes SD |
| Low Delay | l Supported | l Not supported |
| DD AC3 (2.0) audio encode | l Supported | l Not supported |

Internal Test Report of Latency

The values cover the progress from Encoding to Decoding

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227