FMUSER FUQ-4667 Encoder Modulator 1 In 1 Out HDMI-DVB-C DVB-T ISDB-T ATSC Multi-Standard H.265-H.264 4K Encoding Modulation Streaming for Broadcast Hospitality

New Comer!

Product overview

The FMUSER FUQ-4667 encoder modulator converts HDMI inputs into DVB-C, DVB-T, ISDB-T, or ATSC RF outputs or IP streams using H.265 or H.264 encoding. It processes signals for 4K resolution with modulation to broadcast standards. This unit features an LCD screen for control, USB port for TS recording, and OSD insertion. Ideal for TV distribution, digital signage, and IP video applications.

Key feature

Includes Management Tools such as OSD insertion, TS recording via USB, and LCD screen control.

System Connection

Appearance and Description

Technical Specifications

| HDMI Encoding Section |  |  |
| --- | --- | --- |
| Video | Encoding | HEVC/ H.265 , MPEG 4 AVC/H.264 |
|  | Interface | HDMI*1. HDMI loop out*1 |
|  | Resolution | 3840*2160_60/50P(H.265 only), 1920*1080_60/59.94/50P, 1280*720_60/59.94/50P, |
|  | Chroma | Input:4:4:4/4:2:2; output:4:2:0 |
|  | Bitrate | 2Mbps~20Mbps |
|  | Rate Control | CBR/VBR |
|  | GOP Structure | IBBP, IPPP |
| Audio | Encoding Format | MPEG-1 Layer 2, LC-AAC, HE-AAC, HE-AAC V2; AC3 Pass-through |
|  | Sample rate | 32KHz,44.1KHz,48KHz |
|  | Bit-rate | 48Kbps~384Kbps (MPEG-1 Layer 2 & LC-AAC) 24 Kbps~128 Kbps (HE-AAC) 18 Kbps~56 Kbps (HE-AAC V2) |

| IP output |  |  |  |  |
| --- | --- | --- | --- | --- |
| IP out over UDP (Unicast/multicast), RTP/RTSP, (RJ45, 100M/1000M self-adaptive) |  |  |  |  |
| DVB-C Modulator Section |  |  |  |  |
| Standard |  |  | J.83A (DVB-C), J.83B |  |
| MER |  |  | ≥40dB |  |
| RF frequency |  |  | 100-1000MHz, 1KHz step |  |
| RF output level |  |  | -20~ -5dBm, 1dB step |  |
| Symbol rate |  |  | 5.000~7.000Msps adjustable |  |
|  | J.83A |  |  | J.83B |
| Constellation | 16/32/64/128/ 256 QAM |  |  | 64/ 256 QAM |
| Bandwidth | 8M |  |  | 6M |
| DVB-T Modulator Section |  |  |  |  |
| Standard |  | DVB-T COFDM |  |  |
| Bandwidth |  | 6M, 7M, 8M |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |
| Transmission Mode: |  | 2K, 8K |  |  |
| MER |  | ≥35dB |  |  |
| RF frequency |  | 100-900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| ISDB-T Modulator Section |  |  |  |  |
| Standard |  | ARIB STD-B31 |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |
| Transmission Mode |  | 2K, 4K, 8K |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
| RF frequency |  | 100~900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| ATSC Modulator Section |  |  |  |  |
| Standard |  | ATSC A/53 |  |  |
| Constellation |  | 8 VSB |  |  |
| RF output level |  | -63~-16dBm (44~91 dBµV), 1dB step |  |  |
| MER |  | ≥35dB |  |  |
| RF frequency |  | 100~900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| System |  |  |  |  |
| Management |  | Web-GUI, LED+Keyboard |  |  |
| Language |  | English |  |  |
| Upgrade |  | Web update |  |  |
| General |  |  |  |  |
| Power supply |  | DC 12V |  |  |
| Dimensions |  | 203*144*52mm |  |  |
| Weight |  | < 1kg |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227