FMUSER FUQ-4667XX 1-Ch HDMI Input Encoder Modulator 1080p60 H.265-H.264 Encoding DVB-C-T ATSC ISDB-T Multi-Standard RF-IP Output for TV Distribution and Digital Signage

New Comer!

Product overview

The FUQ-4667XX is a professional encoder modulator featuring 1x HDMI input and dual RF/IP outputs. It processes 1080p60 signals via H.265/HEVC or H.264 encoding, then modulates to DVB-C/T, ATSC, or ISDB-T standards. This 1RU rack-mount device supports web-GUI and front-panel control. Ideal for cable TV headends, hospitality TV systems, and digital signage networks requiring multi-format distribution.

Key feature

Remote management via web-GUI with AC3 audio pass-through support

System Connection

Appearance and Description

Technical Specs

| HDMI Encoding Section |  |  |  |  |
| --- | --- | --- | --- | --- |
| Video | Encoding | HEVC/ H.265 , MPEG 4 AVC/H.264 |  |  |
|  | Interface | HDMI*1. HDMI loop out*1 |  |  |
|  | Resolution |  |  | 1920*1080_60P, 1920*1080_50P; 1920*1080_59.94P, 1280*720_60p, 1280*720_59.94 1280*720_50P |
|  | Chroma |  | 4:2:0 |  |
|  | Bitrate |  | 1Mbps~15Mbps |  |
|  | Rate Control |  | CBR/VBR |  |
|  | GOP Structure |  | IP |  |
| Audio | Encoding Format |  | MPEG-1 Layer 2, LC-AAC, HE-AAC, HE-AAC V2; AC3 Pass-through |  |
|  | Sample rate |  | 32KHz,44.1KHz,48KHz |  |
|  | Bit-rate |  | 48~384Kbps |  |

| IP output |  |  |  |  |
| --- | --- | --- | --- | --- |
| IP out over UDP (Unicast/multicast), RTP/RTSP, (RJ45, 100M) |  |  |  |  |
| DVB-C Modulator Section |  |  |  |  |
| Standard |  |  | J.83A (DVB-C), J.83B |  |
| MER |  |  | ≥40dB |  |
| RF frequency |  |  | 100-1000MHz, 1KHz step |  |
| RF output level |  |  | -20~ -5dBm, 1dB step |  |
| Symbol rate |  |  | 5.000~7.000Msps adjustable |  |
|  | J.83A |  |  | J.83B |
| Constellation | 16/32/64/128/ 256 QAM |  |  | 64/ 256 QAM |
| Bandwidth | 8M |  |  | 6M |
| DVB-T Modulator Section |  |  |  |  |
| Standard |  | DVB-T COFDM |  |  |
| Bandwidth |  | 6M, 7M, 8M |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |
| Transmission Mode: |  | 2K, 8K |  |  |
| MER |  | ≥35dB |  |  |
| RF frequency |  | 100-900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| ISDB-T Modulator Section (Optional) |  |  |  |  |
| Standard |  | ARIB STD-B31 |  |  |
| Constellation |  | QPSK, 16QAM, 64QAM |  |  |
| Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |  |
| Transmission Mode |  | 2K, 4K, 8K |  |  |
| Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
| RF frequency |  | 100~900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| ATSC Modulator Section (Optional) |  |  |  |  |
| Standard |  | ATSC A/53 |  |  |
| Constellation |  | 8 VSB |  |  |
| RF output level |  | -63~-16dBm (44~91 dBµV), 1dB step |  |  |
| MER |  | ≥35dB |  |  |
| RF frequency |  | 100~900MHz, 1KHz step |  |  |
| RF output level |  | -63~ -16dBm, 1dB step |  |  |
| System |  |  |  |  |
| Management |  | Web-GUI, LED+Keyboard |  |  |
| Language |  | English |  |  |
| Upgrade |  | Web update |  |  |
| General |  |  |  |  |
| Power supply |  | DC 12V |  |  |
| Dimensions |  | 160*120*52mm |  |  |
| Weight |  | < 1kg |  |  |

OSD (Logo/QR Code/Caption) Insertion

Typical Applications

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227