FMUSER FUQ-467B Series SD IRD Dual RF-ASI-IP Inputs 2 ASI-IP-CVBS Outputs DVB-C DVB-T-T ISDB-T DVB-S-S2-S2X Demodulation Descrambling Decoding Broadcast Reception Its production is stopped temporarily, and the date for resuming production is not fixed yet.

Demodulation Module = 2*RF/Loop out + 2*CAM + 2*Smartcard

Product overview

The FMUSER FUQ-467Y Series SD IRD integrates demodulation, descrambling, and decoding in a 1U chassis, with dual tuner inputs for RF, ASI, and IP sources. It processes signals by converting RF inputs to CVBS output, supporting standards like DVB-S/S2/S2X. The pluggable module design enables component customization. Ideal for broadcast distribution and television signal transmission.

Key feature

Integrated demodulation, descrambling, and decoding in one unit.

Supports dual tuner inputs with DVB-S/S2/S2X, DVB-C, DVB-T/(T), and ISDB-T.

Features BISS and CAM/CI descrambling for encrypted content.

Pluggable modules for easy replacement of demodulator and decoder components.

Delivers CVBS, ASI, and IP (UDP/RTP/RTSP) outputs.

Principle Chart

Technical Specifications

| Input | 2 *Tuner(DVB-C, T/(T), ISDBT, S/S2/S2X optional), F type 1*ASI input for re-mux, BNC interface 1*IP input for re-mux (UDP) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New) |  | DVB-C | Input Frequency |  | 60 ~890 MHz |  |  |
|  |  |  |  | Standard |  | J.83A, J.83C | J.83B |  |
|  |  |  |  | Constellation |  | 16/32/64/128/256 QAM | 64/256 QAM |  |
|  |  |  | DVB-T/(T) | Input Frequency |  | 60 ~890 MHz |  |  |
|  |  |  |  | Bandwidth |  | 6/7/8 M |  |  |
|  |  |  | ISDB-T | Input Frequency |  | 60 ~890 MHz |  |  |
|  | Version 1 |  | DVB-S | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate |  | 1~45 Msps |  |  |
|  |  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  | Constellation |  | QPSK |  |  |
|  |  |  |  | Signal Strength |  | - 65~-25dBm |  |  |
|  |  |  |  | Max input bitrate |  | ≤120 Mbps |  |  |
|  |  |  | DVB-S2 | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate |  | 1~45 Msps |  |  |
|  |  |  |  | FEC |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  | Constellation |  | QPSK, 8PSK |  |  |
|  |  |  |  | Max input bitrate |  | ≤120 Mbps |  |  |
|  | Version 2 (New) |  | DVB-S | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate |  | 0.5~45Msps |  |  |
|  |  |  |  | Signal Strength |  | - 65- -25dBm |  |  |
|  |  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  | Constellation |  | QPSK |  |  |
|  |  |  |  | Max input bitrate |  | ≤160 Mbps |  |  |
|  |  |  | DVB-S2 | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |  |  |
|  |  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |  |  |
|  |  |  |  | Max input bitrate |  | ≤160 Mbps |  |  |
|  |  |  | DVB-S2X | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |  |  |
|  |  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |  |
|  |  |  |  | Max input bitrate |  | ≤160 Mbps |  |  |
| Descrambling |  | CAM/CI Quantity |  | 2 |  |  |  |  |
|  |  | BISS Mode |  | Mode 1, Mode E (up to 120Mbps) |  |  |  |  |
| Output |  | IP (1*MPTS & 8*SPTS) out over UDP, RTP/RTSP. 100Base-T Ethernet interface (unicast / multicast) |  |  |  |  |  |  |
|  |  | 2×ASI mirrored out, BNC interface |  |  |  |  |  |  |
|  |  | 2*CVBS optional |  |  |  |  |  |  |
|  |  | Decode Out |  | Video Interface: 2xCVBS (RCA) Audio Interface: 2xStereo/4xMono (RCA) Resolution: PAL 576i, NTSC 480i Video Decode: MPEG-2, MPEG-4 AVC/H.264(optional) Audio Decode: MPEG 1 Layer II |  |  |  |  |
|  |  | 4*CVBS optional |  |  |  |  |  |  |
|  |  | Decode Out |  |  | Video Interface: 4xCVBS (RCA) Audio Interface: 4xStereo/8xMono (RCA) Resolution: PAL 576i, NTSC 480i Video Decode: MPEG-2, MPEG-4 AVC/H.264 Audio Decode: MPEG 1 Layer II |  |  |  |
| System |  | Local interface |  |  | LCD + control buttons |  |  |  |
|  |  | Remote management |  |  | Web-server Management |  |  |  |
|  |  | Language |  |  | English |  |  |  |
|  |  | Upgrade |  |  | USB, web management |  |  |  |
| Miscellaneous |  | Dimension |  |  | 482*300*44.5mm (W×L×H) |  |  |  |
|  |  | Approx weight |  |  | 3.5kg |  |  |  |
|  |  | Environment |  |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  |  | Power requirements |  |  | 100~240VAC, 50/60Hz |  |  |  |
|  |  | Power consumption |  |  | 20W |  |  |  |

System Connection Sample

Ordering Guide

FU-467X, ‘X’ should be interpreted as different digits which indicate tuners of different standard. * FU-4671 DVB-C SD IRD * FU-4674 DVB-(T) SD IRD * FU-4675 DVB-S/S2 SD IRD Multiple decoding boards are optional: Option 1: 2 A/V output (MPEG-2 Decoding)- FU-467X Option 2: 2 A/V output (MPEG-2 & MPEG-4 Decoding) – FU-467X-A Option 3: 4 A/V output (MPEG-2 & MPEG-4 Decoding) – FU-467X-C

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227