FMUSER FUQ-4696 CI Receiver 2-4 Tuner Inputs DVB-C-T-ISDB-T DVB-S-S2-S2X ATSC ASI-IP Output Multi-Standard Demodulation Descrambling Re-muxing for Broadcast Headends Each Demodulation Module = 2 or 4 *RF/Loop out + 2*CAM + 2*Smartcard

Product Overview

The FMUSER FUQ-4696 CI Receiver integrates demodulation and descrambling in a 1U chassis, supporting 2 or 4 tuner inputs and outputs via ASI or IP. It converts RF signals to TS output after re-mux, with CAM/CI decryption and web NMS management. Designed for broadcast systems, cable headends, and IPTV distribution.

Key feature

Supports 2 or 4 tuner inputs with switchable DVB-C/T/ISDB-T or optional DVB-S/S2/S2X/ATSC.

Features 1 ASI & 16 IP inputs for re-mux with UDP/RTP and BISS descrambling.

Includes CAM/CI slots for decrypting programs from RF, ASI, or IP without external power.

Provides output flexibility: 1 MPTS via ASI/IP and 16 SPTS over IP (UDP/RTP).

Offers web NMS management with remote control, firmware updates, and 256 PID mapping.

Principle Chart (each demodulation module)

Technical Specifications

| Input(per demodulation module) | 2 or 4* tuner inputs (DVB-C/T/ISDB-T switchable or DVB-S/S2/S2X or ATSC optional), F type 1*ASI input for re-mux, BNC interface 16*IP input for re-mux (UDP/RTP) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New) | DVB-C | Input Frequency |  |  | 60 ~890 MHz |  |
|  |  |  | Standard |  |  | J.83A, J.83C | J.83B |
|  |  |  | Constellation |  |  | 16/32/64/128/256 QAM | 64/256 QAM |
|  |  | DVB-T | Input Frequency |  |  | 60 ~890 MHz |  |
|  |  |  | Bandwidth |  |  | 6/7/8 M |  |
|  |  | ISDB-T | Input Frequency |  |  | 60 ~890 MHz |  |
|  | Satellite | DVB-S | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | 0.5~45Msps |  |
|  |  |  | Signal Strength |  |  | - 65- -25dBm |  |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  |  | Constellation |  |  | QPSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |  |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  |  | DVB-S2X | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |  |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  | ATSC | ATSC | Input Frequency |  |  | 54-858MHz |  |
|  |  |  | Bandwidth |  |  | 6 M |  |
|  |  |  | Constellation |  |  | 8 VSB |  |
| Descrambling |  | CAM/CI Quantity |  |  | 2 (per demodulation module) |  |  |
|  |  | BISS Mode |  |  | Mode 1, Mode E |  |  |
| Output (per demodulation module) |  | 1*MPTS & 16*SPTS over UDP, RTP. 1000 Base-T Ethernet interface (unicast / multicast) |  |  |  |  |  |
|  |  | 1×ASI mirrored out of MPTS, BNC interface |  |  |  |  |  |
| System |  | Remote management |  | Web-server Management |  |  |  |
|  |  | Language |  | English |  |  |  |
|  |  | Upgrade |  | web management |  |  |  |
| Miscellaneous |  | Dimension |  | 482*300*44mm (W×L×H) |  |  |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |  |  |
|  |  | Power consumption |  | 15W |  |  |  |

Order Guide

Please specify both Demodulation module and Tuner Input options when you place order:

| Demodulation Module | Option 1 | 1*Demodulation module, 2 CAM/CI Slots |
| --- | --- | --- |
|  | Option 2 | 2*Demodulation modules, 4 CAM/CI Slots |

| Tuner Input (per demodulation module) | Option 1 | 2*DVB-S/S2/S2X tuner |
| --- | --- | --- |
|  | Option 2 | 2*DVB-C/T/ISDB-T tuner |
|  | Option 3 | 2*ATSC tuner |
|  | Option 4 | 4*DVB-S/S2/S2X tuner |
|  | Option 5 | 4*DVB-C/T/ISDB-T tuner |
|  | Option 6 | 4*ATSC tuner |
|  | Option 7 | 2*DVB-S/S2/S2X tuner + 2*DVB-C/T/ISDB-T tuner |
|  | Option 8 | 2*DVB-S/S2/S2X tuner + 2* ATSC tuner |
|  | Option 9 | 2*DVB-C/T/ISDB-T tuner + 2*ATSC tuner |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227