FMUSER FUQ-4696 CI Receiver 8 Tuner Inputs DVB-S-S2-S2X DVB-C-T-ISDB-T ATSC IP ASI Outputs BISS Descrambling Broadcast IPTV Headend Each Demodulation Module = 2 or 4 *RF/Loop out + 2*CAM + 2*Smartcard

Product overview

The FUQ-4696 is a CI receiver integrating demodulation and descrambling, with inputs including 2/4 tuners for RF signals, 1 ASI input, and 16 IP inputs, and outputs such as 1 ASI output and IP TS streams ( 1 MPTS and 16 SPTS). It processes signals by demodulating inputs, descrambling encrypted content using CAM/CI slots and BISS, and re-muxing into transport streams. This 1-U rack-mountable device supports web-based management. Ideal for broadcast headend applications like content distribution and signal processing.

Key feature

Supports multi-standard tuner inputs for DVB-C/T/ISDB-T, DVB-S/S2/S2X, ATSC with 2/4 tuners,

Features re-mux flexibility with 1 ASI input and 16 IP inputs (UDP/RTP),

Provides decryption via 2 CAM/CI slots and BISS descrambling,

Offers versatile outputs including 1 MPTS, 16 SPTS over IP, and ASI output,

Enables web-based management for remote control and updates.

Principle Chart (each demodulation module)

Technical Specifications

| Input(per demodulation module) | 2 or 4* tuner inputs (DVB-C/T/ISDB-T switchable or DVB-S/S2/S2X or ATSC optional), F type 1*ASI input for re-mux, BNC interface 16*IP input for re-mux (UDP/RTP) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New) | DVB-C | Input Frequency |  |  | 60 ~890 MHz |  |
|  |  |  | Standard |  |  | J.83A, J.83C | J.83B |
|  |  |  | Constellation |  |  | 16/32/64/128/256 QAM | 64/256 QAM |
|  |  | DVB-T | Input Frequency |  |  | 60 ~890 MHz |  |
|  |  |  | Bandwidth |  |  | 6/7/8 M |  |
|  |  | ISDB-T | Input Frequency |  |  | 60 ~890 MHz |  |
|  | Satellite | DVB-S | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | 0.5~45Msps |  |
|  |  |  | Signal Strength |  |  | - 65- -25dBm |  |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  |  | Constellation |  |  | QPSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |  |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  |  | DVB-S2X | Input Frequency |  |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |  |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  |  | ≤170 Mbps |  |
|  | ATSC | ATSC | Input Frequency |  |  | 54-858MHz |  |
|  |  |  | Bandwidth |  |  | 6 M |  |
|  |  |  | Constellation |  |  | 8 VSB |  |
| Descrambling |  | CAM/CI Quantity |  |  | 2 (per demodulation module) |  |  |
|  |  | BISS Mode |  |  | Mode 1, Mode E |  |  |
| Output (per demodulation module) |  | 1*MPTS & 16*SPTS over UDP, RTP. 1000 Base-T Ethernet interface (unicast / multicast) |  |  |  |  |  |
|  |  | 1×ASI mirrored out of MPTS, BNC interface |  |  |  |  |  |
| System |  | Remote management |  | Web-server Management |  |  |  |
|  |  | Language |  | English |  |  |  |
|  |  | Upgrade |  | web management |  |  |  |
| Miscellaneous |  | Dimension |  | 482*300*44mm (W×L×H) |  |  |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |  |  |
|  |  | Power consumption |  | 15W |  |  |  |

Order Guide

Please specify both Demodulation module and Tuner Input options when you place order:

| Demodulation Module | Option 1 | 1*Demodulation module, 2 CAM/CI Slots |
| --- | --- | --- |
|  | Option 2 | 2*Demodulation modules, 4 CAM/CI Slots |

| Tuner Input (per demodulation module) | Option 1 | 2*DVB-S/S2/S2X tuner |
| --- | --- | --- |
|  | Option 2 | 2*DVB-C/T/ISDB-T tuner |
|  | Option 3 | 2*ATSC tuner |
|  | Option 4 | 4*DVB-S/S2/S2X tuner |
|  | Option 5 | 4*DVB-C/T/ISDB-T tuner |
|  | Option 6 | 4*ATSC tuner |
|  | Option 7 | 2*DVB-S/S2/S2X tuner + 2*DVB-C/T/ISDB-T tuner |
|  | Option 8 | 2*DVB-S/S2/S2X tuner + 2* ATSC tuner |
|  | Option 9 | 2*DVB-C/T/ISDB-T tuner + 2*ATSC tuner |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227