FMUSER FUQ-4823C Multiplexer Scrambler 4 Tuner Inputs 5 MPTS Outputs DVB-S-DVB-S2-DVB-S2X UDP-RTP PID Remapping Web-Based NMS Satellite Broadcast IPTV

Product overview

The FMUSER FUQ-4823C Multiplexer Scrambler processes DVB-S/S2/S2X signals through 4 tuner inputs and outputs 5 MPTS streams (4 bypassed + 1 multiplexed) via UDP/RTP. It remaps PIDs and multiplexes signals for efficient broadcast transmission. Features include web-based NMS and DiSEqC support for satellite control. Ideal for broadcast, satellite communication, and media distribution applications.

Key Features

- Supports multi-standard tuner inputs for DVB-S/S2/S2X signals across 950-2150MHz with ≤170 Mbps throughput.

- Delivers flexible MPTS output with 4 bypassed and 1 multiplexed streams via UDP/RTP multicast/unicast.

- Enables granular PID control with remapping of up to 256 PIDs per channel.

- Features integrated management via web-based NMS and DiSEqC support.

- Provides high-capacity processing for QPSK/8PSK/16APSK/32APSK constellations and FEC rates.

Inner Principle Chart (Per Card)

Specifications

| Input / Output | 4 Tuner input, F Type 5 MPTS (4 MPTS out thru bypassing, 1 MPTS multiplexing) out over UDP/RTP thru Data port, 100/1000Mself-adaption, unicast/multicast |  |  |  |
| --- | --- | --- | --- | --- |
| Mux | Max number of managing PID: 256 (for 1 MPTS out thru multiplexing) |  |  |  |
|  | Functions | PID remapping (automatically/ manually) |  |  |
|  |  | PID pass-through |  |  |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | 0.5~45 Msps |
|  |  | Constellation |  | QPSK |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps 32APSK:0.5~40 Msps |
|  |  | Constellation |  | QPSK/8PSK/16APSK/32APSK |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9,9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |
|  | DVB-S2X | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |
|  |  | Constellation |  | QPSK/8PSK/16APSK/8APSK/32APSK |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |
|  | Signal Strength | -65 ~ -25dBm |  |  |
| System | Web management:100M NMS port |  |  |  |
|  | Language: English and Chinese |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| General | Dimensions |  | 482mm×328mm×44mm (WxLxH) |  |
|  | Weight |  | 3.5kg |  |
|  | Temperature |  | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power supply |  | AC 110V±10%, 50/60Hz Or AC 220V±10%, 50/60Hz |  |
|  | Consumption |  | ≤40W |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227