FMUSER FUQ-8313 DVB-T Digital Transmitter 800W-1000W-1200W RF Output 470-806MHz LDMOS Amplifier AGC Control MFN-SFN Compatible Terrestrial TV Broadcasting

Product overview

The FUQ-8313 is a high-power DVB-T transmitter with selectable 800W/1000W/1200W RF output. It processes broadband signals in the 470–806MHz range using LDMOS amplification and AGC stabilization. The 2U rack-mount unit supports MFN/SFN topologies via modular design. Ideal for terrestrial TV broadcasting and emergency alert systems requiring low-distortion transmission.

Key feature

Provides front-panel LED/LCD for real-time fault monitoring

Technical Specifications

| Input | System standard | DVB-(T) |
| --- | --- | --- |
|  | frequency | 470MHz~806MHz |
|  | Input Level | -20 dBm±3dB (87dbuv ±3dB) |
|  | Input reflection loss | ≥15dB |
|  | Input Interface | ‘N’ |
| Output | RF output power | 800W/1000W/1200W optional |
|  | Output frequency | 470MHz~806MHz (For Single Channel: Every 8M available; For Broadband Channel: consecutive 60M available) |
|  | Output impedance | 50Ω |
|  | frequency response | ±0.5dB |
|  | Shoulder level | ≥36dB@central freqnencyIF±4.2MHz |
|  | MER | ≥33dB |
|  | output reflection loss | ≥20dB |
|  | variation of output power | ±0.25dB |
|  | In-band stray | ≤-60dBc |
|  | Out-of-band suppression | ≥65dBc |
|  | Output interface | 1-5/8 |
| General Parameters | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95% (25℃ no condensation) |
|  | Cooling mode | inside cooling fan |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220V±10%/50Hz |
|  | Machine room requirement | less dust, no shake |
|  | Dimension(L×W×H) | 850mm×600mm×1695mm |

System Diagram

PS: This diagram is a reference for 800W and 1000W transmitter.

For 1200W transmitter, there are 4 power amplifiers and 4 power supplier included.

Main Components List

| S/N | Component Name |  | Specs | Qty | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Final Power Amplifier |  | 300W | 3 pcs | For 800W |
|  |  |  | 400W | 3 pcs | For 1000W |
|  |  |  | 400W | 4 pcs | For 1200W |
|  | Pre-Exciter |  |  | 1 pc |  |
|  | Monitor Unit |  |  | 1 pc | With LCD monitor on front door |
|  | Power Supply for Final Amplifier |  |  | 3 pcs | For 800W/1000W |
|  |  |  |  | 4 pcs | For 1200W |
|  | Filter |  |  | 1 pc | As per order. (In case multiple single-channel transmitters are involved, a power combiner is required.) |
|  | Cabinet |  |  | 1 pc |  |
|  | Cooling Fan |  |  | 1 set | 2 sets (with 1 set for backup) optional as order or needed |
| Accessaries |  |  |  |  |  |
|  |  | Hard Feeder | Φ40, 2 meter long | 1 | These parts are standard configuration. If there are special requirement, please indicate when place order. |
|  |  | Elbow |  | 3 |  |
|  |  | Hoop |  | 3 |  |
|  |  | Hose Clamp |  | 13 |  |
|  |  | Central Pin |  | 3 |  |
|  |  | Flange | Φ40 | 1 |  |
|  |  | antenna |  | 1 | As per order |
|  |  | Feeder |  |  | As per order |
|  |  | Combiner |  | 1 | As per order |

Order Info

| Model Number | Description |
| --- | --- |
| DUT-8313(800W-I) | 800W Indoor |
| DUT-8313(1000W-I) | 1000W Indoor |
| DUT-8313(1200W-I) | 1200W Indoor |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227