FMUSER FUQ-8322 DVB-T Digital Transmitter 1 Input 1 Output 470-806MHz Frequency Range 4-64 QAM Modulation LDMOS Amplifier Technology High Linearity AGC Control Terrestrial TV Broadcast Emergency Backup

Product overview

The FUQ-8322 is a DVB-T digital transmitter with configurable 200W-600W RF output, featuring LDMOS amplifier technology for high linearity. It processes HD/SD signals with 4/16/32/64 QAM modulation, supporting both SFN (±1Hz sync) and MFN (±100Hz) network modes via 470-806MHz frequency range. This 2U rack-mount unit integrates AGC control, modular redundancy, and intelligent air cooling for 24/7 broadcast operations. Ideal for terrestrial TV transmission and emergency backup systems.

Key feature

Features low-noise cooling and multi-stage lightning protection

Specifications

| Item |  | Technical Spec |
| --- | --- | --- |
| Basic Parameter | Standard | DVB-(T) |
|  | Modulation Mode | support 4/16/32/64 QAM mode |
|  | Working frequency | 470MHz～806MHz |
|  | Output rating | 200W/300W/400W/500W/600W Optional |
|  | Frequency accuracy | MFN: ≤±100Hz / SFN：≤±1Hz |
|  | L.O. Phase Noise | ≤ - 70dBc@10Hz |
|  |  | ≤ - 90dBc@100Hz |
|  |  | ≤ - 100dBc@1kHz |
|  |  | ≤ - 105dBc@10kHz |
|  |  | ≤ - 115dBc@100kHz |
|  |  | ≤ - 130dBc@1MHz |
|  | variation of output power | ±0.2dB |
|  | Inband spectrum ripple | ±0.5dB（fc±3.591MHz） |
|  | Shoulder Level | ≤-40dB@ Central frequency, IF±4.2MHz |
|  | MER | ≥36dB |
| Output Character | Output impedance | 50Ω |
|  | output reflection loss | ≥20dB |
|  | Output interface | 200W/300W/400W: L29-K; 500W/600W: 1-5/8'', directly-fed |
| Environment Condition | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95% (25℃ no condensation) |
|  | Cooling mode | inside cooling fan |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220±20%, 50Hz AC, 380±20%, 50Hz |
|  | Machine room requirement | less dust, no friction |

Principle Chart

Main Components List (Standard Configuration)

| S/N | Component | 200W | 300W | 400W | 500W | 600W | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Qty (piece) |  |  |  |  |  |
| 1 | Cabinet/Rack | N/A | 1 | 1 | 1 | 1 | These configuration is standard. If customer need certain redundancy component, it is available and negotiable. |
| 2 | Power Supply | 1 (Built-in) | 1 | 1 | 1 | 2 |  |
| 3 | Exciter | 1 (Built-in) | 1 | 1 | 1 | 1 |  |
| 4 | Power Amplifier | 1 (Built-in) | 1 | 1 | 1 | 2 |  |
| 5 | Monitor Unit | N/A | 1 | 1 | 1 | 1 |  |
| 6 | Filter | 1 (Built-in) | 1 | 1 | 1 | 1 |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227