FMUSER FUQ-8322 200W-600W DVB-T Transmitter Adjustable Output Power LDMOS Amplifier SFN-MFN Operation Professional Broadcast Terrestrial TV

Product overview

The FMUSER FUQ-8322 is a professional-grade DVB-T transmitter with 200W-600W adjustable output. It processes RF signals through LDMOS amplifier technology, supporting both SFN (±1Hz accuracy) and MFN (±100Hz) operation modes. The 2U rack-mount unit features intelligent AGC control and multi-stage lightning protection, ideal for terrestrial TV broadcasting and single-frequency network deployments.

Key feature

2U rack design with efficient cooling system

Specifications

| Item |  | Technical Spec |
| --- | --- | --- |
| Basic Parameter | Standard | DVB-(T) |
|  | Modulation Mode | support 4/16/32/64 QAM mode |
|  | Working frequency | 470MHz～806MHz |
|  | Output rating | 200W/300W/400W/500W/600W Optional |
|  | Frequency accuracy | MFN: ≤±100Hz / SFN：≤±1Hz |
|  | L.O. Phase Noise | ≤ - 70dBc@10Hz |
|  |  | ≤ - 90dBc@100Hz |
|  |  | ≤ - 100dBc@1kHz |
|  |  | ≤ - 105dBc@10kHz |
|  |  | ≤ - 115dBc@100kHz |
|  |  | ≤ - 130dBc@1MHz |
|  | variation of output power | ±0.2dB |
|  | Inband spectrum ripple | ±0.5dB（fc±3.591MHz） |
|  | Shoulder Level | ≤-40dB@ Central frequency, IF±4.2MHz |
|  | MER | ≥36dB |
| Output Character | Output impedance | 50Ω |
|  | output reflection loss | ≥20dB |
|  | Output interface | 200W/300W/400W: L29-K; 500W/600W: 1-5/8'', directly-fed |
| Environment Condition | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95% (25℃ no condensation) |
|  | Cooling mode | inside cooling fan |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220±20%, 50Hz AC, 380±20%, 50Hz |
|  | Machine room requirement | less dust, no friction |

Principle Chart

Main Components List (Standard Configuration)

| S/N | Component | 200W | 300W | 400W | 500W | 600W | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Qty (piece) |  |  |  |  |  |
| 1 | Cabinet/Rack | N/A | 1 | 1 | 1 | 1 | These configuration is standard. If customer need certain redundancy component, it is available and negotiable. |
| 2 | Power Supply | 1 (Built-in) | 1 | 1 | 1 | 2 |  |
| 3 | Exciter | 1 (Built-in) | 1 | 1 | 1 | 1 |  |
| 4 | Power Amplifier | 1 (Built-in) | 1 | 1 | 1 | 2 |  |
| 5 | Monitor Unit | N/A | 1 | 1 | 1 | 1 |  |
| 6 | Filter | 1 (Built-in) | 1 | 1 | 1 | 1 |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227