FMUSER FUQ-8323 DVB-T Transmitter 1-Channel Input-Output DVB-T Digital RF Signal MFN-SFN Ready High-Power LDMOS Amplifier for Terrestrial Broadcast

Product overview

The DUT-8323 is a high-power DVB-T transmitter delivering 1.5kW-3kW output through modular LDMOS amplifiers. It processes digital signals across 470-806MHz with AGC stability and supports both MFN and SFN network configurations. The 19" rack-mount unit features front-panel controls and LED monitoring for real-time diagnostics. Ideal for terrestrial TV broadcasting and large-scale digital transmission networks requiring scalable power output.

Key feature

Multi-stage lightning protection and wide-voltage PSU (220V±10%)

Technical Specifications

| Input | System standard | DVB-(T) |
| --- | --- | --- |
|  | frequency | 470MHz~806MHz |
|  | Input Level | -20 dBm±3dB (87dbuv ±3dB) |
|  | Input reflection loss | ≥15dB |
|  | Input Interface | ‘N’ |
| Output | RF output power | 1500W/2000W/2500W/3000W optional |
|  | Output frequency | 470MHz~806MHz (For Single Channel: Every 8M available; For Broadband Channel: consecutive 60M available) |
|  | Output impedance | 50Ω |
|  | frequency response | ±0.5dB |
|  | Shoulder level | ≥36dB@central freqnencyIF±4.2MHz |
|  | MER | ≥33dB |
|  | output reflection loss | ≥20dB |
|  | variation of output power | ±0.25dB |
|  | In-band stray | ≤-60dBc |
|  | Out-of-band suppression | ≥65dBc |
|  | Output interface | 1-5/8 |
| General Parameters | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95% (25℃ no condensation) |
|  | Cooling mode | inside cooling fan |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220V±10%/50Hz |
|  | Machine room requirement | less dust, no shake |
|  | Dimension(L×W×H) | 1050mm×600mm×1695mm |

System Diagram

PS: This diagram is a reference for 1500W and 2000W transmitter. For 2500W and 3000W transmitter,

there are 4 power amplifiers and 8 power supplier included.

Main Components List

| S/N | Component Name | Specs | Qty | Remarks |
| --- | --- | --- | --- | --- |
|  | Final Power Amplifier | 800W | 3 pcs | For 2000W |
|  |  | 800W | 4 pcs | For 3000W |
|  |  | 2000W and 3000W are standard power, 1500W and 2500W are customized version |  |  |
|  | Pre-Exciter |  | 1 pc |  |
|  | Monitor Unit |  | 1 pc | With LCD monitor on front door |
|  | Power Supply for Final Amplifier |  | 6 pcs | For 1500W/2000W |
|  |  |  | 8 pcs | For 2500W/3000W |
|  | Filter |  | 1 pc | As per order. (In case multiple single-channel transmitters are involved, a power combiner is required.) |
|  | Cabinet |  | 1 pc |  |
|  | Cooling Fan |  | 1 set | 2 sets (with 1 set for backup) optional as order or needed |
| Accessaries |  |  |  |  |
|  | Hard Feeder | Φ40, 2 meter long | 1 | These parts are standard configuration. If there are special requirement, please indicate when place order. |
|  | Elbow |  | 3 |  |
|  | Hoop |  | 3 |  |
|  | Hose Clamp |  | 13 |  |
|  | Central Pin |  | 3 |  |
|  | Flange | Φ40 | 1 |  |
|  | antenna |  | 1 | As per order |
|  | Feeder |  |  | As per order |
|  | Combiner |  | 1 | As per order |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227