FMUSER FUQ-8323 DVB-T Transmitter 1-Channel 470-806MHz Input Output DVB-T MFN-SFN Support AGC Stabilization Rack-Mount Broadcast Transmission Product overview

The FMUSER FUQ-8323 is a DVB-T digital transmitter delivering scalable 1500W–3000W RF output via 470–806MHz with high-gain LDMOS amplifiers. It processes DVB-T signals with AGC stabilization and supports MFN/SFN networks in a compact rack-mountable chassis. Designed for 24/7 unmanned operation, it features digital front-panel control and multi-stage lightning protection. Ideal for terrestrial TV broadcasting, SFN network deployment, and emergency backup transmission in harsh environments.

Key feature

Robust Design: Rack-mountable chassis built for 24/7 unmanned operation

Technical Specifications

| Input | System standard | DVB-(T) |
| --- | --- | --- |
|  | frequency | 470MHz~806MHz |
|  | Input Level | -20 dBm±3dB (87dbuv ±3dB) |
|  | Input reflection loss | ≥15dB |
|  | Input Interface | ‘N’ |
| Output | RF output power | 1500W/2000W/2500W/3000W optional |
|  | Output frequency | 470MHz~806MHz (For Single Channel: Every 8M available; For Broadband Channel: consecutive 60M available) |
|  | Output impedance | 50Ω |
|  | frequency response | ±0.5dB |
|  | Shoulder level | ≥36dB@central freqnencyIF±4.2MHz |
|  | MER | ≥33dB |
|  | output reflection loss | ≥20dB |
|  | variation of output power | ±0.25dB |
|  | In-band stray | ≤-60dBc |
|  | Out-of-band suppression | ≥65dBc |
|  | Output interface | 1-5/8 |
| General Parameters | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95% (25℃ no condensation) |
|  | Cooling mode | inside cooling fan |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220V±10%/50Hz |
|  | Machine room requirement | less dust, no shake |
|  | Dimension(L×W×H) | 1050mm×600mm×1695mm |

System Diagram

PS: This diagram is a reference for 1500W and 2000W transmitter. For 2500W and 3000W transmitter,

there are 4 power amplifiers and 8 power supplier included.

Main Components List

| S/N | Component Name | Specs | Qty | Remarks |
| --- | --- | --- | --- | --- |
|  | Final Power Amplifier | 800W | 3 pcs | For 2000W |
|  |  | 800W | 4 pcs | For 3000W |
|  |  | 2000W and 3000W are standard power, 1500W and 2500W are customized version |  |  |
|  | Pre-Exciter |  | 1 pc |  |
|  | Monitor Unit |  | 1 pc | With LCD monitor on front door |
|  | Power Supply for Final Amplifier |  | 6 pcs | For 1500W/2000W |
|  |  |  | 8 pcs | For 2500W/3000W |
|  | Filter |  | 1 pc | As per order. (In case multiple single-channel transmitters are involved, a power combiner is required.) |
|  | Cabinet |  | 1 pc |  |
|  | Cooling Fan |  | 1 set | 2 sets (with 1 set for backup) optional as order or needed |
| Accessaries |  |  |  |  |
|  | Hard Feeder | Φ40, 2 meter long | 1 | These parts are standard configuration. If there are special requirement, please indicate when place order. |
|  | Elbow |  | 3 |  |
|  | Hoop |  | 3 |  |
|  | Hose Clamp |  | 13 |  |
|  | Central Pin |  | 3 |  |
|  | Flange | Φ40 | 1 |  |
|  | antenna |  | 1 | As per order |
|  | Feeder |  |  | As per order |
|  | Combiner |  | 1 | As per order |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227