FMUSER FUQ-8351 Digital Transmitter 10W-100W Output DVB-T-ATSC-ISDB-T Input-Output Compliance LDMOS Amplifier AGC Control SFN-MFN Terrestrial Broadcasting

Product overview

The FUQ-8351 is a DVB-T/ATSC/ISDB-T compliant digital transmitter with 10W-100W adjustable output via LDMOS amplification. It processes MPEG-2/4 signals with AGC stability, housed in a 482x504x132mm rack-mount chassis. Designed for SFN/MFN deployments, it supports 470-806MHz frequency range with LED alarm monitoring, ideal for terrestrial TV broadcasting and emergency backup systems.

Key feature

Features real-time monitoring via LED alarms and fault self-diagnosis

Monitoring Objects and Alarm Status

| Monitoring Objects | Alarm Status |
| --- | --- |
| Power Supply | abnormal |
| Overheating | alert |
| Amplifier | abnormal |
| Antenna feed system | abnormal |

Phase noise

| Center frequency deviation (Hz) | Local oscillator (dBc/Hz) |
| --- | --- |
| @10 | ≤ - 70 |
| @100 | ≤ - 85 |
| @1k | ≤ - 95 |
| @10 k | ≤ - 100 |
| @100 k | ≤ - 111 |
| @1 M | ≤ - 125 |

Electrical Parameters

| Item |  | Technical specs |
| --- | --- | --- |
| Basic Parameters | TV Standard | DVB-T/ATSC/ISDB-T |
|  | Modulating mode | 4/16/32/64 QAM |
|  | Frequency stability (3 months) | External Frequency Range: ≤10-10 |
|  |  | Internal Frequency Range: ≤10-7 |
|  | Frequency accuracy | MFN: ±100Hz / SFN: ±1Hz |
|  | Local oscillator phase noise | See table above |
|  | In-band stray | ≤-60dBc |
|  | In-band ripple （fc±3.591MHz） | ±0.5dB |
|  | Out-of-band rejection | ≤-65dBc |
| Input | Frequency range | 470MHz～806MHz (any 100MHz bandwidth) |
|  | level | -20±3dBm |
|  | Input reflection loss | ≥15dB |
|  | connector | N-K |
| Output | Output power | 10W, 20W, 30W, 50W, 100W Optional |
|  | Frequency range | 470MHz～806MHz (any 100MHz bandwidth) |
|  | Output impedance | 50Ω |
|  | In-band ripple (fc±3.591MHz) | ±0.5dB |
|  | Shoulder level | ≥40dB@central frequency FC±4.2MHz (after correction, single channel) |
|  | MER | ≥35dB (after correction, single channel) |
|  | output reflection loss | ≥26dB |
|  | Output power variation | ±0.25dB |
|  | Inner adjacent channel transmission power | Inner adjacent channel transmission power : in-band transmission power≤-60dB, meet that Inner adjacent channel transmission power≤13mW |
|  | External adjacent channel transmission power | External adjacent channel transmission power : in-band transmission power≤-65dB, meet that Inner adjacent channel transmission power≤13mW |
| Environment condition | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95%（no condensation at 25℃） |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220V±10%/50Hz |
|  | Case size (L/W/H) | 482x504x132mm |

Optical Signal Parameters

| Item | Unit | Technical Specs |
| --- | --- | --- |
| Optical wavelength | nm | 1100-1600 |
| nominal optical input power | dBm | -1 |
| Input optical power range | dBm | -15~+2 |
| Max input optical power | dBm | +3 |
| Optical interface |  | FC/APC |
| optical reflection loss | dB | 45 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227