FMUSER DUT-8351 Digital Transmitter 1-Channel DVB-T-ATSC-ISDB-T Input Output Adjustable 10-100W RF Power LDMOS Amplifier Indoor HD SD Broadcasting Product overview

The FUQ-8351 is a professional indoor digital transmitter supporting DVB-T/ATSC/ISDB-T standards with 10-100W adjustable RF output (470-806MHz). It processes signals via 4/16/32/64 QAM modulation and features AGC-controlled power stability (±0.25dB). The 2U rack-mount unit includes LED alarms for real-time fault monitoring, designed for SFN/MFN networks in broadcast studios and mobile TV systems.

Key feature

Broadcast-Grade RF Performance: Achieves shoulder ≥40dB across 470-806MHz frequency range

Monitoring Objects and Alarm Status

| Monitoring Objects | Alarm Status |
| --- | --- |
| Power Supply | abnormal |
| Overheating | alert |
| Amplifier | abnormal |
| Antenna feed system | abnormal |

Phase noise

| Center frequency deviation (Hz) | Local oscillator (dBc/Hz) |
| --- | --- |
| @10 | ≤ - 70 |
| @100 | ≤ - 85 |
| @1k | ≤ - 95 |
| @10 k | ≤ - 100 |
| @100 k | ≤ - 111 |
| @1 M | ≤ - 125 |

Electrical Parameters

| Item |  | Technical specs |
| --- | --- | --- |
| Basic Parameters | TV Standard | DVB-T/ATSC/ISDB-T |
|  | Modulating mode | 4/16/32/64 QAM |
|  | Frequency stability (3 months) | External Frequency Range: ≤10-10 |
|  |  | Internal Frequency Range: ≤10-7 |
|  | Frequency accuracy | MFN: ±100Hz / SFN: ±1Hz |
|  | Local oscillator phase noise | See table above |
|  | In-band stray | ≤-60dBc |
|  | In-band ripple （fc±3.591MHz） | ±0.5dB |
|  | Out-of-band rejection | ≤-65dBc |
| Input | Frequency range | 470MHz～806MHz (any 100MHz bandwidth) |
|  | level | -20±3dBm |
|  | Input reflection loss | ≥15dB |
|  | connector | N-K |
| Output | Output power | 10W, 20W, 30W, 50W, 100W Optional |
|  | Frequency range | 470MHz～806MHz (any 100MHz bandwidth) |
|  | Output impedance | 50Ω |
|  | In-band ripple (fc±3.591MHz) | ±0.5dB |
|  | Shoulder level | ≥40dB@central frequency FC±4.2MHz (after correction, single channel) |
|  | MER | ≥35dB (after correction, single channel) |
|  | output reflection loss | ≥26dB |
|  | Output power variation | ±0.25dB |
|  | Inner adjacent channel transmission power | Inner adjacent channel transmission power : in-band transmission power≤-60dB, meet that Inner adjacent channel transmission power≤13mW |
|  | External adjacent channel transmission power | External adjacent channel transmission power : in-band transmission power≤-65dB, meet that Inner adjacent channel transmission power≤13mW |
| Environment condition | Working temperature | -20~+50℃ |
|  | Storage temperature | -30~+75℃ |
|  | Relatively humidity | ＜95%（no condensation at 25℃） |
|  | atm press | 86~106kPa |
|  | power supply | AC, 220V±10%/50Hz |
|  | Case size (L/W/H) | 482x504x132mm |

Optical Signal Parameters

| Item | Unit | Technical Specs |
| --- | --- | --- |
| Optical wavelength | nm | 1100-1600 |
| nominal optical input power | dBm | -1 |
| Input optical power range | dBm | -15~+2 |
| Max input optical power | dBm | +3 |
| Optical interface |  | FC/APC |
| optical reflection loss | dB | 45 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227